SA WG2 Meeting #143e	S2-2100230

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-2009002)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2329 | rev | 2 | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of the mapping from IP addressing information provided to an AF to the user identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | (Relates with LS in S2-2008358) An AF may only be able to identify the UE target of an AF request for Data Provisioning or for Event Exposure about a UE by providing UE addressing information (e.g. IP address and port information) to identify this UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An AF may request (via the NEF) information exposure or parameter provisioning targetting an individual UE, identifying the target UE by providing UE addressing information (e.g. IP address and port of the UE). In this case the 5GC (NEF) first needs to retrieve a Permanent Identifier of the UE Then the NEF Event exposure or the parameter provisioning as defined in R16 may apply. Nnef_EventExposure_Subscribe, Nnef_Location_LocationUpdateNotify and Nnef_ParameterProvision operations are modified accordingly. The NEF may deliver back to the AF an UE external Identifier for an Application. This UE external Identifier for an Application may later on be used by the AF to issue further requests about the same UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An AF cannot identify the UE target of an AF request for Data Provisioning or for Event Exposure by providing UE addressing information (e.g. IP address and port information) to identify this UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.X (new) ; 5.2.6.2.2 ; 5.2.6.4 ; 5.2.6.21.2 ; 5.2.13.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 2385 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 4.15.3.2.X	Handling AF requests when the UE is identified via DN addressing information

An AF may via the NEF request Event Exposure (as defined otherwise in other sub-clauses of clause 4.15.3) or parameter provisioning (as defined in clause 4.15.6) targetting an individual UE, identifying the target UE by providing DN addressing information (e.g. IP address and port) of the UE.

In this case the NEF first needs to retrieve a GPSI/SUPI of the UE from BSF by invoking Nbsf_Management_Discovery service operation using:

- the DN addressing information (e.g. IP address and port information) of the UE as provided by the AF

- .

- the corresponding DNN and/or S-NSSAI: this may have been provided by the AF or determined by the NEF using the identity of the AF;

Once this is done, the 5GC may carry out the action requested by the AF and may deliver back to the AF an UE external Identifier for an Application. This UE external Identifier for an Application may later on be used by the AF to issue further requests about the same UE. The UE external Identifier for an Application is defined in TS 33.501 [15].

The AF may have its own means to maintain the UE external Identifier for an Application through, e.g., an AF session. After the retrieval of a UE external Identifier for an Application the AF shall not keep maintaining a mapping between the returned UE external Identifier for an Application and the UE IP address as this mapping may change.

NEXT CHANGE (2)

### 4.15.6	External Parameter Provisioning

#### 4.15.6.X	Parameter Provisioning when the UE is identified via DN addressing information

Handling of Parameter Provisioning requests targeting an individual UE when the UE is identified via DN addressing information is described in clause 4.15.3.2.X. Once the procedure described in clause 4.15.3.2.X has taken place, the Parameter Provisioning as described in other sub-clauses of 4.15.6 may apply.

NEXT CHANGE (3)

##### 5.2.6.2.2	Nnef_EventExposure_Subscribe operation

Service operation name: Nnef_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated.

Inputs, Required: (Set of) Event ID(s) as specified in clause 4.15.3.1 or Npcf_PolicyAuthorization_Notify and Naf_EventExposure_Subscribe service operation, Target of Event Reporting (GPSI, SUPI, External Group Identifier, Internal Group Identifier, UE addressing information (e.g. IP address and port) or UE external Identifier for an Application as defined in clause 4.15.3.2.X), Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Event Filter, (set of) Application Identifier(s), Subscription Correlation ID (in the case of modification of the event subscription), Expiry time.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available (see clause 4.15.1), UE external Identifier for an Application (see clause 4.15.3.2.X).

NEXT CHANGE (4)

#### 5.2.6.4	Nnef_ParameterProvision service

##### 5.2.6.4.1	General

This service is for allowing external party to provision of information which can be used for the UE in 5GS.

##### 5.2.6.4.2	Nnef_ParameterProvision_Update service operation

Service operation name: Nnef_ParameterProvision_Update

Description: The consumer updates the UE related information (e.g., Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters) or 5G VN Group related information (e.g. 5G VN group data, 5G VN membership management).

Inputs, Required: AF ID, Transaction Reference ID.

Inputs, Optional: GPSI, UE addressing information (e.g. IP address and port) or UE external Identifier for an Application (as defined in clause 4.15.3.2.X),, External Group ID at least one of the Expected UE Behaviour parameters or at least one of the Network Configuration parameters or 5G VN related information, Validity Time or Location Privacy Indication parameters.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available, UE external Identifyer for an Application (see clause 4.15.3.2.X).

##### 5.2.6.4.3	Nnef_ParameterProvision_Create service operation

Service operation name: Nnef_ParameterProvision_Create

Description: The consumer creates a 5G VN group.

Inputs, Required: AF ID, Transaction Reference ID.

Inputs, Optional: GPSI, UE addressing information (e.g. IP address and port) or UE external Identifier for an Application as defined in clause 4.15.3.2.X, External Group ID for 5G VN group creation, External Group ID, 5G VN group related information (e.g., 5G VN group data, 5G VN membership management).

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available, UE external Identifyer for an Application (see clause 4.15.3.2.X).

##### 5.2.6.4.4	Nnef_ParameterProvision_Delete service operation

Service operation name: Nnef_ParameterProvision_Delete

Description: The consumer deletes a 5G VN group.

Inputs, Required: AF ID, Transaction Reference ID.

Inputs, Optional: External Group ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.4.5	Nnef_ParameterProvision_Get service operation

Service operation name: Nnef_ParameterProvision_Get

Description: The consumer gets the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters).

Inputs, Required: GPSI, UE addressing information (e.g. IP address and port) or UE external Identifier for an Application as defined in clause 4.15.3.2.X, AF ID, requested information (e.g., Expected UE Behaviour, Network Configuration parameters).

Inputs, Optional: None.

Outputs, Required: Requested data, Operation execution result indication.

Outputs, Optional: UE external Identifyer for an Application (see clause 4.15.3.2.X).

NEXT CHANGE (5)

##### 5.2.6.21.2	Nnef_Location_LocationUpdateNotify service operation

Service operation name: Nnef_Location_LocationUpdateNotify

Description: Provides UE location information to the consumer NF.

Inputs, Required: Identity of the AF, UE identifier (GPSI, UE addressing information (e.g. IP address and port) or UE external Identifier for an Application as defined in clause 4.15.3.2.X), event causing the location estimate (5GC-MO-LR), location estimate, age of location estimate, accuracy indication, LCS QoS class.

Inputs, Optional: None.

Outputs, Required: Success/Failure indication.

Output (optional): Failure Cause (in the case of failure indication provided), UE external Identifyer for an Application (see clause 4.15.3.2.X).

END OF CHANGES

##### 5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management discovery

Description: Discovers the PCF and PCF set selected for the tuple (UE address(es), SUPI, GPSI, DNN, S-NSSAI). This operation may also be used to determine the SUPI/GPSI from the tuple (UE address, DNN, S-NSSAI)

Inputs, Required: UE address (i.e. IP address or MAC address), DNN [Conditional], S-NSSAI [Conditional].

NOTE:	For TSN scenarios the UE address contains the DS-TT port MAC address.

Inputs, Optional: SUPI, GPSI.

Outputs, Required: PCF address(es), PCF instance ID [Conditional, if available] and PCF Set ID [Conditional, if available], level of Binding [Conditional, if available] (see clause 6.3.1.0 of TS 23.501 [2]).

Outputs, Optional: , SUPI/GPSI.