3GPP TSG-SA WG2 Meeting #143-e (e-meeting)	S2-2100267r01

# 24 February- 9 March 2021, Elbonia 	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2550 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNPN selection for access to SNPNs using credentials from an entity separate from the SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorprorated, Intel, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces changes into TS 23.501 to enabe SNPN selection for access to SNPNs using credentials from an entity separate from the SNPN in line with the conclusions for Key Issue #1 in TR 23.700-07. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce two new indications ("access using credentials from a separate entity is supported" and "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") and List of supported Group IDs for Network Selection per NID in SIB in clause 5.30.2.2 - Update clause 5.30.2.3 to introduce user-controlled prioritized list of preferred SNPNs, separate entity controlled prioritized list of preferred SNPNs and separate entity-controlled prioritized list of Group IDs for Network Selection in UE configuration and subscription aspects for SNPN-enabled UEs - Create new clause 5.30.2.3a SNPN access mode to shorten the UE configuration clause - Clarify that an SNPN-enabled UE that supports access to an SNPN using credentials from a separate entity and that is equipped with a USIM holding a PLMN subscription needs needs to first enter SNPN access mode to be able to select SNPN - Update clause 5.30.2.4 to reflect agreements for automatic and manual network selection as concluded for key issue #1 in TR 23.700-07 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for SNPN selection for access to SNPNs using credentials from an entity separate from the SNPN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.30.2.2, 5.30.2.3, 5.30.2.3a (new), 5.30.2.4, 5.30.2.4.1 (new), 5.30.2.4.2 (new), 5.30.2.4.3 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Limited service state aspects are included in a separate CR (CR#...) on emergency call support in SNPNs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G-VN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

AF	Application Function

AMF	Access and Mobility Management Function

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

N3IWF	Non-3GPP InterWorking Function

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-Specific Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PTP	Precision Time Protocol

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

**** Next Change ****

5.30.2.2	Broadcast system information

NG-RAN nodes which provide access to SNPNs broadcast the following information:

-	One or multiple PLMN IDs

-	List of NIDs per PLMN ID identifying the non-public networks NG-RAN provides access to

NOTE 1:	It is assumed that an NG-RAN node supports broadcasting a total of twelve NIDs. Further details are defined in TS 38.331 [28].

NOTE°2:	The presence of a list of NIDs for a PLMN ID indicates that the related PLMN ID and NIDs identify SNPNs.

-	Optionally:

-	A human-readable network name per NID.

NOTE 3:	The human-readable network name per NID is only used for manual SNPN selection. The mechanism how human-readable network name is provided (i.e. whether it is broadcasted or unicasted) to the UE is specified in TS 38.331 [28].

-	Information, as described in TS 38.300 [27], TS 38.331 [28] and in TS 38.304 [50], to prevent UEs not supporting SNPNs from accessing the cell, e.g. if the cell only provides access to non-public networks.

-	An indication per NID of whether access using credentials from a Separate Entity is supported

-	List of supported Group IDs for Network Selection (GINs) per NID

-	An indication per NID of whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, i.e., UEs that do not have any PLMN ID and NID nor GIN broadcast by the SNPN in the Separate Entity controlled prioritized lists of preferred SNPNs/GINs.

NOTE 4:	Further details (including number of supported GINs per NID) are defined in TS 38.331 [28].

5.30.2.3	UE configuration and subscription aspects

An SNPN-enabled UE is configured with the following information for each subscribed SNPN:

-	PLMN ID and NID of the SNPN;

-	Subscriber identifier (SUPI) and credentials;

-	Optionally, an N3IWF FQDN and an identifier of the country where the configured N3IWF is located;

-	Optionally, if the UE supports access to an SNPN using credentials from a Separate Entity:

-	User controlled prioritized list of preferred SNPNs;

-	Separate Entity controlled prioritized list of preferred SNPNs;

-	Separate Entity controlled prioritized list of GINs.

The Separate Entity controlled prioritized lists of preferred SNPNs and GINs can be updated by the Separate Entity.

Editor’s note:	Whether the Separate Entity controlled lists can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or the Steering of Roaming (SoR) procedure as defined in TS 23.122 [17] Annex C will be determined based on feedback from CT1 and SA3.

A subscriber of an SNPN is either:

-	identified by a SUPI containing a network-specific identifier that takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19] clause 28.7.2. The realm part of the NAI may include the NID of the SNPN; or

-	identified by a SUPI containing an IMSI.

An SNPN-enabled UE that supports access to an SNPN using credentials from a Separate Entity and that is equipped with a USIM holding a PLMN subscription may additionally be configured with the following information for SNPN selection und registration using the PLMN subscription in SNPN access mode:

-	User controlled prioritized list of preferred SNPNs;

-	Separate Entity controlled prioritized list of preferred SNPNs;

-	Separate Entity controlled prioritized list of preferred GINs.

The Separate Entity controlled prioritized lists of preferred SNPNs and GINs can be updated by the Seperate Entity.

Editor’s note:	Whether the Separate Entity controlled lists can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or the Steering of Roaming (SoR) procedure as defined in TS 23.122 [17] Annex C will be determined based on feedback from CT1 and SA3.

5.30.2.3a	SNPN access mode

An SNPN-enabled UE supports the SNPN access mode. When the UE is set to operate in SNPN access mode the UE only selects and registers with SNPNs over Uu as described in clause 5.30.2.4.

Emergency services are not supported in SNPN access mode.

NOTE 1:	Voice support with emergency services in SNPN access mode is not specified in this release.

If a UE is not set to operate in SNPN access mode, even if it is SNPN-enabled, the UE does not select and register with SNPNs. A UE not set to operate in SNPN access mode performs PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17]. For a UE capable of simultaneously connecting to an SNPN and a PLMN, the setting for operation in SNPN access mode is applied only to the Uu interface for connection to the SNPN. Annex D.4 provides more details.

An SNPN-enabled UE that supports access to an SNPN using credentials from a Separate Entity and that is equipped with a USIM holding a PLMN subscription needs to enter SNPN access mode in order to select SNPNs. Once the UE has entered SNPN access mode, SNPN selection is performed as described in clause 5.30.2.4. Once an SNPN has been selected the UE attempts registration using the PLMN credentials.

NOTE 2:	Details of activation and deactivation of SNPN access mode are up to UE implementation.

5.30.2.4	Network selection in SNPN access mode

5.30.2.4.1	General

When a UE is set to operate in SNPN access mode the UE does not perform normal PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17].

UEs operating in SNPN access mode read the information described in clause 5.30.2.2 from the broadcast system information and take them into account during network selection.

5.30.2.4.2	Automatic network selection

NOTE 1:	If the UE has multiple SNPN subscriptions it is assumed that the subscription to use for automatic selection is determined by implementation specific means prior to network selection.

For automatic network selection the UE selects and attempts registration on available and allowable SNPNs in the following order:

-	the SNPN the UE was last registered with (if available);

-	 the SNPN identified by the PLMN ID and NID for which the UE has SUPI and credentials.;

-	If the UEs supports access to an SNPN using credentials from a Separate Entity then the UE continues in the following order:

-	SNPNs, which broadcast the indication that access using credentials from a Separate Entity is supported and an SNPN ID contained in the user controlled prioritized list of preferred SNPNs (in priority order);

-	SNPNs, which broadcast the indication that access using credentials from a Separate Entity is supported and an SNPN ID contained in the Separate Entity controlled prioritized list of preferred SNPNs (in priority order);

-	SNPNs, which broadcast the indication that access using credentials from a Separate Entity is supported and a GIN contained in the Separate Entity controlled prioritized list of preferred GINs (in priority order);

NOTE 2:	If multiple SNPNs are available that broadcast the same GIN, the order in which the UE selects and attempts a registration with those SNPNs is implementation specific.

-	SNPNs, which broadcast an indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN i.e., the broadcasted NID or GIN is not present in the Separate Entity controlled prioritized lists of preferred SNPNs/GINs in the UE.

NOTE 3:	If multiple SNPNs are available that broadcast the indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, the order in which the UE selects and attempts a registration with those SNPNs is implementation specific.

Editor's note: The SNPN selection details listed above will be replaced by a reference to TS 23.122 [17] once SNPN selection in TS 23.122 [17] has been updated in line with the above.

When a UE performs Initial Registration to an SNPN, the UE shall indicate the selected NID and the corresponding PLMN ID to NG-RAN i.e., the PLMN ID and NID broadcast by the selected SNPN. NG-RAN shall inform the AMF of the selected PLMN ID and NID.

5.30.2.4.3	Manual network selection

For manual network selection UEs operating in SNPN access mode provide to the user the list of SNPNs (each is identified by a PLMN ID and NID) and related human-readable names (if available) of the available SNPNs the UE has respective SUPI and credentials for. If the UEs supports access to an SNPN using credentials from a Separate Entity, the UE also presents available SNPNs which broadcast the "access using credentials from a Separate Entity is supported" indication.

NOTE:	The details of manual SNPN selection are defined in TS 23.122 [17].

When a UE performs Initial Registration to an SNPN, the UE shall indicate the selected NID and the corresponding PLMN ID to NG-RAN. NG-RAN shall inform the AMF of the selected PLMN ID and NID.

*** End of Changes ***