SA WG2 Meeting #143E	S2-2100304r01

Feb 24 – Mar 9, 2021, Electronic

Source:	Futurewei, Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Cisco

Title:	UE Considerations for EAS rediscovery

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEdge_5GC /Rel-17

Abstract of the contribution: This contribution introduces considerations on OS/user for EAS (re-)discovery.

# 1	Discussion

The case that OS/user overrides operator DNS setting is part of the considerations in TR clause 9.1.1. The new text proposed below captures the recommendations and limitations in Annex X.

# 2	Proposal

It is proposed to adopt the following changes into TS23.548.]

* * * * 1st Change * * * *

# Annex X: UE considerations for EAS (re)discovery

## X.1	General

DNS records obtained from a network resolver contains a time-to-live (TTL) value. This is a hint provided by the network resolver and maybe used to determine the length of time that the record is cached. DNS records maybe be cached in the UE by a system wide stub resolver and by application layer name resolution caches. The application (L7) cache is managed on a per application basis while the OS/system DNS cache is common to applications. Name resolution caches in various applications also have different policies and behaviour. Some applications cache the name records for the length of the application session while others have a time limit. The network change indication from the OS/ system to the application can be used to clear the application layer name cache for the distributed and multiple anchor modes. Coordination across the 5GC and application domain for name resolver caching behaviour is required to support some of the solutions in this TS.

The following clauses describe the appropriate DNS configuration for the EAS (re)-discovery to work in the UE.

## X.2	Impact of Unicast/Anycast IP Addresses for DNS resolver and EAS

The UE/application may be configured to use unicast or anycast IP addresses for DNS and application messaging.

The UE may be configured with a unicast address for the MNO DNS resolver using ePCO or IPv6 Router Advertisement (RA). 5GC may reconfigure the MNO DNS resolver IP address for each network change event with the right MNO DNS resolver address. Alternatively, the UE may be configured with an IP anycast address for the MNO DNS resolver using ePCO or RA. In this case, the 5GC is responsible for routing to the “closest” instance of the MNO DNS resolver.

Applications in the UE may request the UE DNS resolver to resolve the FQDN. However, applications may also be configured with their own DNS resolver address may use encrypted messaging based on DoH, DoT. Configuration of application DNS resolvers is out of scope of 5GS. DNS messages (DoT, DoH) should be forwarded transparently to the destination address of DNS resolver in the DNS query. The application DNS resolver may be hosted by the MNO or in a 3rd party network.

The authoritative DNS server for the domain namespace translates the FQDN in the DNS request to an IP address for EAS using UE location information provided by the DNS resolver (see 6.2.2.2) and MNO resolvers cache the result.
If the IP address in the DNS response to the UE is a unicast IP address, it identifies an EAS instance. If the IP address in the DNS response is an anycast IP address, it identifies the set of EAS that provide the service.

A network interface change or NAS SM EAS rediscovery may result in the UE OS clearing name/IP address translations in its DNS cache. An application that receives the network interface change or NAS SM EAS rediscovery indications from the OS may clear its cache and resets its L4 connections. EAS rediscovery procedures are thus initiated at the next application message. An application that does not receive these indications from the OS or use anycast IP addresses may continue the L4 connection with the old EAS IP address until the application resets it.

EAS rediscovery or routing of anycast address from a new UE location can result in the selection of new EAS instances. If a new EAS instance is selected, there is no impact to pure datagram services like UDP. However, a connection-oriented service like TCP or QUIC will require the connection state to be re-established (along with TLS). Re-establishing TLS and application state are handled by application layer mechanisms and transparent to 5GS.

## X.3	UE requirements for EAS re-discovery

An application that complies with EAS (re-)discovery in the UE shall not override operator-provided DNS settings.

The OS DNS server configuration shall not be allowed to override the operator provided DNS in a UE compliant to the EAS (re-)discovery procedure. This is necessary for the “closest” EAS server to be selected.

NOTE 1:	If a non-compliant OS, user or non-compliant applications override the operator-provided DNS settings, the DNS resolvers or servers in the third party may take the source IP address of the DNS request as the location information of UE, which may correspond to the local/remote PSA UPF or other entities (e.g., a NAT server) on the N6 interface.

NOTE 2: If the DNS server configuration in a non-compliant OS overrides the operator provided DNS, the DNS queries continue to be sent over the correct PDU session for the application.

NOTE 3:  The UE modem transparently forwards DNS messages for tethered devices that are loosely coupled.

NOTE 4: 	If the UE (OS or application) uses a DNS Resolver that is different than the one provided by the operator, then the

-	the solutions in this TS will only work if the recursive DNS resolution using the non-operator provided DNS resolver gets UE location to resolve to the IP@ of the proximate EAS for that UPF-PSA/IP subnet (e.g., the operator is a CDN provider), and

-	for the session breakout modes A, B, C in 6.2.3.2, recursive DNS resolution should result in edge EASDF being in the operator DNS resolver chain.

## X.4	UE procedures for session breakout

In the session breakout connectivity model, the selection of a new session breakout path does not result in a new network interface indication at the UE.

NOTE 1:	In case of multiple sessions or distributed anchor point connectivity models, when there is a change of network interface, the UE operating system can provide an indication to notify the application that subscribes to it. This indication of network interface change may be used to flush the application name cache.

Session breakout results in a NAS SM message indicating the need to redo DNS lookup sent by the SMF to the UE modem. Thus, in order to support some solutions of this TS, it is necessary for the operating system to receive information of EAS rediscovery from the modem and use it to inform the application when a new session breakout path is selected and clear the application name cache.

Editor’s Note: Procedures to support EAS (re)discovery with encrypted DNS (DoH, DoT) are FFS. Mechanisms to selectively clear DNS cache entries in tethered device or corresponding to PDU session receiving NAS SM (EAS rediscovery) message are FFS.

* * * * End of Changes * * * *