SA WG2 Meeting #143e	S2-2100314r13

# Feb 24th – March 9th, 2021 ; Elbonia              		 								(revision of S2-210)

Source: 	Ericsson, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Samsung

Title: 	General Discovery aspects

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: This contribution captures common discovery aspects in one place in the TS.

# 1 Discussion

This contribution proposes to capture in a single place the aspects of EAS discovery which are common to all the connectivity models.

The proposal is based on text in the TR, in the KI definition and in the evaluation clause that should improve the understanding of this topic

# 2 Proposal

It is proposed to update TS 23.548 as follows:

*********************************Start Change********

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[6]	IETF RFC 7871: "Client Subnet in DNS Queries".

*********************************Stop Change********

********************************Start Change********

# 6	Procedures for Supporting Edge Computing

## 6.1	General

Editor’s Note: Any requirements on the applications and solution limitations are documented. For the detailed procedures for the management of the connectivity and run-time coordination with the application layer this clause refers to TS 23.502 and to TS 23.503 for the details on the Policy and Charging Control aspects.

## 6.2	EAS discovery and re-discovery

### 6.2.1	General

Editor’s Note: This clause describes general parts including e.g. privacy considerations, which DNS properties that are enabling DNS based Edge AS Discovery, recommendations/limitations for cases that OS/user overrides DNS setting.

In Edge Computing deployment, one application service may be served by multiple Edge Application Servers typically deployed in different sites. These multiple Edge Application Servers that host same content or service may use a single IP address (anycast address) or different IP addresses. Before an application/UE starts to connect to the service, the application/UE is required to discover the IP address of a suitable Edge Application Server (e.g. the closest one), so that the traffic can be locally routed to the Edge Application Server, and service latency, traffic routing path and user service experience can be optimized.

EAS Discovery is the procedure by which a UE discovers the IP address of a suitable Edge Application Server for an application using Domain Name System (DNS). EAS Re-discovery is the EAS Discovery procedure that takes place when the previously discovered Edge Application Server cannot be used or may have become non-optimal (e.g. at edge relocation) to discover an Edge Application Server to replace the old one to serve the application/UE.

NOTE:	This specification describes the discovery procedure based on 5GS NFs. However, this does not exclude any upper layer solution to be adopted by operator or service provider, like the EAS Discovery procedure of the Architecture for enabling Edge Applications defined in TS 23.558 [5], or other alternatives  shown in Annex A and Annex B.

EAS (re)discovery using DNS has no impact on the Application itself that is kept agnostic of the operator connectivity model chosen. The mechanism should enable the UE to be unaware of the application deployment (at edge or at central access to the DN) and application ownership (e.g. the EAS is owned by the MNO or by a third party).

For an optimal EAS (re)discovery the following is required:

-    The corresponding Authoritative DNS needs to support geographical resolution.

-	The operator DNS settings provided for the PDU Session need to be applied

Editor's Note: How to cooperate between the application, the network and the UE to ensure no conflict on selecting the appropriate DNS server is FFS.

In order to provide a translation of the FQDN of an EAS into the address of an EAS as topologically close as possible to the UE the Domain Name System may use following information:

- 	The source IP address of the incoming DNS Query, and/or,

-	a EDNS Client Subnet (ECS) option (as defined in RFC 7871 [xx])

If UE IP address is subject to privacy restrictions, it should not be sent to Authoritative DNS / DNS Resolvers outside the network operator within ECS option or as Source IP address of the DNS Query. UE source IP address can be protected by using NAT mechanism.

*********************************Stop Change********