SA WG2 Meeting #143e	S2-21000367r02

# Feb 24th – March 9th, 2021 ; Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | xx | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Informative guideline on supporting session/service continuity between SNPN and PLMN when using N3IWF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Futurewei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Supporting VIAPA (Video, Imaging and Audio for Professional Application) service is one of key issue of eNPN feature. It’s concluded that the existing Rel-16 N3IWF-architecture can be used as the basis to address data service from both networks and session/service continuity between the two networks. - For single radio UE, PDU session continuity can be realized by utilizing the existing handover procedure between non-3GPP access and 3GPP access for single access PDU session, where one network is acting as non-3GPP access of the other network. - For dual radio UE the UE can use one radio operating in SNPN access mode and the other operating the normal PLMN selection, in order to avoid SNPN access mode switch. This CR proposes informative guidelines in the annex on how to use existing release 16 mechanism to support session/service continuity between SNPN and PLMN for vertical applications, such as VIAPA service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide informative guideline on using existing N3IWF-architecture and mechanisms to support session/service continuity for vertical services, such as VIAPA service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Verticals industries lack of clear understanding how to realize session/service continuity between PLMN and SNPN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

Figure D.3-2: Access to Stand-alone Non-Public Network services via PLMN

# D.Y	Support for session/service continuity between SNPN and PLMN when using N3IWF

Depending on the UE’s radio capability and implementation, the following existing mechanisms can be used to allow session/service continuity between SNPN and PLMN:

-	For Single Radio UE which includes single Rx/Tx and dual Rx /single Tx UE, seamless service continuity is not supported in this release when the UE is moving between the 3GPP access networks of SNPN and PLMN because of the single radio limitation. But the PDU session continuity between SNPN and PLMN can be realized by utilizing the existing handover procedure between non-3GPP access and 3GPP access as defined in clause 4.9.2 in TS 23.502 [3], where one network is acting as non-3GPP access of the other network.

-	For Dual Radio (Dual Rx / Dual Tx) UE, the service continuity can be achieved by utilizing the existing handover procedure between non-3GPP access and 3GPP access as defined in clause 4.9.2 in TS 23.502 [3], where one network is acting as non-3GPP access of the other network, with following additional considerations:

-	for  PDU session on a single access, UE can register to the same 5GC via both Uu and NWu interfaces from two networks when it is possible, by following the procedure defined in clause 4.2.2 in TS 23.502 [3] if the registration is via Uu, or in clause 4.12.2 in TS 23.502 [3] if the registration is via NWu. The registration via NWu utilizes the user plane which is established in another 5GC using another network’s Uu interface. E.g. if UE is moving out of its SNPN 3GPP access coverage and would like to continue its SNPN service in PLMN, UE can register to its SNPN 5GC via PLMN’s 3GPP access network using NWu interface with SNPN’s 5GC before moving out SNPN NG-RAN coverage.

-	for MA PDU session , if supported by UE and network, with the 5GC can be established in both PLMN and SNPN via Uu and NWu interface as defined in clause 4.22.2.2 in TS 23.502 [3], where one network is acting as non-3GPP access of the other network (figure D.3.Z shows the example of UE with MA PDU towards SNPN home network when connected to SNPN 3GPP access and to PLMN 3GPP Access via N3IWF’s SNPN). Upon mobility, UE can add / activate the user plane resource to the corresponding access type basing on the procedures defined in clause 4.22.7 in TS 23.502 [3].

Figure D.3-Z: MA PDU session with ATSSS support for dual radio UE accessing to Stand-alone Non-Public Network services via PLMN and directly Stand-alone Non-Public Network 3GPP Access