SA WG2 Meeting #143e	S2-2100368r04

# Feb 24th – March 9th, 2021 ; Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2564 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Informative guideline on using existing mechanism for QoS Notification between SNPN and PLMN when using N3IWF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Futurewei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 23.700-07 concludes that when N3IWF based solution is used, the QoS notification between the underlay network and overlay network can be supported with the existing QoS notification procedure as shown below. This can be incorporated into TS23.501 as informative guideline because connectivity QoS monitoring is critical to the vertical industry, such as VIAPA. “ When N3IWF based solution is used, the overlay network and its service AF can use existing NEF notification procedures, such as of subscribing the "QoS monitoring" or "QoS sustainability" via the interface between NEF and AF, to subscribe and receive the notification from underlay network regarding the connectivity QoS status or QoS mapping changes that are associated with the IPsec of the overlay network. With the QoS update information from the underlay network, the overlay network can adjust its connectivity QoS accordingly. The opposite way is also applicable that the underlay network and its service AF can use existing NEF notification procedures, such as of subscribing the "QoS monitoring" or "QoS sustainability" via the interface between NEF and AF, to subscribe and receive the notification from overlay network regarding the connectivity QoS status or QoS mapping changes that are associated with the IPsec of the overlay network. With the QoS update information from the underlay network, the underlay network can adjust its connectivity QoS accordingly “ |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide informative guideline on using existing NEF notification procedures and mechanisms to support QoS notification between SNPN and PLMN for the vertical services, such as VIAPA service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of QoS notification support when verticals use N3IWF and PLMN’s network to access their SNPN services. NPN solution for vericals is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# D.3	Support for access to PLMN services via Stand-alone Non-Public Network and access to Stand-alone Non Public Network services via PLMN

Figure D.3-1: Access to PLMN services via Stand-alone Non-Public Network

NOTE 1:	The reference architecture in Figure D.3-1 and Figure D.3-2 only shows the network functions directly connected to the UPF or N3IWF and other parts of the architecture are same as defined in clause 4.2.

In order to obtain access to PLMN services when the UE is camping in NG-RAN of Stand-alone Non-Public Network, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the PLMN.

In the Figure D.3-1, the N1 (for NPN) represents the reference point between UE and the AMF in Stand-alone Non-Public Network. The Nwu (for PLMN) represents the reference point between the UE and the N3IWF in the PLMN for establishing secure tunnel between UE and the N3IWF over the Stand-alone Non-Public Network. N1 (for PLMN) represents the reference point between UE and the AMF in PLMN.

Figure D.3-2: Access to Stand-alone Non-Public Network services via PLMN

In order to obtain access to Non-Public Network services when the UE is camping in NG-RAN of a PLMN, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the Stand-alone Non-Public Network.

In Figure D.3-2, the N1 (for NPN) represents the reference point between UE and the AMF in the Stand-alone Non-Public Network. The NWu (for NPN) represents the reference point between the UE and the N3IWF in the stand-alone Non-Public Network for establishing a secure tunnel between UE and the N3IWF over the PLMN. The N1 (for PLMN) represents the reference point between UE and the AMF in PLMN.

When the solutions described above are used, and subjected to the agreement between PLMN and SNPN, the overlay network (PLMN in Figure D.3-1 and SNPN in Figure D.3-2) can act as authorized 3rd party with AF to interact with NEF in the underlay network (SNPN in Figure D.3-1, PLMN in Figure D.3-2) in order to use the existing network exposure capabilities defined in 4.15 in TS23.502 [3]. Overlay network AF can subscribe and receive the notifications regarding the connectivity, QoS status, changes of QoS parameters (e.g. 5QI, ARP) in the QoS mapping table in the SLA in the underlay network that are associated with the UE and the secure tunnel(s) between UE and N3IWF, such as:

- Subscribe and receive monitoring event (e.g. loss of connectivity, PDU session status defined in 4.15.3 in TS23.502).

- Subscribe and receive QoS analytic information (e.g. QoS sustainability defined in 6.9 in TS23.288[86]).

NOTE:  How underlay network can obtain UE location information to be used for some QoS analytic information subscriptions is up to implementation or the agreement between overlay network and underlay network.

With the QoS notification information from the underlay network, the overlay network can adjust its connectivity QoS accordingly based on the SLA using PDU Session modification procedure.

Subjected to the agreement between PLMN and SNPN, the opposite way is also applicable by the same mechanism as above.