3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100378

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

Title:	[DRAFT] Reply LS on 5MBS progress and issues to address

Response to:	(S2-2100142) LS on Reply LS on LS on 5MBS progress and issues to address, and (S2-2100110) Reply LS on 5MBS progress and issues to address

Release:	Rel-17

Work Item:	FS_5MBS, NR_MBS-Core

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:	SA4

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Agreed version of S2-2100345

1. Overall Description:

SA2 thanks RAN2 and RAN3 for their LSs on RAN impact of FS_5MBS Study. Regarding the exact questions that RAN2 and RAN3 asked SA2 to feedback on, SA2 would like to provide the following answers

In S2-2100142, RAN2 asks

RAN2 respectfully asks SA2 to take the above feedback into account and provide feedback on whether and what the difference is between session start and session activation and between the session stop and session deactivation.

And in S2-2100110, RAN3 asks

RAN3 would like to ask SA2 to clarify the differences between the following terms: session start/session activation and session deactivation/session stop and their implication to NG-RAN functions.

SA2 response:

SA2 agreed to remove session start/stop procedures towards RAN and only have activation/inactivation procedures for multicast. The following states were defined:

-	Active multicast session: Established multicast session in active state. Multicast data are transmitted to UEs that joined the multicast session. 5GC resources for the multicast session are reserved. Corresponding Radio resources are reserved depending on participating UE locations. UEs that joined the multicast session are in CM CONNECTED state. UEs are allowed to join the multicast session (subject to authorization check).

-	Inactive multicast session: Established multicast session in inactive state. No multicast data are transmitted. UEs that joined the multicast session may be in CM CONNECTED or CM IDLE state. UEs are allowed to join the multicast session (subject to authorization check).

For broadcast, only session start/stop are applicable.

More details are provided in the attached agreed PCR

In S2-2100110, RAN3 asks

Further, RAN3 would like to ask for confirmation from SA2 whether a UE is supposed to receive the MBS Session activation notification also when served by a non-supporting NG-RAN node.

SA2 response:

SA2 would like to confirm that it is necessary for UE to receive the MBS Session activation notification (e.g., legacy paging) when it is served by a non-supporting NG-RAN node.

SA2 follow-up question: SA2 asks RAN2/RAN3 for feedback on whether UEs camping on non-supporting NG-RAN nodes can be notified using MBS session ID or the 5GC is required to fallback to regular paging for UEs that have not connected during MBS session activation.

RAN3 would like to ask RAN2 and SA2 to consider how to page the UEs which are not in RRC_CONNECTED state, whether a group notification towards NG-RAN supporting MBS and, if applicable, to NG-RAN nodes not supporting MBS is needed/feasible/beneficial comparing to the legacy paging methods.

SA2 response:

SA2 concludes that it is beneficial, e.g. for signalling efficiency, to support 5GC requesting NG-RAN nodes to notify session activation of an MBS session to UEs based on MBS session ID, at least to supporting NG-RAN node.

SA2 follow-up question: SA2 requests RAN2 for confirmation whether NG-RAN node can notify session activation to UEs based on MBS session ID requires confirmation. SA2 normative work on this aspect will be pending RAN2 conclusion.

2. Actions:

To RAN2 and RAN3 group.

ACTION: 	SA2 respectfully asks RAN2 and RAN3 to take the above feedback into account, and provide feedback on the follow-up questions from SA2.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #144E		April 12 – 16, 2021			Elbonia

TSG-SA2 Meeting #145E		May 17 – 28, 2021			Elbonia