SA WG2 Meeting #143e	S2-2100397r01

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2568 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling restricted PDU Session for remote provisioning of UE via User Plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, Intel, Futurewei, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Implementing TR 23.700-07 conclusion on restricted PDU Session for remote provisioning of UE using User Plane in TS 23.501: - A restricted PDU session is supported to be dedicated for the remote provisioning. Upon successful establishment of restricted access PDU session, if the UE still does not have a PS address via any of the methods listed below, the device uses a well-known FQDN to perform PS discovery. - When Onboarding network is O-SNPN, the information required to restrict the usage is locally configured in the SMF, and the SMF restricts the usage for the UE registered for onboarding. - When Onboarding network is a PLMN, the functionality to restrict usage is activated for the UE by SMF based on received existing subscription profile from the UDM and on existing policies from PCF, or local configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New subclauses 5.30.X.2.8.3 and 5.30.X.3.4.2 describing the functionality of restricted PDU Session for remote provisioning of UE via User Plane in SNPN and PNI-NPN cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 eNPN feature not fully implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New subclauses 5.30.X.2.8.3 and 5.30.X.3.4.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START FIRST CHANGE

#### 5.30.2.X.4.3	User Plane Remote Provisioning of UEs

In case Onboarding Services are provided using a restricted PDU Session for remote provisioning of UE via User Plane, SNPN credentials are sent over the restricted PDU Session. Apart from the credentials, the parameters to enable UE access SO-SNPN are sent to UE by PVS over Control Plane. The AMF selects an SMF used for Onboarding Services using the SMF discovery and selection functionality as described in clause 6.3.2. The AMF Onboarding Configuration Data may contain S-NSSAI(s) and DNN(s) used for Onboarding to select an SMF used for Onboarding Services or may contain a configured SMF for the DNN used for Onboarding.

When a UPF is selected for Onboarding Services, the UPF selection function described in clause 6.3.3 for normal services is applied considering the DNN used for Onboarding.

The SMF or the PCF may store S-NSSAI and DNN information used for Onboarding. Onboarding Configuration Data available to PCF (for details see TS 23.503 [45]) and/or SMF may include Provisioning Server (PS) address(es).

In case a UE with a pre-configured Provisioning Server address receives a Provisioning Server address from the onboarding network during Registration procedure or PDU Session establishment procedure, the Provisioning Server address received from the Onboarding Network shall take precedence over the pre-configured Provisioning Server address. Otherwise, the UE shall use the pre-configured Provisioning Server address.

If the UE does not have a pre-configured Provisioning Server address and does not receive a Provisioning Server address from the Onboarding Network, the device uses a well-known FQDN to perform PS discovery.

When the UE does not have a Provisioning Server address available by any of the above means, the UE shall deregister from the Onboarding Network and select another Onboarding Network for onboarding purposes, if available.

Initial QoS parameters used for establishing Onboarding Services are configured in the SMF, when dynamic PCC is not used.

Dynamic PCC may be used for a PDU session that is established for Onboarding Services as described in TS 23.503 [45]).

The QoS Flows of a PDU Session associated with the restricted DNN for Remote Provisioning shall be dedicated to Onboarding Services. The UPF may block any traffic that is not from or to Provisioning Server address provided by the SMF.

For a UE that is registered for Onboarding:

-	the UE shall only initiate the UE Requested PDU Session Establishment procedure for Onboarding Services via S-NSSAI and DNN to be used for UE onboarding; and

-	the network should reject any PDU Session Establishment request from the UE for S-NSSAI and DNN that are not used for Onboarding.

END FIRST CHANGE

START SECOND CHANGE

#### 5.30.3.X.4.2 User Plane Remote Provisioning of UEs

When Onboarding network is a PLMN, the functionality to restrict network usage is activated for the UE by the SMF based on a subscription profile used for onboarding received from UDM (e.g. containing S-NSSAI, DNN) and on policies from PCF, or local configuration in the SMF.

END SECOND CHANGE