SA WG2 Meeting #143e	S2-2100397r13

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2568 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling restricted PDU Session for remote provisioning of UE via User Plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, Intel, Futurewei, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Implementing TR 23.700-07 conclusion on restricted PDU Session for remote provisioning of UE using User Plane in TS 23.501: - A restricted PDU session is supported to be dedicated for the remote provisioning. Upon successful establishment of restricted access PDU session, if the UE still does not have a PS address via any of the methods listed below, the device uses a well-known FQDN to perform PS discovery. - When Onboarding network is O-SNPN, the information required to restrict the usage is locally configured in the SMF, and the SMF restricts the usage for the UE registered for onboarding. - When Onboarding network is a PLMN, the functionality to restrict usage is activated for the UE by SMF based on received existing subscription profile from the UDM and on existing policies from PCF, or local configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New subclauses describing the functionality of restricted PDU Session for remote provisioning of UE via User Plane in SNPN and PNI-NPN cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 eNPN feature not fully implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X.7.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START FIRST CHANGE

5.30.2.X.4.3	User Plane Remote Provisioning of UEs in SNPNs

In case Onboarding Services are provided using a restricted PDU Session for remote provisioning of UE via User Plane, the AMF selects an SMF used for Onboarding Services using the SMF discovery and selection functionality as described in clause 6.3.2. The AMF Onboarding Configuration Data may contain S-NSSAI(s) and DNN(s) used for Onboarding to select an SMF used for Onboarding Services or may contain a configured SMF for the DNN used for Onboarding.

When a UPF is selected for Onboarding Services, the UPF selection function described in clause 6.3.3 for normal services is applied considering the DNN used for Onboarding.

The SMF or the PCF may store S-NSSAI and DNN information used for Onboarding. Onboarding Configuration Data available to PCF (for details see TS 23.503 [45]) and/or SMF may include Provisioning Server (PS) address(es).

Configuration data for UP Remote Provisioning is comprised of Provisioning Server address (IP address information or FQDN), and Configuration data for UP Remote Provisioning may be stored in the ME or on the UICC. SMF may provide Configuration data for UP Remote Provisioning to the UE during PDU Session Establishment procedure as part of extended Protocol Configuration Options (PCO) in PDU Session Establishment Response or during Registration procedure over Registration Accept. In case a UE with a pre-configured Provisioning Server address receives a Provisioning Server address from the onboarding network during Registration procedure or PDU Session Establishment procedure, the Provisioning Server address received from the Onboarding Network shall take precedence over the pre-configured Provisioning Server address. Otherwise, the UE shall use the pre-configured Provisioning Server address.

Editor's Note:	How the ONN and specifically the SMF is aware of Provisioning Server address and How the ONN determines the Provisioning Server address for the UE when there are multiple Provisioning Server addresses are FFS.

If the UE does not have a pre-configured Provisioning Server address and does not receive a Provisioning Server address from the Onboarding Network, the UE uses a well-known FQDN to perform PS discovery.

When the UE does not have a Provisioning Server address available by any of the above means, the UE may deregister from the Onboarding Network and then select another Onboarding Network for onboarding purposes, if available.

When the UE registered for Onboarding successfully completes the user plane remote provisioning of SNPN credentials via the restricted PDU Session, then the UE shall initiate the release of the PDU Session and deregister from the Onboarding Network. After deregistering from the Onboarding Network the UE can select and register to a SNPN using the received SNPN credentials.

Initial QoS parameters used for establishing Onboarding Services are configured in the SMF, when dynamic PCC is not used.

Dynamic PCC may be used for a PDU session that is established for Onboarding Services as described in TS 23.503 [45]).

The QoS Flows of a PDU Session associated with the restricted DNN for Remote Provisioning shall be dedicated to Onboarding Services. The UPF may block any traffic that is not from or to Provisioning Server address provided by the SMF.

If the UE is registered for Onboarding, the network should reject any PDU Session Establishment request from the UE for S-NSSAI and DNN that are not used for Onboarding Services.

END FIRST CHANGE

END SECOND CHANGE