3GPP TSG SA WG2 Meeting #143E	S2-2100398r02

# Elbonia, February 24 – March 09, 2021	(Revision of S2-2100224)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2475 | rev | 1 | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of DN-AAA interactions in case of EPS IWK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Cisco |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Secondary authentication/authorization with a DN-AAA Server is supported in 5GS, but it has not been defined how to handle DN-AAA server interactions when the UE moves to EPS, where e.g. EAP-based secondary authentication is not supported. It is e.g. unclear what will happen in case the DN-AAA Server initiates a re-authentication when the UE is in EPS. EAP based secondary authentication/re-authentication is not supported over EPS because this impacts UE and EPS procedures do not support this interaction. EAP based secondary authorization/re-authorization can be supported over EPS because authorization/re-authorization is a transaction between SMF and DN-AAA server and does not impact the UE. SMF+PGW-C already supports this procedure when UE attaches via NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify DN-AAA interactions when UE is in EPS, e.g. how to handle a request for EAP-based re-authentication or re-authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to handle a DN-AAA request after UE has moved to EPS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First change ***************

#### 4.11.0a.6	Interactions with DN-AAA Server

In this version of the specification, EAP-based secondary authentication with a DN-AAA server, as specified in clause 4.3.2.3, is not supported when the UE is in EPS. EAP-based secondary authorization with a DN-AAA server, as specified in clause 4.3.2.3, is supported when the UE is in EPS

In case secondary authentication/authorization has been performed for the PDU Session in 5GS, and the UE has moved to EPS, the following applies:

-	DN-AAA re-authorization (without EAP-based re-authentication) can be performed even when the UE is in EPS, e.g. to provide new parameters from the DN-AAA Server to SMF+PGW-C.

-	EAP-based re-authentication cannot be performed while the UE is in EPS. In case the SMF+PGW-C receives a re-authentication request from the DN-AAA, the SMF+PGW-C informs the DN-AAA Server that the UE is not available for EAP-based re-authentication at the moment. The SMF+PGW-C should not initiate PDN connection release. The DN-AAA decides the actions to take, based on the reply from SMF+PGW-C which may induce a DN-AAA server request to release the PDU Session.

*************** End of changes ***************