SA WG2 Meeting #143E (e-meeting)	S2-2100421

# Feb 24 – Mar 09, 2021 									                              (revision of S2-XXXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2572 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | T2: UE Configuration Aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusions for “Remote provisioning for SNPN credentials (Component 2 of KI#4)” in clause 8.4.1 of TR23.700-07, Onboarding configuration is introduced to User Plane Remote Provisioning for SNPN credentials. Provisioning of PS address to the UE can be supported using one of the following methods: - a. SMF may deliver onboarding configuration data as part of extended Protocol Configuration Options (PCO) in PDU Session Establishment Response to UE. This is similar to use of PCO to configure Autoconfiguration server for UE in Wireless and Wireline Convergence (TR 23.716 [28] clause 6.10). - b. Alternatively, onboarding configuration data may be configured in the UE during Registration Procedure. - c. In addition, onboarding configuration data may be configured in the UE using service specific policies subject to UE capabilities similar to what is used for V2X communications as specified in TS 23.287 [29] clause 5.1.1 for ways how parameters may be made available to the UE and TS 23.287 [29] clause 6.2.5 for AF-based service parameter provisioning and TS 24.587 [30] clause 5.2.4 for configuration parameters such as validity timer, server address and geographical area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduece the configuration for UP Remote Provisioning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Onboarding configuration for UP provisioning for UE is not introduced |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.X.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

5.30.X.2.7.2	Onboarding configuration for the UE

Editor’s Note:	This clause should include the configuration data in the UE for UP provisioning.

In order to enable the UP remote provisioning for a UE supporting UP Remote Provisioning, the Configuration for UP Remote Provisioning is pre-configured on the UE or provided by the ON. The Configuration for UP Remote Provisioning provided by the ON is prioritized, if any corresponding configuration stored in the UE.

The Configuration for UP Remote Provisioning includes:

- The PS address information, including the IP address, or the FQDN for the PS.

- The SNPN ID (i.e. the combination of a PLMN ID and Network identifier (NID) for the SNPN). The SNPN’s credential is provisioned by the PS for an Onboarding UE, each SNPN is identified by SNPN ID.

The Configuration for UP Remote Provisioning provided by the ON shall be integrity protected. When the Onboarding-enabled UE access to the ON, after successful authentication, authorization and the UE supports the User Plane Remote Provisioning, one of the following should be supported:

- The AMF may include PS address in the Registration Accept message; The Configuration for UP Remote Provisioning is preconfigured on the AMF or stored in the UDR. The AMF use the SNPN ID provide by the UE to index the corresponding PS address.

- The SMF may deliver PS address during the restricted PDU Session setup for the user plane remote provisioning; The Configuration for UP Remote Provisioning is preconfigured on the SMF or stored in the UDR. The SMF use the SNPN ID provide by the UE to index the corresponding PS address.

The AF may provide the Configuration for UP Remote Provisioning by the Service specific parameter provisioning procedure (see TS 23.502 [3] clause 4.15.6.2). The Configuration for UP Remote Provisioning provided by the AF is stored in the UDR.

The Configuration for UP Remote Provisioning preconfigured on the UE or configured by the ON may be stored in the ME or the in the UICC.

***** End of changes *****