SA WG2 Meeting #143E (e-meeting)	S2-2100423

# Feb 24 – Mar 09, 2021 									                              (revision of S2-XXXXXX)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2573 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the architectures for Time Sensing Communication other than TSN. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIOT |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The arichitecture for TSN is not suitable for TSC in the IP network or ETH network other than TSN, because 1) the End Station device may behind the UE or may be the UE, i.e. the Application client on the UE is the service consumer for Time Sensitive Communication ; 2) the time synchronization may not be required. Even if the time synchronization is required but the GM is configured to 5G GM, the UE has supported to read the reference time from the SIB and to synchronised with the referece time. Considering to this, the DS-TT and NW-TT may not be required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the architectures for Time Sensing Communication in IP network and ETH network which doesn ‘t support TSN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The architectures for Time Sensing Communication in IP network and ETH network, which doesn ‘t support TSN, is not introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8.1, 4.4.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

4.4.8	Time Sensitive Communication

4.4.8.1	General

The 5G System is extended to support Time sensitive communication as defined in IEEE 802.1 Time Sensitive Networking (TSN) standards.

This Release supports of the specification, the following communications are supported:

-    integration of the 5G System with IEEE 802.1 TSN networks that apply the fully centralized configuration model as defined in IEEE Std 802.1Qcc [95]. IEEE TSN is a set of standards to define mechanisms for the time-sensitive (i.e. deterministic) transmission of data over Ethernet networks.

-    Time Sensitive Communication in IP network.

-    Time Sensitive Communication in ETH network, which doesn’t support TSN.

4.4.8.2	Architecture to support Time Sensitive Communication

In order to support the TSN, the 5G System is integrated with the external network as a TSN bridge. This "logical" TSN bridge (see Figure 4.4.8.2-1) includes TSN Translator functionality for interoperation between TSN System and 5G System both for user plane and control plane. 5GS TSN translator functionality consists of Device-side TSN translator (DS-TT) and Network-side TSN translator (NW-TT). The TSN AF is part of 5GC and provides the control plane translator functionality for the integration of the 5GS with a TSN network, e.g. the interactions with the CNC. 5G System specific procedures in 5GC and RAN, wireless communication links, etc. remain hidden from the TSN network. To achieve such transparency to the TSN network and the 5GS to appear as any other TSN Bridge, the 5GS provides TSN ingress and egress ports via DS-TT and NW-TT. DS-TT and NW-TT optionally support:

-	hold and forward functionality for the purpose of de-jittering;

-	per-stream filtering and policing as defined in IEEE Std 802.1Q [98] clause 8.6.5.1.

DS-TT optionally supports link layer connectivity discovery and reporting as defined in IEEE Std 802.1AB [97] for discovery of Ethernet devices attached to DS-TT. NW-TT supports link layer connectivity discovery and reporting as defined in IEEE Std 802.1AB [97] for discovery of Ethernet devices attached to NW-TT. If a DS-TT does not support link layer connectivity discovery and reporting, then NW-TT performs link layer connectivity discovery and reporting as defined in IEEE Std 802.1AB [97] for discovery of Ethernet devices attached to DS-TT on behalf of DS-TT.

NOTE 1:	If NW-TT performs link layer connectivity discovery and reporting on behalf of DS-TT, it is assumed that LLDP frames are transmitted between NW-TT and UE on the QoS Flow with the default QoS rule as defined in the clause 5.7.1.1. Alternatively, SMF can establish a dedicated QoS Flow matching on the Ethertype defined for LLDP (IEEE Std 802.1AB [97]).

There are three TSN configuration models defined in IEEE Std 802.1Qcc [95]. Amongst the three models:

-	fully centralized model is supported in this Release of the specification;

-	fully distributed model is not supported in this Release of the specification;

-	centralized network/distributed user model is not supported in this Release of the specification.

NOTE 2:	This Release supports interworking with TSN using IEEE Std 802.1Q [98] clause 8.6.8.4 scheduled traffic and IEEE Std 802.1Q [98] clause 8.6.5.1 per-stream filtering and policy.

Figure 4.4.8.2-1: System architecture view with 5GS appearing as TSN bridge

NOTE 3:	Whether DS-TT and UE are combined or are separate is up to implementation.

NOTE 4:	TSN AF does not need to support N33 in this release of the specification.

In order to support Time Sensitive Communication in IP network or ETH network which doesn't support TSN, in the logical 5G Bridge (see Figure 4.4.8.2-2), the End Station device may behind the UE or may be the UE, i.e. the Application client on the UE is the service consumer for Time Sensitive Communication; and the DS-TT and NW-TT may or may not be required, as the following:

- DT-TT is required, when 1) the End Station device for the Time Sensitive Communication is behind the UE and is connect to the UE and 5G network via the DS-TT; or 2) the Time Sensitive Communication require the time synchronization and the GM is the external GM. When the UE is the End Station, the DS-TT is collocated with the UE for this case. When the Time Sensitive Communication requires the time synchronization and the GM is the external GM, the NW-TT is also required in the 5G network.

- DS-TT is not required, when the End Station device is the UE and the Time Sensitive Communication doesn’t require the time synchronization or the Time Sensitive Communication requires the time synchronization but the GM is configured to be 5G GM.

Figure 4.4.8.2-2: System architecture1 for 5G Bridge in IP network and ETH network which doesn’t support TSN

END OF CHANGES