SA WG2 Meeting #143E (e-meeting)	S2-2100423r10

# Feb 24 – Mar 09, 2021 									                              (revision of S2-XXXXXX)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2573 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the architectures for Time Sensing Communication other than TSN. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, ZTE, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIOT |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The architecture for integration with IEEE 802.1 TSN is not suitable for TSC in the IP network or ETH network other than TSN, because 1) the End Station device may behind the UE or may be the UE, i.e. the Application client on the UE is the service consumer for Time Sensitive Communication ; 2) the time synchronization may not be required. Even if the time synchronization is required but the GM is configured to 5G GM, the UE has supported to read the reference time from the SIB and to synchronised with the referece time. Considering this, the DS-TT and NW-TT may not be required always. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the architectures for Time Sensitive Communication for IP and ETH sessions that do notsupport IEEE 802.1 TSN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The architectures for Time Sensitive Communication in IP network and ETH network, which doesn ‘t support TSN, is not introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8.1, 4.4.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

4.4.8	Architecture to enable Time Sensitive Communication and Time Synchronization

4.4.8.1	General

The 5G System can be extended to support the following:

a) 	Integration as a bridge in an IEEE 802.1 Time Sensitive Networking (TSN). The 5GS bridge supports the Time sensitive communication as defined in IEEE 802.1 Time Sensitive Networking (TSN) standards. The architecture is described in clause 4.4.8.2.

This Release supports of the specification, integration of the 5G System with IEEE 802.1 TSN networks that apply the fully centralized configuration model as defined in IEEE Std 802.1Qcc [95]. IEEE TSN is a set of standards to define mechanisms for the time-sensitive (i.e. deterministic) transmission of data over Ethernet networks.-

b) 	Enablers for AF requested support of Time Sensitive Communication and Time Synchronization. The architecture is described in clause 4.4.8.3.

4.4.8.2	Architecture to support IEEE Time Sensitive Networking

tThe 5G System is integrated with the external network as a TSN bridge. This "logical" TSN bridge (see Figure 4.4.8.2-1) includes TSN Translator functionality for interoperation between TSN System and 5G System both for user plane and control plane. 5GS TSN translator functionality consists of Device-side TSN translator (DS-TT) and Network-side TSN translator (NW-TT). The TSN AF is part of 5GC and provides the control plane translator functionality for the integration of the 5GS with a TSN network, e.g. the interactions with the CNC. 5G System specific procedures in 5GC and RAN, wireless communication links, etc. remain hidden from the TSN network. To achieve such transparency to the TSN network and the 5GS to appear as any other TSN Bridge, the 5GS provides TSN ingress and egress ports via DS-TT and NW-TT. DS-TT and NW-TT optionally support:

-	hold and forward functionality for the purpose of de-jittering;

-	per-stream filtering and policing as defined in IEEE Std 802.1Q [98] clause 8.6.5.1.

DS-TT optionally supports link layer connectivity discovery and reporting as defined in IEEE Std 802.1AB [97] for discovery of Ethernet devices attached to DS-TT. NW-TT supports link layer connectivity discovery and reporting as defined in IEEE Std 802.1AB [97] for discovery of Ethernet devices attached to NW-TT. If a DS-TT does not support link layer connectivity discovery and reporting, then NW-TT performs link layer connectivity discovery and reporting as defined in IEEE Std 802.1AB [97] for discovery of Ethernet devices attached to DS-TT on behalf of DS-TT.

NOTE 1:	If NW-TT performs link layer connectivity discovery and reporting on behalf of DS-TT, it is assumed that LLDP frames are transmitted between NW-TT and UE on the QoS Flow with the default QoS rule as defined in the clause 5.7.1.1. Alternatively, SMF can establish a dedicated QoS Flow matching on the Ethertype defined for LLDP (IEEE Std 802.1AB [97]).

There are three TSN configuration models defined in IEEE Std 802.1Qcc [95]. Amongst the three models:

-	fully centralized model is supported in this Release of the specification;

-	fully distributed model is not supported in this Release of the specification;

-	centralized network/distributed user model is not supported in this Release of the specification.

NOTE 2:	This Release supports interworking with TSN using IEEE Std 802.1Q [98] clause 8.6.8.4 scheduled traffic and IEEE Std 802.1Q [98] clause 8.6.5.1 per-stream filtering and policy.

Figure 4.4.8.2-1: System architecture view with 5GS appearing as TSN bridge

NOTE 3:	Whether DS-TT and UE are combined or are separate is up to implementation.

NOTE 4:	TSN AF does not need to support N33 in this release of the specification.

4.4.8.3	Architecture for AF requested support of Time Sensitive Communication and Time Synchronization

This clause describes 5G System features to support AF requested time sensitive communication and time synchronization services that are applicable for IP and Ethernet PDU Session Types. In order to support time sensitive communication, Figure 4.4.8.3-1 illustrates the architecture for AF requested support of 5GS TSC and Time Synchronization services.

To support Time Synchronization for Ethernet or IP type PDU Sessions, the DS-TT, NW-TT and NEF support the features in IEEE 802.1AS [104] or IEEE 1588-2019 [xx] as described in clause 5.27. The NEF exposes 5GS capability to support Time Synchronization service as described in clause 5.27.1.z.

Figure 4.4.8.3-1: System architecture view with 5GS Supporting Time Sensitive Communication

Editor’s note: Whether DS-TT and NW-TT is required if the 5GS does not expose Ethernet ports (i.e. is not a 5GS bridge) nor PTP ports (i.e. is not a PTP instance) is FFS.

END OF CHANGES