SA WG2 Meeting #143e	S2-2100444r02

24th February – 9th March, 2021, Electronic

Source:	Ericsson

Title:	KI#1, Conclusions update for mobility

Document for:	Approval

Agenda Item:	8.2.1

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This contribution proposes updates for key issue 1 mobility conclusions.

# 1.	 Discussions

In S2-2100143, RAN2 asked SA2 whether external authentication related parameters can be assumed uniform across a network or registration area.

During SA2-143e we concluded that support of the feature should be homogeneous in a SNPN. This implies that no changes are needed in mobility handling within the SNPN to handle the scenario of inhomogeneous support of the feature.

Even if TR 23.700-07 already mentions that mobility scenarios can be addressed with legacy mechanisms, it would be helpful to add specific examples for further clarification.

Proposal 1: Clarify that support of authentication using credentials from an external entity is homogenous throughout a SNPN and no changes are needed in mobility procedures..

In addition, TR 23.700-07 does not mention UEs in RRC inactive state. Existing resume functionality can be re-used as described in TS 23.502, clause 4.8.2.2. For completeness, this should also be added.

Proposal 2: Clarify that existing functionality can be re-used for UEs in RRC inactive state in case there is a common AMF.

# 2. Proposal

It is proposed to update TR 23.700-07, v1.2.0 as follows.

*** Start of changes ***

### 8.1.2	Conclusions for mobility scenarios

The mobility procedures are based on:

-	In the case that there are common AMF and/or N14 interface between the source network and target network, mechanism defined in TS 23.502 [6] clause 4.9.1 is re-used to address UE mobility.

-	In the case that there is a common AMF, a UE in RRC inactive state, mechanism defined in TS 23.502 [6] clause 4.8.2.2 is re-used to address UE mobility.

-	In the case of idle mode mobility, the UE performs initial or mobility registration as specified in clause 4.2.2.2.2 of TS 23.502 [6].

-	Support of authentication using credentials from an external entity is homogenous throughout a SNPN i.e., the SIB information in clause 8.1.4 should be set uniformly and no changes in mobility handling are needed to address inhomogeneous support of the feature.

.

NOTE:	Needed updates to find the correct source or target AMF and what are the applicable UE identities in the registration message will be determined in normative phase.

*** End of changes ***