3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100470r02

# Elbonia, February 24 – March 09, 2021								            (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0213 | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Hierarchical Principles and Interactions on Multiple NWDAFs in TS23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To consolidate in TS 23.288 the conclusions in TR.23.700-91 clause 8.2.1 related to the hierarchical NWDAFs: - “a) The NWDAF architecture should allow for a hierarchy/tree with flexible number of layers/branches. The number and organisation of the layers, as well as the capabilities of each NWDAF instance remain deployment choices.” - “e) NWDAFs and NWDAF service consumers use the discovery mechanism from NRF as defined in clause 6.3.13, TS 23.501 [2] to identify NWDAFs with certain capabilities (e.g. Analytics aggregation), covering certain area of interest (e.g. providing data / analytics for specific TAI(s)).” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the principles of hierarchical NWDAFs and NWDAF interactions for NWDAF in hierarchical deployment to efficiently collect data from other NWDAFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description of NWDAF enhancements in R17 is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 4.X, 6.2.1, New 6.2.X, New 6.2.X.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change ALL NEW TEXT* * * *

## 4.X	Architecture for Multiple NWDAF Instances

### 4.X.1	Hierarchical Interactions Among NWDAFs

The NWDAF architecture allows for arranging multiple NWDAF instances in a hierarchy/tree with a flexible number of layers/branches. The number and organisation of the hierarchy layers, as well as the capabilities of each NWDAF instance remain deployment choices.

The hierarchical interactions among NWDAFs can be either dynamic established or can be fixed by deployment configuration.

In the case of dynamic established hierarachical interactions, the tree and branches of the hierarchical NWDAFs can dynamically be created based on the needs for a particular analytics request.

In the case of fixed hierarchical interactions, there exist a predefined hierarchy among the NWDAFs and when a particular analytics request reaches one of the NWDAFs of the hierarchy, such NWDAF interacts with other NWDAFs in the immediate lower levels of the hierarchy in order to generate the requested analytics ID.

The lower level NWDAFs can be co-located with NFs.

The discovery of NWDAFs is performed as defined in TS 23.501 Clause 6.13.1.

Editor’s note: It is for FFS to determine if and which extra information needs to be registered in NRF for the discovery of NWDAFs in a fixed hierarchy.

In hierarchical interactions:

-	NWDAF may provide data collection exposure capability for generating analytics based on the data collected by other NWDAFs in the hierarchy, when DCCF, MFAF are not present in the network.

-	NWDAF may provide the analytics aggregation capability for aggregation of analytics exposed by NWDAFs in the hierarchy.

-	In fixed hierarchies, NWDAF Service Consumer can reach any NWDAF of the hierarchy.

NOTE: It is defined by configuration on the NWDAFs of the hierarchy which option among data collection or analytics aggregation from other NWDAFs is performed for the requested analytics generation.

Editor's note:	It is FFS to align the data collection exposure capabilities of NWDAFs in a hierarchical deployment with the further definitions of NWDAF data exposure capabilities.

# * * * * Second change * * * *

### 6.2.1	General

The Data Collection feature permits NWDAF to retrieve data from various sources (e.g. NF such as AMF, SMF, PCF, and AF; OAM), as a basis of the computation of network analytics.

All available data encompass:

-	OAM global NF data,

-	Data available in NFs, e.g. behaviour data related to individual UEs or UE groups (e.g. UE reachability), and pre-computed metrics covering UE populations (e.g. number of UEs present in a geographical area), per spatial and temporal dimensions (e.g. per region for a period of time),

-	NF data available in the 5GC (e.g. NRF),

-	Data available in AF.

The NWDAF shall use at least one of the following services:

-	the Generic management services as defined in TS 28.532 [6], the Performance Management services as defined in TS 28.550 [7] or the Fault Supervision services as defined in TS 28.545 [9], offered by OAM in order to collect OAM global NF data.

-	the Exposure services offered by NFs in order to retrieve data and other non-OAM pre-computed metrics available in the NFs.

-	Other NF services in order to collect NF data (e.g. NRF)

The NWDAF shall obtain the proper information to perform data collection for a UE, a group of UEs or any UE:

-	For an Analytics ID, NWDAF is configured with the corresponding NF Type(s) and/or event ID(s) and/or OAM measurement types.

-	NWDAF shall determine which NF instance(s) of the relevant NF type(s) are serving the UE, the group of UEs or any UE, taking into account the S-NSSAI(s) and area of interest as defined in clause 7.1.3, TS 23.501 [2].

-	NWDAF invokes Nnf_EventExposure_Subscribe services to collect data from the determined NF instance(s), and/or triggers the procedure in clause 6.2.3.2 to subscribe to OAM services to collect the OAM measurement.

The NWDAF performs data collection from an AF directly as defined in clause 6.2.2.2 or via NEF as defined in clause 6.2.2.3.

The NWDAF shall be able to discover the events supported by a NF.

Data collection procedures enables the NWDAF to efficiently obtain the appropriate data with the appropriate granularity.

When a request or subscription for statistics or predictions is received, the NWDAF may not possess the necessary data to perform the service, including:

-	Data on the monitoring period in the past, which is necessary for the provision of statistics and predictions matching the Analytics target period.

-	Data on longer monitoring periods in the past, which is necessary for model training.

Therefore, in order to optimize the service quality, the NWDAF may undertake the following actions:

-	The NWDAF may return a probability assertion as stated in clause 6.1.3 expressing the confidence in the prediction produced. Prediction may be returned with zero confidence as described below. This confidence is likely to grow in the case of subscriptions.

-	The value of the confidence depends on the level or urgency expressed by the parameter "preferred level of accuracy of the analytics" as listed in clause 6.1.3, the parameter "time when analytics information is needed" as listed in clause 6.1.3, and the availability of data. If no sufficient data is collected to provide an estimation for the requested level of accuracy before the time deadline, the service shall return a zero confidence. Otherwise, the NWDAF may wait until enough data is collected before providing a response or a first notification.

-	In order to be prepared for future requests on analytics from NFs/OAM, the NWDAF, upon operator configuration, may collect data on its own initiative, e.g. on samples of UEs, and retain the data collected in the data storage.

NOTE 1:	The NWDAF can send an error response to the analytics consumer to indicate that statistics are unavailable if the NWDAF was not prepared for future requests and did not collect data on its own initiative.

The volume and maximum duration of data storage is also subject to operator configuration.

The NWDAF may decide to reduce the amount of data collected to reduce signalling load, by either prioritizing requests received from analytics consumers, or reducing the extent (e.g. duration, scope) of data collection, or modifying the sampling ratios.

The NWDAF may skip data collection phase when the NWDAF already has enough information to provide requested analytics.

The data which NWDAF may collect is listed for each analytics in input data clause and is decided by the NWDAF.

NOTE 2:	NWDAF can skip data collection phase for some specific input data per the requested analytics e.g. when some of the data is already available at NWDAF for the requested analytics, or when NWDAF considers that some of the data is not needed at all to provide the requested analytics as per the analytics consumer request (e.g. based on preferred level of accuracy or based on the time when analytics are needed).

In fixed hierarchical interactions among NWDAFs, without standalone DCCF, or co-located DCCF, the efficiency of data collection can be achieved by inter-NWDAF instance cooperation among NWDAF instances on different levels of the hierarchy.  An efficient data collection means that the same data required for the same Analytics ID or different Analytics IDs should not be collected multiple times, instead, the data collection should be triggered by the first NWDAF requesting the data and once the data is available it can be distributed to the other NWDAFs requesting the same data. The same data refers to the same data type to be collected from the same source NFs or the aggregated data or the analytics data to be collected from other NWDAF instances.

# * * * * Third change – ALL NEW TEXT* * * *

#### 6.2.X.	Data Collection with fixed hierarchical NWDAF interactions

When no standalone DCCF or co-located DCCF with NWDAF are present in the network, a fixed hierarchy is used by operator, and the data collection is the configuration option for analytics generation, multiple NWDAF instances (hosting DCCF) can cooperate with each other to support efficient data collection, i.e. to avoid the duplicatied collection of the same data (i.e., raw data, aggregated data). To achieve this, the NWDAF on a higher level should first check with the NWDAF on lower levels if the required data is available before triggering data collection from the data sources (e.g., NFs, OAM). Figure 6.2.x.1-1 depicts the procedure for data collection in a hierarchical NWDAF.

Figure 6.2.X.1-1- Data Collection in Fixed Hierarchical NWDAF Interactions

- NWDAF Service Consumer invokes NRF services to discovery the NWDAF matching the search criteria based on analytics ID and NWDAF Serving Area. NRF returns the NWDAF in any of the levels of the hierarchy that is able to provide the required analytics ID (Figure 6.2.X.1-1 illustrates the case where the returned NWDAF is the Top Level NWDAF).

- An Analytics consumer requests to the returned NWDAF, e.g., Top Level NWDAF of the hierarchy, for an Analytics subscription or request of a specific Analytics ID.

- The Top Level NWDAF acquires or is configured with the corresponding NF Type(s) and/or event ID(s) required to generate the requested Analytics ID. The Top Level NWDAF checks its local association of   analytics IDs to analytics ID subscription identification and to the list of triggered event subscription identifications from NFs, OAM.  Based on this local association, the Top Level NWDAF checks if the data to be collected is available at itself. If the data to generate the analytics ID is already stored and available at the Top Level NWDAF, it uses the information to calculate the analytics.

- [Optional - Depending on the step 2] If the data is not available, the Top Level NWDAF determines the NWDAFs in the lower levels (e.g., Middle Level NWDAF) to collect the data for analytics generation. If NWDAF does not know the NWDAFs in the lower levels, the Top Level NWDAF invokes NRF discovery services to identify the proper NWDAFs as described in Clause 4.X.1.

- [Conditional - depending on the step 2] If the data is not available, the Top Level NWDAF sends a subscription/request for specific data (e.g., Event ID(s)) to all NWDAF(s) in the middle level required for the analytics generation and updates the local association with the mapping of analytics ID to the identification of the request/subscription for data collection from the further NWDAF.

Editor's note:	The NWDAF services for data collection listed in Figure 6.2.x.1-1 need to be aligned with the definition of the NWDAF services exposed for data collection.

- [Conditional - depending on the step 3] Each Middle Level NWDAF receiving a request checks the collected data for the received subscription/request based on its local association (similar to step 2). If the data is already stored and available at the middle level, each Middle Level NWDAF sends/notifies the data to the Top Level NWDAF.

- [Optional - depending on the step 5] If the requested data is not available at the Middle Level NWDAF, the Middle Level NWDAF determines the NWDAFs in the lower levels (e.g., Lower Level NWDAF) to collect the data. If Middle Level NWDAF does not know the NWDAFs in the lower levels, the Middle level NWDAF invokes NRF discovery services to identify the proper NWDAFs as described in Clause 4.X.1.

- [Conditional - depending on the step 5] If the requested data is not available at the Middle Level NWDAF, the Middle Level NWDAF sends a subscription request for specific data (e.g., Event ID(s)) to each required NWDAFs in the Lower Level.

- [Conditional - depending on the step 7] Each Lower Level NWDAF receiving a request checks the collected data for the received subscription/request based on its local association (similar to step 2 and 4). If an existing subscription to the same request Event ID is already available, the Lower Level NWDAF sends the requested data to the Middle Level NWDAF. If there is no existing subscription to the same request Event ID, the Lower Level NWDAF subscribes/request the required data to the data source, e.g. NF/OAM, and updates its local association with the mapping of the requested data to the triggered event subscription identifications.

- [Conditional - depending on the step 8] Each Lower Level NWDAF stores the collected data and sends the data to the Middle Level NWDAF that requested it.

- [Conditional - depending on the step 5-9] Each Middle Level NWDAF stores the collected data and sends the data to the Top Level NWDAF that requested it.

NOTE 2: 	For hierarchical deployments with more than three-levels, the Steps 5 – 7 and 9-10 can be executed recursively to enable the data collection from all levels of NWDAFs that are required for the analytics output generation.

- When the analytics output is available, the Top Level NWDAF notifies the analytics output to the NWDAF Service Consumer.

Editor's note:	Utilization of this procedure for data collection from a lower level NWDAF to an upper level NWDAF is FFS.

# * * * * End of changes * * * *