3GPP TSG-SA WG2 Meeting #143E (e-meeting)	S2-2100487r1

# February 24 – March 09, 2021, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2587 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Packet Loss Rate measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Huawei, Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2017-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to conclusion of TR 23.700-93, PMF needs to be enhanced to support PLR measurements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description on PLR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support PLR measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.5.2a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 5.32.5.2a	Packet Loss Rate Measurements

The UE and the UPF may decide to estimate the Packet Loss Rate (PLR) for an SDF over both accesses. For example, the UE may take this decision when an ATSSS rule in the UE requires the traffic of an SDF to be steered in accordance with a PLR-based threshold condition (e.g. PLR < 2%).

The UE and the UPF calculate the PLR for an SDF by exchanging PMF-PLR Report messages, as specified below. A PMF-PLR Report message is sent over 3GPP access or over non-3GPP access, using either the default QoS flow or a "target" QoS flow, as specified in clause 5.32.5.1.

The calculation of the PLR by the UE and by the UPF is based on the following mechanism. It is assumed that the PLR should be calculated for a target QoS flow, however, the same mechanism applies when the PLR should be calculated for the default QoS flow.

-	The UE sends a PMF-PLR Report Request message to UPF over the target QoS flow. This message contains a sequence number SN and the number of UL PDUs that were transmitted by the UE over the target QoS flow since last the time the UE sent a PMF-PLR Report Request to UPF over the target QoS flow.

-	The UPF sends a PMF-PLR Report Response message to UE over the target QoS flow. This message contains the same sequence number SN and the number of UL PDUs that were successfully received by the UPF over the target QoS flow since the last time the UPF received a PMF-PLR Report Request from the UE over the target QoS flow.

-	A sequence number SN is used to associate a PMF-PLR Report Request message and the corresponding PMF-PLR Report Response message.

-	The UE calculates the UL PLR by considering the number of UL PDUs contained in the PMF-PLR Report Request and the number of UL PDUs contained in the corresponding PMF-PLR Report Response.

-	The UPF applies the same procedure for calculating the DL PLR.

-	When the UP connection of the MA PDU session is deactivated on an access, no PMF-PLR Report messages are sent on this access. The PMF in the UPF shall not send PMF-PLR Report messages on this access, if the UP connection is not available or after it receives notification from the (H-)SMF to stop sending the PMF-PLR Report message on this access.

-	The UE and the UPF derive an estimation of the average PLR per QoS Flow over an access type by averaging the PLR measurements obtained over this access.

* * * * End of Changes * * * *