3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100503r01

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2593 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling remote provisioning of UE using Control Plane for PNI-NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE onboarding and remote provisioning is a new feature introduced in R17 eNPN topic, and last meeting most sub-parts of the feature have achieved an agreement. In case of PNI-NPN onboarding scenarios, using control plane to realize the UE remote provisioning has been concluded as an alternative solution. It is necesssay to draft a general introduction to the alternative solution in TS 23.501 and corresponding procedure in TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a general introduction to remote provisioning of UE for PNI-NPN in TS 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No introduction to remote provisioning of UE for PNI-NPN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.X, 4.2.X(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change, ALL TEXT ARE NEW * * * *

### 5.30.3.X	UE Onboarding for PNI-NPN

5.30.3.X.1	General

UE Onboarding enables a UE to get 3GPP connectivity to a Provisioning Server via PLMN, by which Subscription Owner (SO) can remote provision the NPN credentials to UE which is used for access to specific slice(s) and/or Data network offering NPN services, i.e. for Network Slice Specific Authentication and Authorization and/or secondary authentication for PDU Sessions. To provision NPN credentials, UE Register to PLMN with PLMN credentials and establishes a secure connection with that PLMN, see more details in clause 5.30.3.X.3.

After the secure connection is established, the UE is provisioned with NPN credentials and possibly other data to enable access to a PNI-NPN service, see more details in clause 5.30.3.X.4.

The architecture for UE onboarding for PNI-NPN is depicted in clause 5.30.3.X.2.

Both Control Plane based remote provisioning and User plane based remote provisioning are supported.

- In case of Control Plane based remote provisioning, UE register to PLMN and obtain the credentials from PVS via Control Plane signalling.

- In case of User Plane based remote provisioning, UE register to PLMN and obtain the credentials from PVS via User Plane connection.

Additionally, UE may obtain updated subscription and/or configuration during the remote provisioning procedure.

5.30.3.X.2	Architecture

As depicted in figure 5.30.3.X.2-1, the Provisioning Server is used for provide the NPN credential and parameters to UE via PLMN.

Figure 5.30.3.X.2-1: architecture of onboarding for PNI-NPN

AAA server may be deployed used to authenticate the UE based on the UE default credentials during the Onboarding procedure.

The PVS is an entity that interacts with the Subscription Owner, using mechanisms out of 3GPP scope, for the purpose of provisioning network credentials and other data in the UE to enable PNI-NPN access.

NOTE 3:	 The AAA server and PVS can be owned by an administrative entity that can be different from PLMN or Subscription Owner. The ownership of AAA server and PVS is outside the scope of 3GPP.

Editor’s note:	whether the AAA server in figure 5.30.3.X.2-1 is needed should be confirmed by SA3.

# * * * * End of changes * * * *