3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100505r03

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2518 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service discovery between SNPN and separate entity hosting AUSF and UDM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TR23.700-07 clause 8.1.6 concluded that “Discovery of AUSF/UDM in the separate entity can be supported by cross network service discovery and registration procedure as specified in TS 23.502 [6] clause 4.17.5 where Home PLMN is replaced by the separate entity. “ This contribution implements the related procedures in TS23.502 clause 4.17.5a. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify how SNPN discovery service provided by a separate entity using the same procedure of service discovery cross PLMNs, and add the alternative which uses cross network registration in clause 4.17.5a - Replace “PLMN” with “PLMN or SNPN”, “PLMN ID” with “PLMN ID and in addition NID in case of SNPN” in clause 5.2.7.2.2, 5.2.7.2.5, 5.2.7.3.2 to cover the SNPN case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistant with TR 23.700-07 and TS 23.501 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 4.17.5a, 5.2.7.2.2, 5.2.7.2.5, 5.2.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.700-07 ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR 23.501... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.17.5a	NF/NF service discovery between SNPN and separate entity hosting AUSF/UDM

Figure 4.17.5a-1: NF/NF service discovery cross SNPN and Separate Entity

In case of a UE accessing SNPN using credentials from a separate entity hosting AUSF/UDM, similar procedure can be used for service discovery cross network as specified in clause 4.17.5 with the difference as below:

-	In step 1, the serving PLMN ID is replaced by PLMN ID and NID which used to identify the serving SNPN. In case Separate Entity is the subscribed SNPN of UE, the home PLMN ID is replaced by the PLMN ID and NID which used to identify the subscribed SNPN.

-	In step 2, the separate entity may register the correspondent AUSF/UDM services to the NRF of SNPN using cross network NF/NF service registration as depicted in the figure 4.17.5a-2. In this case, step 2 of the procedure in Figure 4.17.5a-1 shall be skipped for service discovery in the separate entity.

-	In step 3, the serving PLMN ID is replaced by PLMN ID and NID which used to identify the serving SNPN.

NOTE:	It is to be FFS whether other parameters (e.g., realm from SUCI/SUPI) can be used to identify the separate entity in case it is a SNPN.

Figure 4.17.5a-2: NF/NF service registration cross Separate Entity and SNPN

1.	The NF Service Provider in separate entity invokes Nnrf_NFManagement_NFRegister_request to an appropriate configured NRF in the separate entity with its profile as specified in clause 4.17.1.

2.	Based on input information or configuration (e.g., this NF Service Provider is dedicated for NF Service Consumer in a SNPN), the NRF in the separate entity is expected to provide the registration information further to SNPN. It identities the NRF in SNPN based on the PLMN ID and NID, and requires "NF Registration" service from SNPN NRF to register the NF profile(s) of the NF instance(s) deployed in the separate entity. As the NRF in the separate entity triggers the “NF Registration” on behalf of the NF service provider, the NRF in the separate entity shall not replace the information of the NF Service Provider, i.e. NF provider ID, in the Registration Request message it sends to the NRF in SNPN.

3.	NRF in the separate entity responses to NF Service Provider on the registration results same as step 3 in clause 4.17.1.

# * * * * Second change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *