3GPP TSG-WG SA2 Meeting #143E e-meeting	S2-2100517r01

# Elbonia, February 24 – March 9, 2021						       			                                (was xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0221 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Extension of functional descriptions for Model training logical function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TR 23.700-91 conclusion, Rel-17 NWDAF decomposes into Analytics logical function and Model training logical function, in addition, an NWDAF instance can have either of them or both. However, Rel-16 NWDAF only supports Analytics logical function as its functionality. Thus, this CR proposes to add the descriptions for Model training logical function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of descriptions for Model training logical function in NWDAF general feature with the discovering mechanism for each logical function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The decomposition of NWDAF is not well supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of change * * * *

5.1	General

The NWDAF provides analytics to 5GC NFs, and OAM as defined in clause 7.

Analytics information are either statistical information of the past events, or predictive information.

Different NWDAF instances may be present in the 5GC, with possible specializations per type of analytics. The capabilities of a NWDAF instance are described in the NWDAF profile stored in the NRF.

In case NWDAF instances support AnLF, in order to support NFs that are consumers of analytics with the discovery of a NWDAF instance that is able to provide some specific type of analytics, each NWDAF instance should provide the Names of supported services for AnLF (i.e., Nnwdaf_AnalyticsSubscription/Info) with the list of Analytics ID(s) that it supports when registering to the NRF, in addition to other NRF registration elements of the NF profile. Other NFs requiring the discovery of an NWDAF instance that provides support for some specific type of analytics may query the NRF and include the required service names with Analytics ID(s) that identifies the desired type of analytics for that purpose.

The consumers i.e. 5GC NFs and OAM decide how to use the data analytics provided by NWDAF.

In case NWDAF instances support MTLF, in order to support NFs that are consumers of ML model provisioning (i.e., an AnLF of NWDAF) with the discovery of an NWDAF instance that is able to expose a trained ML model, each NWDAF should provide the Names of supported services for MTLF (i.e., Nnwdaf_MLModelProvision) and the list of trained ML model information (i.e., Analytics ID) that can expose when registering to the NRF, in addition to other NRF registration elements of the NF profile. Other NFs requiring the discovery of an NWDAF instance that provides exposing trained ML model(s) may query the NRF and include the required service name(s) with Analytics ID(s) that identifies the desired type of trained ML model.

Editor's note:	Whether service names can be replaced to NWDAF capability for NWDAF service consumer to respectively discover AnLF and MTLF is FFS.

NOTE:	In this release of specification, MTLF exposes a trained ML model within a single vendor environment only.

The interactions between 5GC NF(s) and the NWDAF take place within a PLMN.

The NWDAF has no knowledge about NF application logic. The NWDAF may use subscription data but only for statistical purpose.

# * * * * End of change * * * *