3GPP TSG-WG SA2 Meeting #143E e-meeting	S2-2100523r01

# Elbonia, February 24 – March 9, 2021						       			                                (was xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0222 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#19: Procedures for ML model provisioning and training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Huawei(?), HiSilicon(?), CATT(?), Lenovo(?), Motorola Mobility(?), Vivo, China Mobile(?) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is concluded in KI#1 that an NWDAF decomposes into Analytics logical function and Model training logical function where the Model training logical function provides new services. Also, the new services, i.e., MLModelProvision, MLModelInfo, and MLModelUpdate, were endorsed in KI#19 as supporting services of Model training logical function. Thus, this CR proposes to add the procedures for the new services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add the procedures for Model training logical function to expose and train an ML model(s) to the consumer NF with introduction of new services, i.e., Nnwdaf_MLModelProvision, Nnwdaf_MLModelInfo, Nnwdaf_MLModelUpdate, and Nnwdaf_MLTraining. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is uncertain how Model training logical function train and expose an ML model to the consumer NF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x, 6.x.1, 6.x.2, 6.x.3, 6.x.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a merged one with S2-2100479/596/696/859/886 with respect to the procedures of Model Training Logical Function services. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of change (all new text) * * * *

6.2x	Procedure for ML Model Provisioning and Training

6.2x.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2x.1-1 is used by NWDAF service consumer (e.g., an NWDAF containing an Analytics Logical Function) to subscribe/unsubscribe at NWDAF which supports Model Training Logical Function to be notified for an ML model information on the related Analytics, using Nnwdaf_MLModelProvision Services as defined in clause 7.y. The ML model information is used by the NWDAF containing an Analytics Logical Function to derive analytics. The service is also used by an NWDAF service consumer to modify existing ML Model Subscription(s). An NWDAF (e.g., containing both Analytics Logical Function and Model Training Logical Function) can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2x.1-1: ML Model for analytics subscribe/unsubscribe

1.	The NWDAF service consumer (e.g., other NWDAF which supports Analytics Logical Function) subscribes to or cancels subscription for an (set of) ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2x.6.

When a subscription to an ML model information for the Analytics is received, the NWDAF determines a specific ML model for each Analytics indicated in the subscription, and whether an existing trained ML Model can be used for the subscription or triggering further training for ML models is needed. In case the NWDAF determines further training is needed, the NWDAF may initiate data collection from NFs, DCCF or OAM as described in clause 6.2, 6.2x for further training itself, or retrain the ML models by other NWDAF supporting Model Training Logical Function as described in clause 6.2x.3 and 6.2x.4.

If the service invocation is for a subscription modification, the service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF service consumer subscribes to ML model information for an Analytics, the NWDAF notifies the NWDAF service consumer with the ML model information by invoking Nnwdaf_MLModelProvision_Notify service operation, based on the request from the NWDAF service consumer. The detail of ML model information that can be provided by the NWDAF are listed in clause 6.2x.a.

The NWDAF also invokes the Nnwdaf_MLModelProvision_Notify service operation to provide a re-trained ML model when the NWDAF determines that the previously provided trained ML Model required re-training at step 1.

When a notification of an ML model is received, the NWDAF service consumer may determine whether triggering further training for the ML model is needed. In case the NWDAF service consumer determines further training is needed, the NWDAF service consumer trigger the further training of ML model as described in clause 6.2x.3 and 6.2x.4.

NOTE:	This procedure is only used within a single vendor environment in this release of the specification.

6.2x.2	ML Model Request

The procedure in Figure 6.2x.2-1 is used by NWDAF service consumer (e.g., an NWDAF containing an Analytics Logical Function) to request and get an ML model on the related Analytics from NWDAF which supports Model Training Logical Function, using Nnwdaf_MLModelInfo Services as defined in clause 7.y+1. The ML model information is used by the NWDAF containing an Analytics Logical Function to derive Analytics.

Figure 6.2x.2-1: ML Model for analytics request

1.	The NWDAF service consumer (e.g., other NWDAF which supports Analytics Logical Function) requests an (set of) ML Model(s) information associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelInfo_Request service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2x.6.

When a request for an ML model information for the Analytics is received, the NWDAF determines a specific ML model for each Analytics indicated in the request, and whether an existing trained ML Model can be used for the request or triggering further training for ML models is needed. In case the NWDAF determines further training is needed, the NWDAF may initiate data collection from NFs, DCCF or OAM as described in clause 6.2, 6.2x for further training itself, or retrain the ML models by other NWDAF as described in clause 6.2x.3 and 6.2x.4.

2.	The NWDAF responses the NWDAF service consumer with the ML model information by invoking Nnwdaf_MLModelInfo_Response service operation, based on information in the request from the NWDAF service consumer. The detail of ML model information that can be provided by the NWDAF are listed in clause 6.2x.a.

When a response for an ML model is received, the NWDAF service consumer may determine whether triggering further training for the ML model is needed. In case the NWDAF service consumer determines further training is needed, the NWDAF service consumer triggers the further training of ML model as described in clause 6.2x.3 and 6.2x.4.

NOTE:	This procedure is only used within a single vendor environment in this release of the specification.

6.2x.3	ML Model Update Subscribe/Unsubscribe

The procedure in Figure 6.2x.3-1 is used by NWDAF service consumer (e.g., an NWDAF containing an Analytics Logical Function, an Model Training Logical Function, or both) to subscribe/unsubscribe at NWDAF which supports Model Training logical function to be notified for the further trained ML model, using Nnwdaf_MLModelUpdate Services as defined in clause 7.y+2. The service is also used by an NWDAF service consumer to modify existing ML Model Update subscription(s).

Figure 6.2x.3-1: ML Model Update subscribe/unsubscribe

1.	The NWDAF service consumer (e.g., other NWDAF) subscribes to or cancels subscription for training an (set of) ML Model(s) by invoking the Nnwdaf_MLModelUpdate_Subscribe / Nnwdaf_MLModelUpdate_Unsubscribe service operation.

When a subscription to training an ML model is received, the NWDAF determines whether triggering new data collection is needed.

If the service invocation is for a subscription modification, the service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelUpdate_Subscribe.

2.	If the NWDAF service consumer subscribes to training an ML model, the NWDAF notifies the NWDAF service consumer with the further trained ML model by invoking Nnwdaf_MLModelUpdate_Notify service operation, based on the parameter in the subscription from the NWDAF service consumer.

Editor's Note:	The parameters that can be provided by the NWDAF service consumer are FFS.

NOTE:	This procedure is only used within a single vendor environment in this release of the specification.

6.2x.4	ML Model Training Request

The procedure in Figure 6.2x.4-1 is used by NWDAF service consumer (e.g., an NWDAF containing an Analytics Logical Function, an Model Training Logical Function, or both) to request and get a further trained ML model from NWDAF which supports Model Training Logical Function, using Nnwdaf_MLModelTraining Services as defined in clause 7.y+3.

Figure 6.2x.4-1: ML Model Training request

1.	The NWDAF service consumer (e.g., other NWDAF) requests training an (set of) ML Model(s) by invoking the Nnwdaf_MLModelTraining_Request service operation.

When a request for training an ML model is received, the NWDAF determines whether triggering new data collection is needed.

2.	The NWDAF responses the NWDAF service consumer with the trained ML model by invoking Nnwdaf_MLModelTraining_Response service operation, based on the parameters in the request from the NWDAF service consumer.

Editor's Note:	The parameters that can be provided by the NWDAF service consumer are FFS.

NOTE:	This procedure is only used within a single vendor environment in this release of the specification.

6.2x.5	Update of Provided ML Model

This procedure is used by an NWDAF supporting Model Training Logical Function to provide updated Trained ML model to one or more NWDAF service consumer(s) (e.g., an NWDAF containing an Analytics Logical Function) using an Nnwdaf_MLModelInfo_Update service.

Figure 6.2x.5-1: Trained ML Model Update

1.	The NWDAF supporting Model Training Logical Function may determine that a trained ML Model requires retraining. This may happen when the NWDAF is configured with an updated initial ML model or when the data that were used to train the trained ML model have changed.

2.	The NWDAF supporting Model Training Logical Function initiates data collection from Data Producer NFs, DCCF or OAM as described in clause 6.2, 6.2x, or retrain the ML models by other NWDAF as described in clause 6.2x.3 and 6.2x.4.

3.	The NWDAF provides updated trained ML model information to the NWDAF service consumer by invoking the Nnwdaf_MLModelInfo_Update service operation.

Editor's Note:	The parameters that can be provided by the NWDAF service consumer are FFS.

# * * * * End of changes * * * *