3GPP TSG-WG SA2 Meeting #143E e-meeting	S2-2100523r04

# Elbonia, February 24 – March 9, 2021						       			                                (was xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0222 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#19: Procedures for ML model provisioning and training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Huawei(?), HiSilicon(?), CATT(?), Lenovo(?), Motorola Mobility(?), Vivo, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is concluded in KI#1 that an NWDAF decomposes into Analytics logical function and Model training logical function where the Model training logical function provides new services. Also, the new services, i.e., MLModelProvision, MLModelInfo, and MLModelUpdate, were endorsed in KI#19 as supporting services of Model training logical function. Thus, this CR proposes to add the procedures for the new services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add the procedures for Model training logical function to expose and train an ML model(s) to the consumer NF with introduction of new services, i.e., Nnwdaf_MLModelProvision, and Nnwdaf_MLModelInfo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is uncertain how Model training logical function train and expose an ML model to the consumer NF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2x, 6.2x.0, 6.2x.1, 6.2x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a merged one with S2-2100479/596/696/859/886 with respect to the procedures of Model Training Logical Function services. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of change (all new text) * * * *

6.2x	Procedures for ML Model Provisioning and Request

6.2x.0	General

This clause presents the procedures for the ML Model provisioning and request.

NOTE: These procedures are only used within a single vendor environment in this release of the specification.

Editor´s Note: Whether/how support for hierarchy of MTLF is provided is FFS.

Editor´s Note: Whether/how ML model consumer NWDAF (i.e, AnF) can negotiate the ML model update is FFS.

6.2x.1		ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2x.1-1 is used by an NWDAF (e.g., an NWDAF (AnLF)) to subscribe/unsubscribe at another NWDAF (e.g., an NWDAF (MTLF)), to be notified for an ML model information on the related Analytics, using Nnwdaf_MLModelProvision Services as defined in clause 7.y. The ML model information is used by the NWDAF (AnLF) to derive analytics. The service is also used by an NWDAF to modify existing ML Model Subscription(s). An NWDAF (e.g., an NWDAF (MTLF+AnLF)) can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2x.1-1: ML Model for analytics subscribe/unsubscribe

NOTE 1: In Figure 6.2x.1-1, "NWDAF 1" identifies an NWDAF (AnLF). "NWDAF 2" identifies an NWDAF (MTLF).

1.	The NWDAF 1 (e.g., an NWDAF (AnLF)) subscribes to or cancels subscription for an (set of) ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the NWDAF 1 are listed in clause 6.2x.a.

When a subscription to an ML model information for the Analytics is received, the NWDAF 2 determines a specific ML model for each Analytics indicated in the subscription, and whether an existing trained ML Model can be used for the subscription or triggering further training for ML models is needed. In case the NWDAF 2 determines further training is needed, the NWDAF 2 may initiate data collection from NFs, DCCF or OAM as described in clause 6.2 for further training itself.

If the service invocation is for a subscription modification, the NWDAF 1 includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF 1 subscribes to ML model information for an Analytics, the NWDAF 2 notifies the NWDAF 1 with the ML model information by invoking Nnwdaf_MLModelProvision_Notify service operation, based on the request from the NWDAF 1. The detail of ML model information that can be provided by the NWDAF 2 are listed in clause 6.2x.a.

The NWDAF 2 also invokes the Nnwdaf_MLModelProvision_Notify service operation to provide a re-trained ML model when the NWDAF 2 determines that the previously provided trained ML Model required re-training at step 1.

6.2x.2	ML Model Request

The procedure in Figure 6.2x.2-1 is used by an NWDAF (e.g., an NWDAF (AnLF)) to request and get an ML model on the related Analytics from another NWDAF (e.g., an NWDAF (MTLF)), using Nnwdaf_MLModelInfo Services as defined in clause 7.y+1. The ML model information is used by the NWDAF (AnLF) to derive Analytics.

Figure 6.2x.2-1: ML Model for analytics request

NOTE 1: In Figure 6.2x.2-1, "NWDAF 1" identifies an NWDAF (AnLF). "NWDAF 2" identifies an NWDAF (MTLF).

1.	The NWDAF 1 (e.g., an NWDAF (AnLF)) requests an (set of) ML Model(s) information associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelInfo_Request service operation. The parameters that can be provided by the NWDAF 1 are listed in clause 6.2x.a.

When a request for an ML model information for the Analytics is received, the NWDAF 2 determines a specific ML model for each Analytics indicated in the request, and whether an existing trained ML Model can be used for the request or triggering further training for ML models is needed. In case the NWDAF 2 determines further training is needed, the NWDAF 2 may initiate data collection from NFs, DCCF or OAM as described in clause 6.2 for further training itself.

2.	The NWDAF 2 responses the NWDAF 1 with the ML model information by invoking Nnwdaf_MLModelInfo_Response service operation, based on information in the request from the NWDAF 1. The detail of ML model information that can be provided by the NWDAF 2 are listed in clause 6.2x.a.

# * * * * End of changes * * * *