3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100569

Elbonia, Feb 24 – Mar 09, 2021	(revision of S2-2008410)

Source:	Huawei, HiSilicon

Title:	KI#3A, Conclusion update for correcting BAT

Document for:	Approval

Agenda Item:	8.5.1

Work Item / Release:	FS_IIoT / Rel-17

Abstract: Update of conclusion for correcting BAT.

# 1. Introduction/Discussion

It is concluded that BAT may be corrected based on monitoring at the  UPF..

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-20 V1.3.0.

# * * * * First change * * * *

## 8.5	Key Issue #3A: Exposure of deterministic QoS

The following is taken as the basis for the way forward:

Take solution#5 as basis for KI#3A:

-	The AF provides traffic related description and QoS requirement:

-	UE related Identification used to determine target UE PDU Session, AF Identification, a Traffic Description, a 5GS delay, Bandwidth, which are used to identify the target traffic and related QoS requirement.

-	Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Size, Burst Periodicity, and  a Timing Domain, which are used for efficient scheduling in RAN for Ethernet PDU sessions.

-	Burst Arrival Time on UPF is provided to the UPF. The UPF sends a notification to the AF/NEF via SMF/PCF for TSCAI adjustment when bursts arrive later or earlier than indicated by thresholds. The notifications contain a correction to the Burst Arrival Time

-	For ETH PDU Sessions, in order to reuse hold and forward functionality in the DS-TT and NW-TT, Qbv parameters can be derived by NEF/PCF based on AF request (with no impact to nodes other than NEF/PCF) and provided to NW-TT/DS-TT. It is assumed that Rel-16 hold and forward functionality in DS-TT and NW-TT is re-used.

Editor's note:	Whether a requirement exists that hold and forward functionality is needed for VIAPA services needs to be confirmed by SA1.

Editor's note:	Need for Jitter measurement is FFS.

# * * * * End of changes * * * *