SA WG2 Meeting #S2-143E	S2-2100596r01

24 February - 09 March, 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0224 | rev | 01 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for model sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Parameters for model sharing between NWDAFs, i.e. contents of model exposure, needs to be specified based on the conclusion of TR 23.700-91. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add parameters for model sharing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Model sharing between NWDAFs is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X.Y (new) |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change (all new text)********************

6.X.Y	Contents of Model Exposure

The consumers of the Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request service operations described in clause 7 provide the following input parameters listed below.

-	Information of the analytics for which the requested model is to be used, including:

-	A list of Analytics ID(s): identifies the analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in procedures.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_MLModelProvision_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the model provider NWDAF with this subscription.

-	Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) Model Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	Preferred level of accuracy of the model, i.e. level of accuracy of the analytics when the model is used.

-	(Only for Nnwdaf_MLModelInfo_Request) Time when model information is needed (if applicable). If the time is reached the consumer does not need to wait for the model information any longer, yet the NWDAF may send an error response to the consumer.

The model provider NWDAF provides to the consumer of the Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) The Notification Correlation Information.

-	Model information, which includes the file address (URL or FQDN) of the requested model(s) for the Analytics ID(s).

-	(Only for Nnwdaf_MLModelProvision_Subscribe) Indication of subscription to notification of model update from the model consumer NWDAF.

-	In addition, the following information:

-	Timestamp of model generation.

-	Validity period, which defines the time period for which the model information is valid.

-	Probability assertion: confidence in prediction when the model is used.

******************** End of Change ********************