SA WG2 Meeting #S2-143E	S2-2100607r06

24 February - 9 March 2021, Electronic, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2616 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 – T5, Enable mobility between networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, OPPO, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the conclusion of KI#1 of FS_eNPN, SNPN mobility can be supported from the source network to target network. - In the case that there are common AMF and/or N14 interface between the source network and target network, mechanism defined in TS 23.502 [6] clause 4.9.1 is re-used to address UE mobility. - In the case of idle mode mobility, the UE performs initial or mobility registration as specified in clause 4.2.2.2.2 of TS 23.502 [6]. This is modify the general registration procedureb to implement this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the restriction on the handover. Adding a new subclause on the mobility between SNPNs and PLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobility between SNPNs and PLMN is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.30.2.X	UE Mobility support for SNPN

If a UE access an SNPN using credentials from a Credential Holder and the UE is using a PLMN subscription, then the UE performs initial or mobility registration as specified in TS 23.502 [3] clause 4.2.2.2.2 when the UE moves between SNPN and PLMN.

If a UE access an SNPN using credentials from a Credential Holder, then the UE performs initial or mobility registration as specified in TS 23.502 [3] clause 4.2.2.2.2 when the UE moves between SNPNs.

END OF CHANGES