SA WG2 Meeting #143E (e-meeting)	S2-2100635r01

# 24 February – 09 March 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0225 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NWDAF discovery for no AOI case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current spec, a consumer may need data analytics without indicating AOI, e.g. UE communication prediction, NF load predication etc., In such cases, the consumer will not provide any area information to the NRF when performing NWDAF discovery. Then, the NRF can only return all NWDAFs supporting the requested analytics ID. While receiving the NWDAF list from the NRF, the consumer doesn’t know which NWDAF should be chosen. Even worse, the consumer may select a NWDAF which can not provide the required data analytics, e.g. the NF to be contacted for data collection is not within the serving area of selected NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying that, if the NWDAF service consumer requests analytics not limited to a specific area, it should select a NWDAF with PLMN wide serving area. If no such NWDAF exists, the consumer selects a NWDAF(s) and subscribes data analytics using implementation dependent method. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The data analytics will be failed due to wrong NWDAF was selected by the consumer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information by using the NWDAF discovery principles defined in clause 6.3.13, TS 23.501 [2].

NOTE:	In this release, for a deployment scenario where each NWDAF is serving an area, and the consumers requests analyutics not bound to a specific service area. , it is up to the consumer's implementation to decide how to subscribe to Analytics.

* * * End of changes * * *