SA WG2 Meeting #143E (e-meeting)	S2-2100641r02

# 24 February – 09 March 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0230 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of analytics aggregation without provision of AOI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2020-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in S2-2100639, a NWDAF with aggregation capability (i.e., aggregator NWDAF) may be requested by a consumer to provide data analytics for a UE or a NF without indicating AOI, which requires the aggregator NWDAF to not use AOI to discover other instances. In order to discover other NWDAFs, it is proposed that, the aggregator NWDAF query NRF with the UE location or the service area of NFs to be contacted for data collection to obtain candidate NWDAFs. In addition, based on the discussion in S2-2100639, if an aggregator NWDAF receives an analytics request including AOI, and decides to request other NWDAFs to provide the required analytics, then the aggregator NWDAF shall divide the original AOI received from consumer into sub-AOI based on the serving area of each NWDAF discovered by the aggregator NWDAF. Otherwise, the discovered NWDAF will provide error response since it can not serve the original AOI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | add a procedure for Analytics aggregation without provision of Area of Interest; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multiple NWDAF Analytics aggregation would not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1A.3.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

#### 6.1A.3.2	Procedure for Analytics Aggregation without Provision of Area of Interest

The procedure depicted in figure 6.1A.3.2-1 is used to address cases where an NWDAF service consumer requests Analytics ID(s) without providing an Area of Interest, but requires multiple NWDAFs to collectively serve the request.

Figure 6.1A.3.2-1: Procedure for analytics aggregation without provision of Area of Interest

1.	This step is a NWDAF discovery procedure without providing any area of interest. NRF may return multiple NWDAF candidates matching the requested capabilities and supported Analytics ID(s).

Depending on the requested Analytics ID, the NWDAF service consumer, based on internal logic, may be able to determine which NWDAF should be selected for the required data analytics, e.g. for the analytics ID "Load level information". But if not, the NWDAF service consumer should select a NWDAF with large serving area from the candidicate NWDAFs, e.g., NWDAF 1 which supports analytics aggregation.

2.	NWDAF service consumer sends Analytics information or analytics subscription request to the aggregator NWDAF, i.e., NWDAF 1 in the Figure 6.1A.2-1. In the request, NWDAF service consumer provides the requested Analytics ID(s), e.g. Analytics ID 1.

Once receiving the request, the Aggregator NWDAF 1 may decide to subscribe data analytics from other NWDAF instances which can provide the requested data analytics. Based on the analytics ID, there are two cases for the Aggregator NWDAF 1 to subscribe data analytics from other NWDAF instances:

3a.	If the data analytics requires UE location information, e.g., for the analytics ID "UE Mobility","Abnormal behaviour", or "User Data Congestion", the Aggregator NWDAF 1 can determine which NWDAF can provide the required analytics for the area that UE locates. In order to discover such NWDAF, the Aggregator NWDAF 1 firstly determines the AMF serving the UE as specified in the clause 6.2.2.1, then requests UE location information from the AMF.

NOTE:	If an Aggregator NWDAF receives an Analytics request for a group of UEs, i.e. the Target of Analytics Reporting set to an Internal Group ID, it performs NWDAF discovery based on location information of all UEs in the group, and then requests all discovered NWDAFs to provide the required analytics.

Editor’s Note: It is FFS if other mechanism can be used, such as a query to UDM, that doesn’t require the location query.

3b.	If the data analytics doesn’t require to collect UE location information, e.g., for the analytics ID "Service Experience","NF load information", or "UE Communication", the Aggregator NWDAF 1 can determine the NFs to be contacted for data collection as specified in the clause 6.2.2.1, then discover other NWDAFs that can collect data from such NFs to generate the required analytics. In order to discover such NWDAFs, the Aggregator NWDAF 1 firstly retrieves NF service area from each NF.

Editor’s Note: Whether the NF service area is obtained from that NF or NRF is FFS.

4.	The Aggregator NWDAF 1 sends a NF discovery request(s) including UE location (e.g. TAI-1) or NF serving area (e.g., TAI list-1) as a filter to NRF, and obtains candidates NWDAFs that can provide the required analytics.

Editor’s Note: It is FFS how to find an NWDAF that already receives data from an NF, and thereby minimizing the data collection from the NFs.

5.	Similar as the steps 4-9 in the subclause 6.1A.3.1.

* * * End of changes * * *