SA WG2 Meeting #143E (e-meeting)	S2-2100643r01

Elbonia, 24 Feb - 09 Mar 2021

Title:	[draft] LS on collecting data for E-UTRA cells in 5GS

Reply to

Release:	Rel-17

Work Item:	eNA_Ph2

Source:	SA2

To:	SA5, SA3

Cc:	SA6

Contact Person:

Name:	 Hucheng Wang

E-mail Address:	 wanghucheng at catt dot cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2100054 (S6-210331)

1. Overall Description:

SA2 received the LS on RAT type for network monitoring from SA6 as attached and discussed the following question from SA6.

- Can 5GC provide network data analytics (e.g. QoS sustainability, congestion information) per RAT type?

Currently, the measurement information for NR cells can be collected from the OAM by the NWDAF as specified in TS 23.288, for User Data Congestion analytics, QoS Sustainability analytics.

The measurement information collected from OAM for User Data Congestion analytics is shown in the Table 6.8.2-1 in TS 23.288:

| Information | Source | Description |
| --- | --- | --- |
| Measurements | OAM | Performance Measurements that will be used by the NWDAF to determine congestion levels. Performance Measurements are related to information transfer over the user plane and/or the control plane (e.g. UE Throughput, DRB Setup Management, RRC Connection Number, PDU Session Management, and Radio Resource Utilization as defined in TS 28.552 [8]). The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7]. |

The measurement information collected from OAM for QoS Sustainability analytics is shown in the Table 6.9.2-1 in TS 23.288:

| Information | Source | Description |
| --- | --- | --- |
| RAN UE Throughput | OAM TS 28.554 [10] | Average UE bitrate in the cell (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| QoS flow Retainability | OAM TS 28.554 [10] | Number of abnormally released QoS flows during the time the QoS Flows were used per timeslot, per cell, per 5QI and per S-NSSAI. |

However, SA2 is wondering whether the above measurement information for E-UTRA cells connected to 5GC can also be collected from the OAM.

Therefore, SA2 would like to ask the following question to SA5.

Q1: Can the NWDAF collect the above measurement information also for E-UTRA cells connected to 5GC from the OAM to derive analytics outputs?

Q2: If an answer to Q1 is NO, whether SA5 considers to provide the above measurement information?

In addition, disclosing internal network information as specified in clause 9.7.2.3 "Network monitoring information notification" of TS 23.286 can raise security concerns. Therefore, SA3 needs to investigate further network information that can be made available outside the network.

2. Actions:

To SA5 WG.

ACTION: 	SA2 kindly asks SA5 to provide answer and feedback to the question above.

To SA3 WG.

ACTION: 	SA2 kindly asks SA3 to provide answer and feedback to the question above.

3. Date of Next SA2 Meetings:

3GPP TSG SA2#144E		12 – 16 April 2021		eMeeting

3GPP TSG SA2#145E		17 – 28 May 2021		eMeeting