SA WG2 Meeting #143e	S2-2100654r9

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-210)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2625 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNPN with separate entity hosting subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 23.700-07 conclusion for the following item needs to be implemented in TS 23.501: KI#1:T1-b; Feature description: Architectures for access to SNPN using credentials from a separate entity. - Separate entity with AUSF and UDM - Separate entity with PLMN subscription Here is the conclusion from TR 23.700-07 for access to SNPN using credential from separate entity hosting AUSF and UDM. ### 8.1.6 Conclusions for scenario where the SNPN offers connectivity for UE(s) with credentials owned by separate entity offering AUSF and UDM The scenario where the separate entity hosts UDM, AUSF and owns the subscription is supported. It is proposed to adopt the following conclusion principles: - Discovery of AUSF/UDM in the separate entity can be supported by cross network service discovery and registration procedure as specified in TS 23.502 [6] clause 4.17.5 where Home PLMN is replaced by the separate entity. In order to facilitate selection of the separate entity owning the subscription (i.e. AUSF, UDM in the Home SNPN), the SUPI/SUCI provided by the UE contains a Home Network Identifier. When the SUPI type is an IMSI, the Home Network Identifier should comprise of PLMN ID + NID that points to the external entity; if SUPI is a Network Specific Identifier, the domain name corresponds to the realm part and the realm should identify the external entity. - It is recommended to enhance Nudm, Nausf, Namf and Nsmf services to support access to an SNPN using credentials from a separate entity that has UDM and AUSF, with the limitation that Session Management procedures are only supported for PDU Sessions terminating in the SNPN. |
| Summary of change: |  | Implement the above conclusion as follows: - Introduce SNPN with separate entity (hosting UDM, AUSF) architecture - Update SUPI for identification of external entity. If USIM credentials are used when separate entity is PLMN, IMSI based SUPIs are used. If USIM credentials are used when separate entity is SNPN, NSI based SUPIs are used. - Update SNPN General description to refer to the architecture and state the SM restriction. - Update UE subscription section to list SNPN selection method when it needs to select SNPN but contains PLMN subscription. - Update AUSF and UDM discovery and selection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNPN Feature not implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X(new), 5.30.2.X.1 (new), 5.30.2.X.Z (new), 5.30.2.3, 6.3.4, 6.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.30.2.X	SNPN connectivity for UEs with credentials owned by Credentials Holder

##### 5.30.2.X.1	General

SNPNs may support UE access using credentials owned by a Credentials Holder separate from the SNPN. In this case the Session Management procedures (i.e. PDU Sessions) terminate in an SMF in the SNPN.

##### 5.30.2.X.Z	Credentials Holder using AUSF and UDM for primary authentication and authorization

Figure 5.30.2.X.Z-1 depicts the 5G System architecture for SNPN with Credentials Holder using AUSF and UDM for primary authentication and authorization.

NOTE: 	The architecture for SNPN and Credentials Holder is depicted as a non-roaming reference architecture as the UE is not considered to be roaming, even though some of the roaming architecture reference points are also used.

Figure 5.30.2.X.Z-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AUSF and UDM

NEXT CHANGE

#### 5.30.2.3	UE configuration and subscription aspects

An SNPN-enabled UE is configured with subscriber identifier (SUPI), credentials for each subscribed SNPN identified by the combination of PLMN ID and NID. If an SNPN-enabled UE is configured with an N3IWF, it is also configured with an identifier of the country where the configured N3IWF is located.

A subscriber of an SNPN is either:

-	identified by a SUPI containing a network-specific identifier that takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19] clause 28.7.2. The realm part of the NAI may include the NID of the SNPN; or

-	identified by a SUPI containing an IMSI.

In case of access to an SNPN using credentials owned by a Credentials Holder as specified in 5.30.2.X.Z, the SUPI shall also contain identification for the Credentials Holder (i.e., the realm in the case of Network Specific Identifier based SUPI or the MCC and MNC in the case of an IMSI based SUPI).

NOTE 1:	When Credentials Holder is an SNPN, and the MCC and MNC of the SNPN is not unique, then IMSI based SUPI is not supported as the MCC and MNC need not be unique always; instead USIM credentials are supported using Network Specific Identifier based SUPI.

An SNPN-enabled UE supports the SNPN access mode. When the UE is set to operate in SNPN access mode the UE only selects and registers with SNPNs over Uu as described in clause 5.30.2.4.

Emergency services are not supported in SNPN access mode.

NOTE 1:	Voice support with emergency services in SNPN access mode is not specified in this release.

If a UE is not set to operate in SNPN access mode, even if it is SNPN-enabled, the UE does not select and register with SNPNs. A UE not set to operate in SNPN access mode performs PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17]. For a UE capable of simultaneously connecting to an SNPN and a PLMN, the setting for operation in SNPN access mode is applied only to the Uu interface for connection to the SNPN. Annex D.4 provides more details.

NOTE 2:	Details of activation and deactivation of SNPN access mode are up to UE implementation.

NEXT CHANGE

### 6.3.4	AUSF discovery and selection

In the case of NF consumer based discovery and selection, the following applies:

-	The AMF performs AUSF selection to allocate an AUSF Instance that performs authentication between the UE and 5G CN in the HPLMN. The AMF shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on AMF. The AUSF selection function in the AMF selects an AUSF instance based on the available AUSF instances (obtained from the NRF or locally configured in the AMF).

-	The UDM shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on UDM. The UDM selects an AUSF instance based on the available AUSF instance(s) obtained from the NRF or based on locally configured information, and information stored (by the UDM) from a previously successful authentication.

AUSF selection is applicable to both 3GPP access and non-3GPP access.

The AUSF selection function in AUSF NF consumers or in SCP should consider one of the following factors when available:

1.	Home Network Identifier (e.g., MNC and MCC, realm) of SUCI/SUPI (by an NF consumer in the Serving network) along with the selected NID (provided by the NG-RAN) in the case of SNPN and Routing Indicator.

NOTE 1:	The UE provides the Routing Indicator to the AMF as part of the SUCI as defined in TS 23.003 [19] during initial registration. The AMF can provide the UE's Routing Indicator to other AMFs as described in TS 23.502 [3].

NOTE 2:	In the case of SNPN and if the UE provides IMSI as SUPI/SUCI to the AMF and if SUCI/SUPI is for an SNPN served by the AMF, the AMF uses the selected NID provided by the NG-RAN together with the selected PLMN ID (from SUCI/SUPI) as the SUCI/SUPI does not always include the NID. In the case of SNPN and the UE provides NSI as SUPI/SUCI to the AMF, the AMF uses the Home Network Identifier for selection of AUSF.

When the UE's Routing Indicator is set to its default value as defined in TS 23.003 [19], the AUSF NF consumer can select any AUSF instance within the home network for the UE.

2.	AUSF Group ID the UE's SUPI belongs to.

NOTE 3:	The AMF can infer the AUSF Group ID the UE's SUPI belongs to, based on the results of AUSF discovery procedures with NRF. The AMF provides the AUSF Group ID the SUPI belongs to other AMFs as described in TS 23.502 [3].

3.	SUPI; e.g. the AMF selects an AUSF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for AUSF discovery.

In the case of delegated discovery and selection in SCP, the AUSF NF consumer shall send all available factors to the SCP.

NEXT CHANGE

### 6.3.8	UDM discovery and selection

The NF consumer or the SCP performs UDM discovery to discover a UDM instance that manages the user subscriptions.

If the NF consumer performs discovery and selection, the NF consumers shall utilize the NRF to discover the UDM instance(s) unless UDM information is available by other means, e.g. locally configured on NF consumers. The UDM selection function in NF consumers selects a UDM instance based on the available UDM instances (obtained from the NRF or locally configured).

The UDM selection functionality is applicable to both 3GPP access and non-3GPP access.

The UDM selection functionality in NF consumer or in SCP should consider one of the following factors:

1.	Home Network Identifier (e.g. MNC and MCC, realm) of SUCI/SUPI, along with the selected NID (provided by the NG-RAN) in the case of SNPN, and UE's Routing Indicator.

NOTE 1:	The UE provides the Routing Indicator to the AMF as part of the SUCI as defined in TS 23.003 [19] during initial registration. The AMF provides the UE's Routing Indicator to other NF consumers (of UDM) as described in TS 23.502 [3].

NOTE 2:	In the case of SNPN and the UE provides IMSI as SUPI/SUCI to the AMF and if SUCI/SUPI is for an SNPN served by the AMF, the AMF uses the selected NID provided by the NG-RAN together with the selected PLMN ID (from SUCI/SUPI) as the SUCI/SUPI does not always include the NID. In the case of SNPN and the UE provides NSI as SUPI/SUCI to the AMF, the AMF uses the Home Network Identifier for selection of UDM.

When the UE's Routing Indicator is set to its default value as defined in TS 23.003 [19], the UDM NF consumer can select any UDM instance within the home network of the SUCI/SUPI.

2.	UDM Group ID of the UE's SUPI.

NOTE 3:	The AMF can infer the UDM Group ID the UE's SUPI belongs to, based on the results of UDM discovery procedures with NRF. The AMF provides the UDM Group ID the SUPI belongs to other UDM NF consumers as described in TS 23.502 [3].

3.	SUPI or Internal Group ID; the UDM NF consumer selects a UDM instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI or Internal Group ID as input for UDM discovery.

4.	GPSI or External Group ID; UDM NF consumers which manage network signalling not based on SUPI/SUCI (e.g. the NEF) select a UDM instance based on the GPSI or External Group ID range the UE's GPSI or External Group ID belongs to or based on the results of a discovery procedure with NRF using the UE's GPSI or External Group ID as input for UDM discovery.

In the case of delegated discovery and selection in SCP, NF consumer shall include one of these factors in the request towards SCP.

END CHANGE