3GPP TSG-SA2 Meeting #143E	S2-2100660r07

# Feb 24th - March 9th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2629 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3B-1: Exposure of Time synchronization as a service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell, NTT DOCOMO, Samsung, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This contribution addresses the following item in the work plan: - KI#3B-1: Impact to exposure procedure to introduce Time Sync activation/deactivation SA2 FS_IIoT study has concluded the support for exposure of Time synchronization service and it need to be captured in TS 23.501. The principles to be included are from TR 23.700-20 clause 8.3 “Key Issue #3B: Exposure of Time Synchronization”: 1) AF can learn 5GS capabilities to support time synchronization, request time synchronization with specified requirements, and supply information that can be used to optimize and configure time synchronization procedure for connected devices. 2) Time Synchronization service, policy and charging control can be provided for Ethernet PDU sessions and IP PDU sessions. This service is applicable for deployments without TSN integration, for these deployments, the NEF may be used instead of TSN AF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the overall general description of time synchronization activation/deactivation support in 23.501 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed conclusion not specified as part of the work item. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20, 5.27.z(new), 6.2.5.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

5.20	External Exposure of Network Capability

The Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability and Analytics reporting capability. The Monitoring capability is for monitoring of specific event for UE in 5G System and making such monitoring events information available for external exposure via the NEF. The Provisioning capability is for allowing external party to provision of information which can be used for the UE in 5G System. The Policy/Charging capability is for handling QoS and charging policy for the UE based on the request from external party. The Analytics reporting capability is for allowing an external party to fetch or subscribe/unsubscribe to analytics information generated by 5G System.

Monitoring capability is comprised of means that allow the identification of the 5G network function suitable for configuring the specific monitoring events, detect the monitoring event, and report the monitoring event to the authorised external party. Monitoring capability can be used for exposing UE's mobility management context such as UE location, reachability, roaming status, and loss of connectivity. AMF stores URRP-AMF information in the MM context to determine the NFs that are authorised to receive direct notifications from the AMF. UDM stores URRP-AMF information locally to determine authorised monitoring requests when forwarding indirect notifications. In order to provide time synchronization services, the NEF may monitor events at the NW-TT regarding IEEE Std 1588 [x] or IEEE Std 802.1AS [104] configuration operation as specified in clause 5.27.1.3.

Provisioning capability allows an external party to provision the Expected UE Behaviour or the 5G-VN group information or AF influence on time synchronization information or service specific information to 5G NF via the NEF. The provisioning comprises of the authorisation of the provisioning external third party, receiving the provisioned external information via the NEF, storing the information, and distributing that information among those NFs that use it. The externally provisioned data can be consumed by different NFs, depending on the data. In the case of provisioning the Expected UE Behaviour, the externally provisioned information which is defined as the Expected UE Behaviour parameters in TS 23.502 [3] clause 4.15.6.3 or Network Control parameter TS 23.502 [3] clause 4.15.6.3a consists of information on expected UE movement, Expected UE Behaviour parameters or expected Network Configuration parameter. The provisioned Expected UE Behaviour parameters may be used for the setting of mobility management or session management parameters of the UE. In the case of provisioning the 5G-VN group information the externally provisioned information is defined as the 5G-VN group parameters in TS 23.502 [3] clause 4.15.6.7, and it consists of some information on the 5G-VN group. In order to provision the time synchronization parameters, the procedure to provision external information is defined in TS 23.502 clause 4.15.6.9, and it consist of information regarding the time synchronization service requirements and operation within the 5GS. The affected NFs are informed via the subscriber data update as specified in TS 23.502 [3] clause 4.15.6.2. The externally provisioned information which is defined as the Service Parameters in clause 4.15.6.7 of TS 23.502 [3] consists of service specific information used for supporting the specific service in 5G system. The provisioned Service Parameters may be delivered to the UEs. The affected NFs are informed of the data update.

Policy/Charging capability is comprised of means that allow the request for session and charging policy, enforce QoS policy, and apply accounting functionality. It can be used for specific QoS/priority handling for the session of the UE, and for setting applicable charging party or charging rate.

Analytics reporting capability is comprised of means that allow discovery of type of analytics that can be consumed by external party, the request for consumption of analytics information generated by NWDAF.

An NEF may support CAPIF functions for external exposure as specified in clause 6.2.5.1.

An NEF may support exposure of NWDAF analytics as specified in TS 23.288 [86].

## ---Start of the 4th Change---

5.27.1.z	Exposure of Time Synchronization

5G System supports time synchronization service that can be activated and deactivated by AF. Exposure of time synchronization comprises the following capabilities:

- The AF may learn 5GS capabilities to support time synchronization.

- The AF may request time synchronization service with specific requirements and supply information that can be used to configure time synchronization distribution for targeted UE(s). The AF controls activation and deactivation of the time synchronization service for the target UE(s).

The NEF exposes the 5GS capabilities to support time synchronization service to the AF. The exposed information may include the supported time synchronization distribution methods (i.e., supported IEEE Std 1588 [x] or IEEE Std 802.1AS operation modes or Access Stratum-based 5G clock sync).

The AF requests sent to the NEF may target a UE or multiple UEs. The request may include the following information:

- Target UE(s) Identification. This may correspond to:

  - Individual UEs identified using GPSI, or an IP address/Prefix or a MAC address. Any UE accessing the combination of DNN, S-NSSAI and DNAI(s).

- Requested time synchronization distribution method:

  - IEEE Std 1588 [x] or IEEE Std 802.1AS operation (i.e. as a Boundary Clock, peer-to-peer Transparent Clock, or end-to-end Transparent Clock or as a PTP relay instance).

  - Access Stratum-based 5G clock sync. In this case, the time source is provided by the 5GS. 5G-AN provides a 5GS reference time to the UE via 3GPP radio access; UE/DS-TT may provide 5G reference timing information to end stations using implementation specific means. This method is applicable for both IP and ETH PDU Sessions.

Editor’s note: 	Whether there is any impact on SA2 by this “Access Stratum-based 5G clock sync” is FFS.

- For IEEE Std 1588 [x] or IEEE Std 802.1AS [104] operation, information to be used by NEF to configure IEEE Std 1588 or IEEE Std 802.1AS functionality in DS-TT or NW-TT, if applicable.

- Spatial Validity Condition on the UE(s) location as described in clause 5.6.7.1.

Editor’s note: 	The clock accuracy attribute for time synchronization configuration, NEF exposure of 5G time domain accuracy, AF input regarding requested accuracy and use of time sync accuracy within 5G System is FFS.

The NEF uses the Time Synchronization parameters received from AF. When IEEE Std 1588 [x] or IEEE Std 802.1AS [104] operation have been selected, the NEF determines the necessary (g)PTP parameters to activate and control the service in DS-TT(s) and NW-TTs. For this purpose, the NEF uses the PMIC or BMIC to manage the IEEE Std 1588 [x] or IEEE Std 802.1AS [104] operation in the DS-TT or NW-TT, respectively (see clause 5.27.1.X).

Editor’s note: 	FFS the time synchronization information provided by the AF that may be stored in the UDR and how the NEF may update or revoke information at the UDR for existing or future PDU Sessions.

For handling (g)PTP traffic, the PCF, according to PCC rule authorization, chooses a 5QI and dynamically set the PDB and/or MDBV according to requirements for (g)PTP protocol. The PCF provides the SMF with a PCC rule generated based on the AF request. The SMF may take the information in the PCC rule to may decide to modify or establish a PDU Session to create or modify or release a QoS flow for transmitting the (g)PTP messages. The PCF acknowledges the time synchronization service request to the NEF and the NEF replies to the AF with the result.

---Start of the 6th Change---

6.2.5	NEF

6.2.5.0	NEF functionality

The Network Exposure Function (NEF) supports the following independent functionality:

-	Exposure of capabilities and events:

NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing as described in clause 5.13.

NEF stores/retrieves information as structured data using a standardized interface (Nudr) to the Unified Data Repository (UDR).

-	Secure provision of information from external application to 3GPP network:

It provides a means for the Application Functions to securely provide information to 3GPP network, e.g. Expected UE Behaviour, 5G-VN group information, time synchronization service information, and service specific information. In that case the NEF may authenticate and authorize and assist in throttling the Application Functions.

-	Translation of internal-external information:

It translates between information exchanged with the AF and information exchanged with the internal network function. For example, it translates between an AF-Service-Identifier and internal 5G Core information such as DNN, S-NSSAI, as described in clause 5.6.7.

In particular, NEF handles masking of network and user sensitive information to external AF's according to the network policy.

-	The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). NEF stores the received information as structured data using a standardized interface to a Unified Data Repository (UDR). The stored information can be accessed and "re-exposed" by the NEF to other network functions and Application Functions, and used for other purposes such as analytics.

-	A NEF may also support a PFD Function: The PFD Function in the NEF may store and retrieve PFD(s) in the UDR and shall provide PFD(s) to the SMF on the request of SMF (pull mode) or on the request of PFD management from NEF (push mode), as described in TS 23.503 [45].

-	A NEF may also support a 5G-VN Group Management Function: The 5G-VN Group Management Function in the NEF may store the 5G-VN group information in the UDR via UDM as described in TS 23.502 [3].

-	Exposure of analytics:

NWDAF analytics may be securely exposed by NEF for external party, as specified in TS 23.288 [86].

-	Retrieval of data from external party by NWDAF:

Data provided by the external party may be collected by NWDAF via NEF for analytics generation purpose. NEF handles and forwards requests and notifications between NWDAF and AF, as specified in TS 23.288 [86].

-	Support of Non-IP Data Delivery:

NEF provides a means for management of NIDD configuration and delivery of MO/MT unstructured data by exposing the NIDD APIs as described in TS 23.502 [3] on the N33/Nnef reference point. See clause 5.31.5.

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

The services provided by the NEF are specified in clause 7.2.8.

The IP address(es)/port(s) of the NEF may be locally configured in the AF, or the AF may discover the FQDN or IP address(es)/port(s) of the NEF by performing a DNS query using the External Identifier of an individual UE or using the External Group Identifier of a group of UEs, or, if the AF is trusted by the operator, the AF may utilize the NRF to discover the FQDN or IP address(es)/port(s) of the NEF as described in clause 6.3.14.

For external exposure of services related to specific UE(s), the NEF resides in the HPLMN. Depending on operator agreements, the NEF in the HPLMN may have interface(s) with NF(s) in the VPLMN.

When a UE is capable of switching between EPC and 5GC, an SCEF+NEF is used for service exposure. See clause 5.17.5 for a description of the SCEF+NEF.

## ---End of Changes---