SA WG2 Meeting #143E	S2-2000xxx

24 February - 9 March 2021, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #3A: Conclusion update

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution proposes to resolve ENs in KI#3A.

# 1		Discussion

Pending ENs should be resolved. Based on outcome of email discussion, it seems there is no consensus to proceed with Burst Spread parameter addition, Jitter measurement:

https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_143e_Electronic/INBOX/CCs/Moderated_Email_Discussion/FS_IIoT/SA2%23143E_Email_Discussion_WID_BriefIssueDesciption_FS_IIoT_r11_summary-v1.docx

Proposal 1: Remove the corresponding ENs.

It was also agreed during SA2 CC for IIoT that we should include TSCAI for IP PDU Session, snippet from CC notes:

- “Create CR to FS_IIoT TR conclusion update to include TSCAI for IP PDU Session.“

Proposal 2: Remove reference to ETH PDU Session and/or change it to any PDU session.

# 2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

## 8.5	Key Issue #3A: Exposure of deterministic QoS

The following is taken as the basis for the way forward:

Take solution#5 as basis for KI#3A:

-	The AF provides traffic related description and QoS requirement:

-	UE related Identification used to determine target UE PDU Session, AF Identification, a Traffic Description, a 5GS delay, Bandwidth, which are used to identify the target traffic and related QoS requirement.

-	Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Size, Burst Periodicity, and a Timing Domain, which are used for efficient scheduling in RAN for any PDU session.

-	For ETH PDU Sessions, in order to reuse hold and forward functionality in the DS-TT and NW-TT, Qbv parameters can be derived by NEF/PCF based on AF request (with no impact to nodes other than NEF/PCF) and provided to NW-TT/DS-TT. It is assumed that Rel-16 hold and forward functionality in DS-TT and NW-TT is re-used.

Editor's note:	Whether a requirement exists that hold and forward functionality is needed for VIAPA services needs to be confirmed by SA WG1.