SA WG2 Meeting #143E	S2-2000xxx

24 February - 9 March 2021, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #3A: Conclusion update

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution proposes to resolve ENs in KI#3A.

# 1		Discussion

Pending ENs should be resolved. Based on outcome of email discussion, it seems there is no consensus to proceed with Burst Spread parameter addition, Jitter measurement:

https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_143e_Electronic/INBOX/CCs/Moderated_Email_Discussion/FS_IIoT/SA2%23143E_Email_Discussion_WID_BriefIssueDesciption_FS_IIoT_r11_summary-v1.docx

Proposal 1: Remove the corresponding ENs aligned with conclusion that Burst Spread and jitter are not supported.

It was also agreed during SA2 CC for IIoT that we should include TSCAI for IP PDU Session, snippet from CC notes:

- “Create CR to FS_IIoT TR conclusion update to include TSCAI for IP PDU Session.“

# 2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

## 8.5	Key Issue #3A: Exposure of deterministic QoS

The following is taken as the basis for the way forward:

Take solution#5 as basis for KI#3A:

-	The AF provides traffic related description and QoS requirement:

-	UE related Identification used to determine target UE PDU Session, AF Identification, a Traffic Description, a 5GS delay, Bandwidth, which are used to identify the target traffic and related QoS requirement.

-	Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Size, Burst Periodicity, and a Timing Domain, which are used for efficient scheduling in RAN for Ethernet PDU session.

-	For ETH PDU Sessions, in order to reuse hold and forward functionality in the DS-TT and NW-TT, Qbv parameters can be derived by NEF/PCF based on AF request (with no impact to nodes other than NEF/PCF) and provided to NW-TT/DS-TT. It is assumed that Rel-16 hold and forward functionality in DS-TT and NW-TT is re-used.

Editor's note:	Whether a requirement exists that hold and forward functionality is needed for VIAPA services needs to be confirmed by SA WG1.

Editor's note:	Need for Jitter measurement is FFS.