3GPP TSG-WG SA2 Meeting #143E 	S2-2100729

Feb 24 – Mar 09, 2021, Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0534 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | GPSI translation based on UE IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In some use cases, it may be reasonable to expect that the Application Client (AC) within the UE can provide the GPSI (e.g. MSISDN) to the EAS as part of the application layer protocol, however, typically this may not be the case (i.e. the AC is not aware of its GPSI, neither MSISDN nor External Identifier). Yet in some other use cases, the user due to privacy concerns may not be comfortable to share his/her MSISDN with certain applications. This means, there are many scenarios where the application is unaware of the UE identity and yet to serve the user/UE, the application server (i.e. EAS) would need to query for certain information about the UE (e.g. location). Additionally, the AC within the UE may not be a trustworthy source for identifiers, thus a network provided identifier is preferred. Under such circumstances, the EAS that is not aware of UE identifier(s) can only provide UE's IP address (over the API) as the identifier of the UE. Therefore, It is required to provide exposure of network services to AFs using a static identifier when only an IP address of the UE is known to AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to return GPSI as a static UE identifiers when the static UE identifier is requested by AF using Nbsf_Management_Discovery service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GC operator is unable to provide network exposure service to AF that is unaware of UE identifier. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2.1, 6.1.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 2385 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 23.502 CR 2555 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (6.1.1.2.1) * * * *

#### 6.1.1.2	Binding an AF request targeting an UE address to the relevant PCF

##### 6.1.1.2.1	General

When multiple and separately addressable PCFs have been deployed, a network functionality is required in order to ensure that an AF needing to send policies about UE traffic identified by an UE address can reach over N5 the PCF holding the corresponding PDU Session information. This network functionality has the following characteristics:

-	It has information about the user identity, the DNN, the UE (IP or MAC) address(es), the S-NSSAI and the selected PCF address for a certain PDU Session.

-	For IP PDU Session type, it shall receive information when an IP address is allocated or released for a PDU Session.

-	For Ethernet PDU Sessions supporting binding of AF request based on MAC address, it shall receive information when a MAC address is detected as being used by the UE over the PDU Session (this detection takes place at the UPF under control of SMF and is defined in TS 23.501 [2] clause 5.8.2). In addition, it receives the DS-TT port MAC address in case of IEEE TSN integration (as described in clause 5.28.2 of TS 23.501 [2]).

-	The functionality determines the PCF address and if available the associated PCF instance ID and PCF set ID, selected by the PCF discovery and selection function described in TS 23.501 [2], according to the information provided by the AF or the NEF.

-	The functionality returns the GPSI as a static UE identifier if requested by the NEF or AF.

A private IPv4 address may be allocated to different PDU Sessions, e.g.:

-	The same UE IPv4 address is allocated to different PDU Sessions to the same DNN and different S-NSSAI;

-	The same UE IPv4 address is allocated to different PDU Sessions to the same S-NSSAI and different DNN.

In the case of private IPv4 address being used for the UE, the AF or the NEF may send DNN S-NSSAI, in addition, in Npcf_PolicyAuthorization_Create request and Nbsf_Management_Discovery request. The DNN and S-NSSAI can be used by the PCF for session binding, and they can be also used to help selecting the correct PCF.

# * * * * Second change (6.1.1.2.2) * * * *

##### 6.1.1.2.2	The Binding Support Function (BSF)

The BSF has the following characteristics:

-	For a certain PDU session, the BSF stores internally information about the user identity, the DNN, the UE (IP or MAC) address(es), the S-NSSAI, the selected PCF address and if available the associated PCF instance ID, PCF set ID and the level of binding (see clause 6.3.1.0 of TS 23.501 [2]).

NOTE 1:	Only NF instance or NF set Level of Binding indication are supported at the BSF.

-	The PCF registers, updates and removes the stored information in the BSF using the Nbsf management service operations defined in TS 23.502 [3].

-	The PCF ensures that it is updated each time an IP address is allocated or de-allocated to the PDU Session or, for Ethernet PDU Sessions supporting binding of AF request based on MAC address, each time it has been detected that a MAC address is used or no more used by the UE in the PDU Session.

-	Based on operator's policies and configuration, the PCF determines whether the same PCF shall be selected for the SM Policy associations to the same UE ID, S-NSSAI and DNN combination in the non-roaming or home-routed scenario.

NOTE 2:	This applies to usage monitoring.

-	The selected PCF (if needed) downloads the user profile from the UDR as described in TS 23.502 [3] 4.16.4 step 2. If usage monitoring is enabled for the user, and based on operator's policies, the PCF checks if the BSF has already existing PCF serving the combination of SUPI, S-NSSAI, DNN.

-	If no such PCF is found the PCF shall register itself to the BSF as described above in this clause.

-	Else if an existing PCF is found for the above combination, the PCF shall return to the SMF the available information about the existing PCF and a redirection indication.

NOTE 3:	The assumption is that for DNN, S-NSSAI combinations where usage monitoring be applied, the same BSF instance or the same BSF SET is selected for all UE PDU Sessions to the same DNN, S-NNSAI.

-	For retrieval binding information, any NF, such as NEF or AF, that needs to discover the selected PCF address(es), and if available, the associated PCF instance ID, PCF set ID and level of binding (see clause 6.3.1.0 of TS 23.501 [2]) for the tuple (UE address, DNN, S-NSSAI, SUPI, GPSI) (or for a subset of this Tuple) uses the Nbsf management service discovery service operation defined in TS 23.502 [3]. If the NF requests a static identifier of the UE, the BSF returns the a GPSI of the UE, if available, as the static UE identifier.

NOTE 4:	If the UE is associated with more than one GPSI, then the GPSI that is returned is based on local BSF configuration.

-	The NF may discover the BSF via NRF or based on local configuration. In the case of via NRF the BSF registers the NF profile in NRF. The Range(s) of UE IPv4 addresses, Range(s) of UE IPv6 prefixes supported by the BSF may be provided to NRF.

-	If the NF received a PCF set ID or a PCF instance ID with an indication of level of binding as result of the Nbsf management service discovery service operation, it should use that information as NF set level or NF instance level Binding Indication to route requests to the PCF as defined in clause 6.3.1.0 of TS 23.501 [2] and according to the following provisions:

-	For the NF set level of binding, the NF will receive a PCF set ID but no PCF instance ID. If an NF is not able to reach the received PCF address(es) and applies direct discovery, it should query the NRF for PCF instances within the PCF set and select another instance.

-	For the NF instance level of binding, the NF will receive a PCF set ID and a PCF instance ID. If an NF is not able to reach the received PCF address(es) and applies direct discovery, it should query the NRF for PCF service instances within the PCF and select another instance.

-	The NF should provide a Routing Binding Indication based on the received PCF set ID, level of binding and possible PCF instance ID in requests it sends to the PCF.

-	For an ongoing NF service session, the PCF may provide Binding indication to the NF (see clause 6.3.1.0 of TS 23.501 [2]). This Binding indication shall then be used instead of any PCF information received from the BSF.

-	If a new PCF instance is selected, the new PCF should invoke Nbsf_Management_Update service operation to update the binding information in BSF.

The BSF may be deployed standalone or may be collocated with other network functions, such as PCF, UDR, NRF, SMF.

NOTE 5:	Collocation allows combined implementation.