3GPP TSG-WG SA2 Meeting #143E e-meeting	S2-2100732r01

Elbonia, February 24 – March 09, 2020	(revision of S2-2008840)

Source:	Huawei, HiSilicon, Interdigital Inc.

Title:	KI#4, Sol#9: Update on extended link establishment for Layer 2 UE-to-UE Relay

Document for:	Approval

Agenda Item:	8.8.1

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract: This contribution proposes to update the connection establishment procedure for Layer 2 UE-to-UE Relay based on the discussion in SA2#141E meeting.

# 1. Introduction/Discussion

Sol#9 provides a solution on connection establishment via Layer-2 UE-to-UE Relay. Both the per-hop unicast link establishment and the extended unicast link establishment are needed and should be supported. In current Sol#9, the extended unicast link and per-hop unicast link are established in two separate procedures, shown in Figure 6.9.2.1-1 and Figure 6.9.2.2-1, and the per-hop unicast link is established after the extended unicast link. The message and the data related to the extended unicast link should be carried by the per-hop unicast link, so the per-hop unicast link should be established before the extended unicast link.

This paper proposes that the per-hop unicast link establishment and the extended unicast link establishment are in the same procedure, and the extended link establishment is completed after the per-hop unicast link establishment. That is, the per-hop links between the Source UE and UE-to-UE Relay, and between the Target UE and UE-to-UE Relay are setup, and then the extended link between the Source UE and Target UE is setup by sending the E2E PC5-S messages via UE-to-UE Relay.

Proposal 1: Clarify that extended link establishment (steps 7-9 of Figure 6.9.2.1-1) is completed after the per-hop unicast link establishment (steps 2-6 of Figure 6.9.2.1-1).

Sol#9 proposes to use the link mapping for data/signalling routing on UE-to-UE Relay. However, in TR 38.836 (clause 5.5.1), RAN2 has decided that the identify information of Source Remote UE and/or the identity information of Destination Remote UE can be included in adaptation layer for data and signalling routing. So based on the functionality of adaptation layer defined by RAN2, adaptation layer is not used for per-hop unicast link establishment (steps 2-6 of Figure 6.9.2.1-1).

Proposal 2: The Adaption Layer is used for extended link establishment (steps 7-9 of Figure 6.9.2.1-1), not used for per-hop unicast link establishment (steps 2-6 of Figure 6.9.2.1-1).

In the connection establishment procedure, some messages are per-hop PC5-S messages and some are E2E PC5-S messages. This paper points out which messages are per-hop PC5-S messages or are E2E PC5-S messages. It is proposes how to differentiate per-hop PC5-S messages and E2E PC5-S messages depends upon RAN solution.

Proposal 3: Identify the per-hop PC5-S messages and E2E PC5-S messages, and add a NOTE to clarify the RAN dependency.

Per-hop unicast link and the extended unicast link use the same pair of Layer-2 IDs on each hop. The management link in Figure 6.9.2.2-1 can be replaced by the per-hop unicast link. So the Link Identifier Update on Layer-2 ID aspect can be done on the two per-hop unicast links, which can directly reuse the V2X mechanism which avoids the cross-link management issue. The Link Identifier Update on Application Layer ID aspect may need some interactions between Source UE and Target UE. So the Link Identifier Update procedure can be further simplified.

Proposal 4: For the Link Identifier Update procedure, it does not need to establish an additional management link. It can be further updated based on the agreed connection establishment procedure during normative phase.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.752.

# * * * * First change * * * *

## 6.9	Solution #9: Connection establishment via UE-to-UE Layer-2 Relay

### 6.9.1	Description

#### 6.9.1.1	General

Using the solution described in this clause, a UE-to-UE Relay enables the discovery of a source UE by a target UE. A UE-to-UE Relay is authorized to relay messages between two UEs over the PC5 interface via authorization and provisioning, as defined in clause 6.36 Solution for Key Issue #4: UE-to-UE Relay Authorization and Provisioning.

The source UE announces its supported applications or discovers a target UE using a known discovery mechanism, e.g. using user-oriented or service-oriented methods as defined in TS 23.287 [5].

The UE-to-UE Relay listens for ProSe applications advertisements (e.g. Direct Discovery or Direct Communication Request messages) from surrounding UEs and if a broadcasted application matches one of the applications from its provisioned relay policy/parameters, the UE-to-UE Relay advertises it as a relayed application by adding a relay indication to the message.

A target UE discovers a source UE via a UE-to-UE Relay. The target UE receives a broadcast Direct Communication Request message with a relay indication.

A secured "extended" PC5 link is set up between the source UE and the target UE via the UE-to-UE Relay. Source/Target UEs send messages to the UE-to-UE Relay and receive messages through the UE-to-UE Relay. However, the security association and the PC5 unicast link are established directly between the source UE and target UE. The UE-to-UE Relay forwards the messages in opaque mode, without the ability to read, modify their content or replay them, with the exception of the Discovery messages and Direct Communication Request message that are sent in cleartext (i.e. without security protection), visible to the UE-to-UE Relay, and are modified to add the relay indication. The source/target UEs detect that the link establishment is going through a UE-to-UE Relay upon detecting a relay indication included in the received messages.

The UE-to-UE Relay maintains a management connection between the Relay UE and each Source/Target UE, and also maintains 1 or more “extended” end-to-end PC5 unicast links between a Source UE and Target UE(s). The UE-to-UE Relay forwards messages between the Source UEs and Target UEs using an Adaption Layer. The details of the identity information of Source UE and Target UE used to forward or route messages in the Adaption Layer will be defined in cooperation with RAN WGs during normative phase.

NOTE:	Additional security-related parameters and procedures may be needed for the protection of relay related messages. Their definitions need to be coordinated with SA WG3.

Link management (i.e. keepalive, link modification, link identifier update and link release) is supported over direct unicast links and needs to be supported over extended PC5 links as well. Since the security association of extended PC5 links is between the peer UEs, all messages sent over the extended PC5 link, including link management (i.e. PC5-S) messages, may only be processed by those two UEs.

#### 6.9.1.2	Control and User Plane Protocol Stacks

Figure 6.9.1.2-1 illustrates control plane protocol stacks using a UE-to-UE Layer-2 Relay. UE-to-UE Layer-2 Relay uses self-generated own Layer-2 IDs inserted as source Layer-2 ID in the already formed L2 frames relayed between UE1 and UE2. L2 frames are forwarded based on mapped UE1 Layer-2 ID, UE2 Layer-2 ID. The signalling messages are transparently transferred between UE1 and UE2 over UE-to-UE Layer-2 Relay without any modification other than the source and destination Layer-2 IDs. The security is established end to end between UE1 and UE2 as shown by the PDCP layer terminating in UE1 and UE2.

Figure 6.9.1.2-1: Control Plane protocol stacks for PC5 using a UE-to-UE Layer-2 Relay

Figure 6.9.1.2-2 illustrates user plane protocol stacks using a UE-to-UE Layer-2 Relay. As for the control plane, the security is established end to end between UE1 and UE2. Therefore, user data is never exposed at the relay node since the relay function does not process/apply any security on relayed IP packets.

Figure 6.9.1.2-2: User Plane protocol stacks using a UE-to-UE Layer-2 Relay

The SDAP and PDCP protocols above are as specified in TS 38.300 [11].

Editor's note:	The proposed protocol stacks are to be confirmed with RAN WG2.

### 6.9.2	Procedures

#### 6.9.2.1	Connection establishment

Connection establishment for L2 UE-to-UE relay includes two cases:

-	Connection establishment integrating UE-to-UE relay discovery and selection, and

-	Connection establishment after UE-to-UE relay discovery and selection

##### 6.9.2.1.1	Connection establishment integrating UE-to-UE relay discovery and selection

The two methods defined in TS 23.287 [5], i.e. service-oriented and user-oriented are supported using the procedure described in this clause.

Figure 6.9.2.1.1-1 shows the peer discovery and unicast link establishment over PC5 reference point via a UE-to-UE Relay.

Figure 6.9.2.1.1-1: Connection establishment procedure integrating UE-to-UE relay discovery and selection

0.	UE-to-UE Relay registers with the network and specifies its UE-to-UE Relay capabilities. UE-to-UE Relay is provisioned from the network with relay policy parameters and with a unique Relay identifier (RID).

1.	The target UEs (i.e. UE2, UE3 and UE4) determine the destination Layer-2 ID for signalling reception for PC5 unicast link establishment as specified in TS 23.287 [5] clause 5.6.1.4. The destination Layer-2 ID is configured with the target UEs as specified in TS 23.287 [5] clause 5.1.2.1.

2.	On the source UE (i.e. UE1), the application layer provides information to the ProSe layer for PC5 unicast communication (e.g. broadcast Layer-2 ID, ProSe Application ID, UE's Application Layer ID, target UE's Application Layer ID, relay applicable indication), as specified in TS 23.287 [5] clause 6.3.3.1.

ProSe layer triggers the peer UE discovery mechanism by sending a broadcast Direct Communication Request message. The message is sent using the source Layer-2 ID and broadcast Layer-2 ID as destination, and includes other parameters related to the application offered, as specified in TS 23.287 [5] clause 6.3.3.1. Optionally, ProSe Application Code obtained from 5G DDNMF via the solution for KI#1 is also included in the message. In addition, if the Relay Service Code is obtained from the PCF, then the Relay Service Code shall also be included in the message.

3.	The UE-to-UE Relay receives the broadcast Direct Communication Request message and verifies if it's configured to relay this application, i.e. it compares the announce ProSe Application ID with its provisioned relay policy/parameters. If the Relay Service Code is received in step 2, then the UE-to-UE Relay should check if the Relay Service Code provisioned by PCF matches the received Relay Service Code in step 2.

The UE-to-UE Relay forwards the broadcast Direct Communication Request message by using its Layer-2 ID, and additionally includes the Adaptation layer info identifying UE1 and Relay UE's ID in the message.

4a.	Target UE3 is interested in the announced application or it can match the ProSe Application Code contained in the request message thus, it triggers the per-hop link establishment with the UE-to-UE Relay if there is not existing per-hop link between UE3 and UE-to-UE Relay. UE3 sends a per-hop link establishment procedure message its Layer-2 ID as the source and the Layer-2 ID from the relay as the destination.

4b.	The per-hop link establishment procedure is performed between the UE-to-UE relay and UE1, if there is not an existing per-hop link between the UE-to-UE relay and UE1 its Layer-2 ID as the source and UE1's Layer-2 ID as the destination.

5.	E2E Authentication and security establishment messages are exchanged between UE1 and UE3 via the UE-to-UE Relay. including the Adaptation layer identifying the source and/or destination UE.

Editor's note:	The Details of the authentication and security procedure will be investigated by SA WG3 group.

NOTE:	Step4b is triggered by step 5.

6.	Once the end-to-end security is established between UE3 and UE1, UE3 completes the end-to-end link establishment between UE3 and UE1 by sending a Direct Communication Accept message including the Adaptation layer info identifying UE1.

7.	UE-to-UE Relay forwards the Direct Communication Accept message, including the Adaptation layer info identifying UE3.

NOTE:	The details of the Adaptation layer information identifying the Source UE and Target UEs, and how to differentiate the E2E PC5-S message from the per-hop PC5-S message will be defined in cooperation with RAN WGs during normative phase.

8.	An "extended" unicast link has been established between UE1 and UE3, via the UE-to-UE Relay. The extended link is secured end to end, i.e. a security association has been created between UE1 and UE3. Confidentiality and/or integrity/replay protected messages (i.e. data or PC5-S) may be exchanged between UE1 and UE3. The UE-to-UE Relay is not involved in the security association thus it cannot read nor modify the secured portion of the message (which excludes the source and destination fields).

In addition, the UE-to-UE Layer-2 Relay operation is also supported with the following principles:

-	UE-to-UE Relay selection.

It may be the situation where multiple UE-to-UE relays can be used to achieve the indirect communication between the target UE and source UE. The selection of the UE-to-UE Relay may be based on local configured rules on the UE, or based on other UE-to-UE Relay selection solutions, e.g. "UE-to-UE Relay Selection Without Relay Discovery" described in clause 6.8.

-	QoS handling.

During the process of the connection establishment between the Source UE1 and the Target UE3, the Source UE1 negotiates the PC5 QoS parameters with the UE-to-UE Relay UE and Target UE3 for fulfilling E2E QoS requirements. After that, PC5 QoS parameters for PC5 link between the Source UE and UE-to-UE Relay UE and PC5 link between the UE-to-UE Relay UE and Target UE are determined. AS layer configurations for PC5 QoS parameters in each PC5 link can be achieved according to legacy mechanisms in Rel-16 V2X. Refer to Solution #31 for the details.

QoS flow concept in particular can be reused between the Source UE and the Target UE, where the UE-to-UE Relay UE performs the necessary adaptation between two PC5 interfaces, i.e. PC5 for the Source UE and UE-to-UE Relay UE and PC5 for the UE-to-UE Relay UE and Target UE.

Editor's note:	The details of the adaptation between two PC5 interfaces are confirmed by RAN WG2.

-	Charging support.

The charging for Source UE and Target UE can be based on charging usage information configuration and UE reporting usage information. Solution for charging usage information configuration can reuse the PCF based solution, i.e., Solution #14. Solution for UE reporting usage information can reuse SMF based or AMF based solution, i.e., Solution #13 or Solution #15.

##### 6.9.2.1.2	Connection establishment after UE-to-UE relay discovery and selection

Figure 6.9.2.1.2-1 shows the connection establishment procedure after UE-to-UE relay discovery and selection. The UE-to-UE relay discovery can be Model A or Model B. For the Model B, this procedure corresponds to the Alternative 2 in Clause 6.8.2.2 of Solution #8, where the UE-to-UE relay discovery and selection is integrated into Model B direct discovery procedure.

Figure 6.9.2.1.2-1: Connection establishment procedure after UE-to-UE relay discovery and selection

The Steps are same as the steps described in clause 6.9.2.1.1 with the following differences:

-	For step 2, the standalone UE-to-UE relay discovery is performed, including the Model A or Model B.

-	For step 3, the procedure specified in clause 6.9.2.1.1 is performed, in which the Direct Communication Request message is sent to UE3 via the per-hop link and the Adaptation layer info identifying the peer UE as the destination. The initiator knows the Adaptation layer info identifying the peer UE after a discovery procedure.

NOTE:	How to send DCR message in step 3 to selected Relay once the Relay selection was performed during discovery procedure will be determined in the normative phase.

#### 6.9.2.2	Connection Management

##### 6.9.2.2.1	Link Identifier Update via a Management Link with the UE-to-UE Relay

Figure 6.9.2.2-1 shows the link identifier update procedure when an extended PC5 link is used. The procedure uses a management link established between UE1 and the UE-to-UE Relay serving this extended link and another management link established between UE2 and the same UE-to-UE Relay.

NOTE:	The details of the link identifier update procedure can be aligned during normative phase.

Figure 6.9.2.2-1: Link Identifier Update procedure via a Management Link with a UE-to-UE Relay

0)	An "extended" unicast link is established between two peer UEs via a UE-to-UE Relay, i.e. as described in clause 6.9.2.1 with end-to-end security enabled.

1)	UE1 receives a trigger (e.g. privacy timer expiry or Application Layer ID change) to update its identifiers (i.e. Layer-2 ID, security information, Application Layer ID or IP address/prefix) associated to the extended link with UE2. The management link between UE1 and UE-to-UE Relay has already been established.

2)	UE1 updates its identifiers (i.e. Layer-2 ID, security information and optionally Application Layer ID and IP address/prefix) and sends a Link Identifier Update Request message to the UE-to-UE Relay via the management link. The message includes UE1's new Layer-2 ID and an indication (e.g. "extended link" indication) which specifies that the message is related to an extended link, i.e. it does not apply to the management link per se. The message also includes the UE-to-UE Relay's Layer-2 ID and UE1's Layer-2 ID used to identify the extended link.

a.	Other identifiers (i.e. security info, Application Layer ID and IP address/prefix) are not included since they are not used by the UE-to-UE Relay and should not be exposed to the UE-to-UE Relay.

3)	UE-to-UE Relay saves UE1's new Layer-2 ID in its mapping table, while preserving the current one, and updates its own Layer-2 ID to replace the current UE-to-UE Relay L2 ID used on the extended link and known by UE2. It replies with Link Identifier Update Response message including its new UE-to-UE Relay Layer-2 ID and the "extended link" indication.

4)	UE1 sends a Link Identifier Update Request message to UE2 including the new UE-to-UE Relay L2 ID received at step 3, UE1's updated security information and optionally new Application Layer ID and IP address/prefix.

a.	The Link Identifier Update Request message is used as usual, except for the new L2 ID parameter that carries the new UE-to-UE Relay L2 ID to be used by UE2.

5)	UE2 keeps track of the received parameters. The management link between UE2 and the UE-to-UE Relay has already been established.

6)	As for UE1 in step 2, UE2 updates its identifiers associated to the extended link with UE1 and sends a Link Identifier Update Request message to UE-to-UE Relay via the management link. The Link Identifier Update Request message includes the "extended link" indication, the current UE-to-UE Relay's Layer-2 ID and UE2's Layer-2 ID (to identify the extended link) as well as UE2's new Layer-2 ID associated to the extended link.

7)	UE-to-UE Relay saves UE2's new Layer-2 ID in its mapping table, while preserving the current one, and updates its own Layer-2 ID to replace the current UE-to-UE Relay L2 ID used on the extended link and known by UE1. It replies with Link Identifier Update Response message including its new UE-to-UE Relay Layer-2 ID and the "extended link" indication.

8)	UE2 sends a Link Identifier Update Response message to UE1 including the new UE-to-UE Relay L2 ID received at step 7, UE2's updated security information and optionally new Application Layer ID and IP address/prefix. UE2 also includes the parameters received on the Link Identifier Update Request message at step 4.

9)	UE1 keeps track of the received updated parameters from UE2 and sends a Link Identifier Update Ack message to UE2, including the parameters received on the Link Identifier Update Response message at step 8.

10)	UE1 sends a Link Identifier Update Ack message to the UE-to-UE Relay, including the new UE-to-UE Relay Layer-2 ID received at step 3 and the "extended link" indication.

UE2 sends a Link Identifier Update Ack message to the UE-to-UE Relay, including the new UE-to-UE Relay Layer-2 ID received at step 7 and the "extended link" indication. All UEs (i.e. UE1, UE2 and UE-to-UE Relay) start using the new Layer-2 IDs, new security information and optionally new Application Layer ID and new IP address/prefix.

# * * * * End of changes * * * *