3GPP TSG-WG SA2 Meeting #143E e-meeting  	S2-2100761r02

# February 24 – March 9, 2021, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2642 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architectural changes to increasing efficiency of data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | Introduce the DCCF, ADRF and MFAF in 23.501 as agreed in the TR 23.700-91 Conclusion in Clause 8.11.4 “Signalling reduction via architectural changes” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Section – 3.2 Update Abbreviationsf - Section - 4.2.2 add DCCF, ARDF and MFAF to list of NFs - Section 4.2.6 – Introduce DCCF, ADRF and MFAF Service-Based Interfaces - Section 6.2 – summarize DCCF, ADRF and MFAF functionallity - Section 6.3 – describe DCCF, ADRF and MFAF discovery and selection - Section 7.2 – describe DCCF, ADRF and MFAF new services |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 23.501 is not updated to reflect architecture agreed in TR conclusion and Signaling Reduction enabled architectural changes are not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2.2, 4.2.3, 4.2.6, (NEW) 6.2.x, (NEW) 6.2.x+1, (NEW) 6.3.x, (NEW) 6.3.x+1, (NEW) 6.3.x+2, (NEW) 7.2.x, (NEW) 7.2.x+1, (NEW) 7.2.x+2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G-VN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AMF	Access and Mobility Management Function

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GFBR	Guaranteed Flow Bit Rate

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

N3IWF	Non-3GPP InterWorking Function

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-Specific Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PTP	Precision Time Protocol

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

# * * * * Second change * * * *

### 4.2.2	Network Functions and entities

The 5G System architecture consists of the following network functions (NF).

-	Authentication Server Function (AUSF).

-	Access and Mobility Management Function (AMF).

-	Data Network (DN), e.g. operator services, Internet access or 3rd party services.

-	Unstructured Data Storage Function (UDSF).

-	Network Exposure Function (NEF).

-	Network Repository Function (NRF).

-	Network Slice Specific Authentication and Authorization Function (NSSAAF).

-	Network Slice Selection Function (NSSF).

-	Policy Control Function (PCF).

-	Session Management Function (SMF).

-	Unified Data Management (UDM).

-	Unified Data Repository (UDR).

-	User Plane Function (UPF).

-	UE radio Capability Management Function (UCMF).

-	Application Function (AF).

-	User Equipment (UE).

-	(Radio) Access Network ((R)AN).

-	5G-Equipment Identity Register (5G-EIR).

-	Network Data Analytics Function (NWDAF).

-	CHarging Function (CHF).

-	Data Collection Coordination Function (DCCF)

-	Analytics Data Repository Function (ADRF)

-	Messaging Framework Adaptor Function (MFAF)

NOTE 1:	The functional description on architecture and principles of the CHF is specified in TS 32.240 [41].

NOTE 2:	The functionalities provided by the DCCF or ADRF can also be hosted by an NWDAF.

The 5G System architecture also comprises the following network entities:

-	Service Communication Proxy (SCP).

-	Security Edge Protection Proxy (SEPP).

The functional descriptions of these Network Functions and entities are specified in clause 6.

-	Non-3GPP InterWorking Function (N3IWF).

-	Trusted Non-3GPP Gateway Function (TNGF).

-	Wireline Access Gateway Function (W-AGF).

-	Trusted WLAN Interworking Function (TWIF).

# * * * * Third change * * * *

### 4.2.3	Non-roaming reference architecture

Figure 4.2.3-1 depicts the non-roaming reference architecture. Service-based interfaces are used within the Control Plane.

Figure 4.2.3-1: 5G System architecture

NOTE:	If an SCP is deployed it can be used for indirect communication between NFs and NF services as described in Annex E. SCP does not expose services itself.

Figure 4.2.3-2 depicts the 5G System architecture in the non-roaming case, using the reference point representation showing how various network functions interact with each other.

NOTE 1:	N9, N14 are not shown in all other figures however they may also be applicable for other scenarios.

NOTE 2:	For the sake of clarity of the point-to-point diagrams, the UDSF, NEF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDSF, UDR, NEF and NRF as necessary.

NOTE 3:	The UDM uses subscription data and authentication data and the PCF uses policy data that may be stored in UDR (refer to clause 4.2.5).

NOTE 4:	For clarity, the UDR and its connections with other NFs, e.g. PCF, are not depicted in the point-to-point and service-based architecture diagrams. For more information on data storage architectures refer to clause 4.2.5.

NOTE 5:	For clarity, the NWDAF, DCCF, MFAF and ADRF and their connections with other NFs, e.g. PCF, are not depicted in the point-to-point and service-based architecture diagrams. For more information on network data analytics architecture refer to TS 23.288 [86].

Figure 4.2.3-2: Non-Roaming 5G System Architecture in reference point representation

Figure 4.2.3-3 depicts the non-roaming architecture for UEs concurrently accessing two (e.g. local and central) data networks using multiple PDU Sessions, using the reference point representation. This figure shows the architecture for multiple PDU Sessions where two SMFs are selected for the two different PDU Sessions. However, each SMF may also have the capability to control both a local and a central UPF within a PDU Session.

Figure 4.2.3-3: Applying non-roaming 5G System architecture for multiple PDU Session in reference point representation

Figure 4.2.3-4 depicts the non-roaming architecture in the case of concurrent access to two (e.g. local and central) data networks is provided within a single PDU Session, using the reference point representation.

Figure 4.2.3-4: Applying non-roaming 5G System architecture for concurrent access to two (e.g. local and central) data networks (single PDU Session option) in reference point representation

Figure 4.2.3-5 depicts the non-roaming architecture for Network Exposure Function, using reference point representation.

Figure 4.2.3-5: Non-roaming architecture for Network Exposure Function in reference point representation

NOTE 1:	In figure 4.2.3-5, Trust domain for NEF is same as Trust domain for SCEF as defined in TS 23.682 [36].

NOTE 2:	In figure 4.2.3-5, 3GPP Interface represents southbound interfaces between NEF and 5GC Network Functions e.g. N29 interface between NEF and SMF, N30 interface between NEF and PCF, etc. All southbound interfaces from NEF are not shown for the sake of simplicity.

# * * * * Fourth Change * * * *

### 4.2.6	Service-based interfaces

The 5G System Architecture contains the following service-based interfaces:

Namf:	Service-based interface exhibited by AMF.

Nsmf:	Service-based interface exhibited by SMF.

Nnef:	Service-based interface exhibited by NEF.

Npcf:	Service-based interface exhibited by PCF.

Nudm:	Service-based interface exhibited by UDM.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

Nnssaaf:	Service-based interface exhibited by NSSAAF.

Nnssf:	Service-based interface exhibited by NSSF.

Nausf:	Service-based interface exhibited by AUSF.

Nudr:	Service-based interface exhibited by UDR.

Nudsf:	Service-based interface exhibited by UDSF.

N5g-eir:	Service-based interface exhibited by 5G-EIR.

Nnwdaf:	Service-based interface exhibited by NWDAF.

Nchf:	Service-based interface exhibited by CHF.

Nucmf:	Service-based interface exhibited by UCMF.

Ndccf:	Service based interface exhibited by DCCF.

Nmfaf:	Service based interface exhibited by MFAF.

Nadrf:	Service based interface exhibited by ADRF.

NOTE:	The Service-based interface exhibited by CHF is defined in TS 32.290 [67].

# * * * * Fifth Change (all new text) * * * *

### 6.2.x	DCCF

The Data Collection Coordination Function (DCCF) supports the following functionality:

-	Determining Data Sources that can provide data for a received data request.

-	Determining whether data is already being collected from a data source.

-	Instructing a Messaging Framework to send data to consumers or notification endpoints.

-	Instructing a Messaging Framework to do formatting and processing of the data sent via the Messaging Framework.

-	Formatting and processing of data.

-	Sending data to consumers or notification endpoints.

-	Registering NWDAFs and ADRFs that are already receiving data from a Data Source.

The DCCF functionality is specified in 3GPP TS 23.288 [86].

### 6.2.x+1	MFAF

The Messaging Framework Adaptor Function (MFAF) supports the following functionality:

-	Interfacing with a DCCF that controls how a messaging framework will process, format and send data to consumers or notification endpoints.

-	Receiving data from Data Sources via services offered by those Data Sources.

-	Sending data received from Data Sources to a messaging framework (outside the scope of 3GPP).

-	Receiving data from a messaging framework (outside the scope of 3GPP).

-	Processing, formatting and sending data to specified consumers or notification endpoints.

The MFAF functionality is specified in 3GPP TS 23.288 [86].

### 6.2.x+2	ADRF

The Analytics Data Repository Function (ADRF) supports the following functionality:

- Storage and retrieval of analytics generated by NWDAFs and collected data.

The Analytics Data Repository Function (ADRF) is specified in 3GPP TS 23.288 [86].

# * * * * Sixth Change (all new text) * * * *

### 6.3.x	DCCF discovery and selection

Multiple instances of DCCF may be deployed in a network.

The NF consumers shall utilize the NRF to discover DCCF instance(s) unless DCCF information is available by other means, e.g. locally configured on NF consumers. The DCCF selection function in NF consumers selects a DCCF instance based on the available DCCF instances.

The following factors may be considered by the NF consumer for DCCF selection:

-	S-NSSAI;

-	NF types of the Data Sources that a DCCF coordinates;

-	DCCF Serving Area information, i.e. list of TAIs for which the DCCF coordinates Data Sources.

### 6.3.x+1	ADRF discovery and selection

Multiple instances of ADRF may be deployed in a network.

The NF consumers shall utilize the NRF to discover ADRF instance(s) unless ADRF information is available by other means, e.g. locally configured on NF consumers. The ADRF selection function in NF consumers selects an ADRF instance based on the available ADRF instances.

NOTE:	When NF consumer is DCCF, the DCCF can have information available already from previous registrations of ADRFs. In this case, NRF discovery is not needed.

The following factors may be considered by the NF consumer for ADRF selection:

-	S-NSSAI;

-	NF Types of the Data Sources for which the ADRF stores data;

-	ADRF Serving Area information, i.e. list of TAIs for which the ADRF stores data.

### 6.3.x+2	MFAF discovery and selection

Multiple instances of MFAF may be deployed in a network.

The MFAF selection function is supported by the DCCF. The DCCF shall utilize the NRF to discover MFAF instance(s) unless MFAF information is available by other means, e.g. locally configured on the DCCF. The MFAF selection function in the DCCF selects a MFAF instance based on the available MFAF instances.

The following factors may be considered by the DCCF for MFAF selection:

-	S-NSSAI;

-	NF Types of the Data Sources handled by the MFAF;

-	MFAF Serving Area information, i.e. list of TAIs for which the MFAF may receive data from Data Sources.

# * * * * Seventh Change (all new text) * * * *

### 7.2.x	DCCF Services

The following NF services are specified for DCCF:

Table 7.2.x-1: NF Services provided by DCCF

| Service Name | Description | Reference in TS 23.288 [86] |
| --- | --- | --- |
| Ndccf_DataManagement | This service enables a Data Consumer to request data, and have it delivered via a messaging framework or via the DCCF, while avoiding redundant requests to data sources. | TBD |
| Ndccf_ContextManagement | This service allows a network function to register/deregister the availability of data with the DCCF. | TBD |

### 7.2.x+1	MFAF Services

The following NF services are specified for MFAF:

Table 7.2.x+1-1: NF Services provided by MFAF

| Service Name | Description | Reference in TS 23.288 [86] |
| --- | --- | --- |
| Nmfaf_3daDataManagement | This service allows a DCCF to request a messaging framework to provide collected data to consumers or notification endpoints according to processing and formatting instructions. | TBD |
| Nmfaf_3caDataManagement | This service allows a messaging framework to provide data to consumers or notification endpoints according to instructions received via the Nmfaf_3da_DataManagement service. | TBD |

### 7.2.x+2	ADRF Services

The following NF services are specified for ADRF:

Table 7.2.x+2-1: NF Services provided by ADRF

| Service Name | Description | Reference in TS 23.288 [86] |
| --- | --- | --- |
| Nadrf_DataStorage | This service allows consumers to store data in an ADRF. | TBD |
| Nadrf_DataRetrieval | This service allows consumers to query an ADRF for stored data and to retrieve data from an ADRF. | TBD |

# * * * * End of changes * * * *