SA WG2 Meeting #143e	S2-2100764r01

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-210)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0237 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for data collection using DCCF and Messaging Framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As part of KI#11 conclusions for eNA_Ph2, SA2 “Signaling reducation via architectural changes” it was agreed that the DCCF would coordinate data collection and send notifications via a messaging framework |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of procedures to obtain data using a DCCF and a messaging framework. Two cases are covered. One for data collection and one to obtain historical data |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data collection via DCCF and Messaging Framework is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (NEW) 6.2.S.x.4, (NEW) 6.2.S.x.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (all new text)

#### 6.2.S.X	Data Collection using DCCF

##### 6.2.S.x.1	General

##### 6.2.S.x.2	Data Collection via DCCF

##### 6.2.S.x.3		Historical Data Collection via DCCF

#### 6.2.S.x.4	Data Collection via Messaging Framework

This procedure depicted in figure 6.2.S.x.4-1 is used by a data consumer (e.g. NWDAF) to obtain data and be notified of events using the DCCF and a Messaging Framework. The 3GPP DCCF Adaptor (3da) Data Management service and 3GPP Consumer Adaptor (3ca) Data Management service of the Messaging Framework Adaptor Function (MFAF) are used to interact with the 3GPP Network and the Messaging Framework. Whether the data consumer directly contacts the Data Source or goes via the DCCF is based on configuration.

Figure 6.2.S.x.4-1: Data Collection via Messaging Framework

- The data consumer subscribes to data via the DCCF by invoking the Ndccf_DataManagement_Subscribe(Service_Operation, Data Specification, Formatting Instructions, Processing Instructions, NF (or NF-Set) ID, ADRF Information) service operation. The data consumer may specify one or more notification endpoints and the NF or NF set to collect data from.

Service_Operation is the service operation to be used by the DCCF to request data (e.g.: Namf_EventExposure_Subscribe or OAM Subscribe). Data Specification provides Service_Operation-specific required parameters (e.g. event IDs, UE-ID(s), target of event reporting) and optional input parameters used to retrieve the data. Formatting and Processing Instructions are as defined in clause 5A.4. The data consumer may optionally include the Data Source NF Instance (or NF Set) ID and ADRF information indicating whether the data are to be stored in an ADRF and, optionally, an ADRF ID.

Editor’s Note: Clause xx will provide the Ndccf_DataManagement service description.

- If the NF instance or NF Set ID is not provided by the data consumer, the DCCF determines the NF instances that can provide data as described in clause 5A.2 and clause 6.2.2.2. If the consumer requested storage of data in an ADRF, but the ADRF ID is not provided by the data consumer, or the collected data is to be stored in an ADRF according to configuration on the DCCF, the DCCF selects an ADRF to store the collected data.

- The DCCF determines whether the data requested in step 1 are already being collected, as described in clause 5A.2.

If the data requested are already being collected by a data consumer, the DCCF adds the data consumer to the list of data consumers that are subscribed for these data.

- The DCCF sends an Nmfaf_3daDataManagement_Configure(Data Consumer Information, Notification Information, Formatting Conditions, Processing Instructions) to configure the MFAF to map notifications received from the Data Source to outgoing notifications sent to endpoints, and to instruct the MFAF how to format and process the outgoing notifications.

Consumer Information contains for each notification endpoint, the data consumer provided Notification Target Address (+ Data Consumer Notification Correlation ID to be used by the MFAF when sending notifications in step 7. Notification Information identifies Event Notifications received from the Data Sources and contains the MFAF Notification Target Address (+ MFAF Notification Correlation ID).

Editor’s Note: It is FFS how the DCCF learns/chooses the MFAF Notification Target Address and MFAF Notification Correlation ID (eg: the Target Address may be obtained from the NRF when discovering the MFAF or via a request/response from the MFAF, and Correlation IDs may be selected by the DCCF or obtained via request/response from the MFAF).

- If the data subscribed in step 1 partially matches data that are already being collected by the DCCF from a Data Source, and a modification of this subscription to the Data Source would satisfy both the existing data subscriptions as well as the newly requested data, the DCCF invokes Nnf_EventExposure_Subscribe(Subscription Correlation ID) with parameters indicating how to modify the previous subscription (as specified in clause 5A.2). The DCCF adds the data consumer to the list of data consumers that are subscribed for these data.

If the data requested at step 1 are not already available or not being collected yet, the DCCF subscribes to data from the NF using the Nnf_EventExposureSubscribe(Data Specification, Notification Target Address=MFAF (+ MFAF Notification Correlation ID)) service operation as specified in clause 5A.2 and clause 6.2.2.2. The DCCF adds the data consumer to the list of data consumers that are subscribed for these data.

6.	When new output data are available, the Data Source uses Nnf_EventExposure_Notify to send the data to the MFAF. The Notification includes the MFAF Notification Correlation ID.

7.	The MFAF uses Nmfaf_3caDataManagement_Notify to send the data  to all notification endpoints indicated in step 4. Notifications are sent to the Notification Target Address(es) using the Data Consumer Notification Correlation ID(s) received in step 4. Data sent to notification endpoints may be processed and formatted by the MFAF, so they conform to delivery requirements specified by the data consumer. The MFAF may store the information in ADRF if requested by consumer or if required by DCCF configuration

NOTE:	According to Formatting Instructions provided by the data consumer, multiple notifications from a Data Source can be combined in a single Nmfaf_3caDataManagement_Notify, so many notifications from the Data Source results in fewer notifications (or one notification) to the data consumer. Alternatively, a notification can instruct the data notification endpoint to fetch the data from the MFAF before an expiry time.

8.	If a Nmfaf_3caDataManagement_Notify contains a fetch instruction, the notification endpoint sends a Nmfaf_3caDataManagement_Fetch request to fetch the data from the MFAF.

9.	The MFAF delivers the data to the notification endpoint.

10.	When the data consumer no longer wants data to be collected, it invokes Ndccf_DataManagement_Unsubscribe(Subscription Correlation ID), using the Subscription Correlation Id received in response to its subscription in step 1.

11.	If there are no other data consumers subscribed to the data, the DCCF unsubscribes with the Data Source.

12.	The DCCF deconfigures the MFAF so it no longer maps notifications received from the Data Source to the notification endpoints configured in step 4.

#### 6.2.S.x.5	Historical Data Collection via Messaging Framework

The procedure depicted in figure 6.2.S.x.5-1 is used by data consumers (e.g. NWDAF) to obtain historical data, i.e. data related to past time period . The data consumer obtains data using Ndccf_DataManagement_Subscribe service operation, where the subscription results in one or more notifications depending on how the data is retrieved from the ADRF or NWDAF and how the data is formatted. Whether the data consumer uses this procedure or directly contacts the ADRF or NWDAF is based on configuration.

Figure 6.2.S.x.5-1: Historical Data Collection via Messaging Framework

- The data consumer requests data via DCCF by invoking the Ndccf_DataManagement_Subscribe(Service Operation, Data Specification, Time Window, Formatting Instructions, Processing Instructions, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID). The data consumer may specify one or more notification endpoints to receive the data.

Service_Operation is the service operation used to acquire the data from a data source, Data Specification provides Service_Operation-specific required parameters (e.g. event IDs, UE-ID(s) and optional input parameters used to retrieve the data. Time Window specifies a past time period and comprises a start and stop time, and Formatting and Processing Instructions are as defined in clause 5A4. The data consumer may optionally include the ADRF or NWDAF instance (or ADRF Set or NWDAF Set) ID where the stored data resides.

Editor’s Note: Clause xx will provide the Ndccf_DataManagement service description.

- If an ADRF or NWDAF instance or ADRF Set ID or NWDAF Set ID is not provided by the data consumer, the DCCF determines if any ADRF or NWDAF instances might provide the data as described in clause 5B and 5A.2.

NOTE:	An ADRF or NWDAF may have previously registered data it is collecting with the DCCF.

Editor’s Note:	clause 5B is to provide an ADRF functional description. ADRF or NWDAF selection by the DCCF may be based on the ADRF or NWDAF profile in the NRF (eg: ADRF or NWDAF that stores data for a specific NF Type in a geographic area) in addition to registrations by the ADRF or NWDAF to the DCCF of data the ADRF or NWDAF is collecting.

- The DCCF sends an Nmfaf_3daDataManagement_Configure(Data Consumer Information, Notification Information, Formatting Conditions, Processing Instructions) to configure the MFAF to map notifications received from the ADRF or NWDAF to outgoing notifications sent to endpoints, and to instruct the MFAF how to format and process the outgoing notifications.

"Data Consumer Information" contains for each notification endpoint, the data consumer provided Notification Target Address (+ Data Consumer Notification Correlation ID) to be used by the MFAF when sending notifications. "Notification Information" identifies Event Notifications received from the ADRF or NWDAF,  and contains the MFAF Notification Target Address (+ MFAF Notification Correlation ID)

Editor’s Note: It is FFS how the DCCF learns/chooses the MFAF Notification Target Address and MFAF Notification Correlation ID (eg: the Target Address may be obtained from the NRF when discovering the MFAF or via a request/response from the MFAF, and Correlation IDs may be selected by the DCCF or obtained via request/response from the MFAF)

- (conditional) If the DCCF determines that an ADRF instance might provide the data, or an ADRF instance or Set was supplied by the data consumer, the DCCF sends a request to the ADRF, using Nadrf_DataRetrieval_Subscribe(Data Specification, Notification Target Address=MFAF).

Editor’s note:	Appropriate reference to service operation specification clause will be added when available.

- The ADRF responds to the DCCF with an Nadrf_DataRetrieval_Subscribe response indicating if the ADRF can supply the data. If the data can be provided, the procedure continues with step 8.

- (conditional) If the DCCF determines that an NWDAF instance might provide the data, or an NWDAF instance or NWDAF Set was supplied by the data consumer, the DCCF sends a request to the NWDAF, using Nnwdaf_DataManagement_Subscribe(Data Specification, Notification Target Address=MFAF) service operation.

Editor’s note:	Appropriate reference to service operation specification clause will be added when available.

- The NWDAF responds to the DCCF with an Nnwdaf_DataManagement_Subscribe response indicating if the NWDAF can supply the data.

- The ADRF or the NWDAF sends the requested data (e.g. one or more stored notifications archived from a data source) to the MFAF. The data may be sent in one or more notification messages.

- The MFAF uses Nmfaf_3caDataManagement_Notify to send data to all notification endpoints indicated in step 3. Notifications are sent to the Notification Target Address(es) using the Data Consumer Notification Correlation ID(s) received in step 3. Data sent to notification endpoints may be processed and formatted by the MFAF, so they conform to delivery requirements specified by the data consumer.

NOTE:	According to Formatting Instructions provided by the data consumer, multiple notifications from an ADRF or NWDAF can be combined in a single Nmfaf_3caDataManagement_Notify so many notifications from the ADRF or NWDAF results in fewer notifications (or one notification) to the data consumer. Alternatively, a Nmfaf_3caDataManagement_Notify can instruct the data notification endpoint to fetch the data from the MFAF before an expiry time.

- If a notification contains a fetch instruction, the notification endpoint sends a Nmfaf_3caDataManagement_Fetch request to fetch the data from the MFAF.

- The MFAF delivers the data to the notification endpoint.

- When the data consumer no longer wants data to be collected or has received all the data it needs, it invokes Ndccf_DataManagement_Unsubscribe(Subscription Correlation ID), using the Subscription Correlation Id received in response to its subscription in step 1.

- If the data are being provided by an ADRF and there are no other data consumers subscribed to the data, the DCCF unsubscribes with the ADRF.

- If the data are being provided by an NWDAF and there are no other data consumers subscribed to the data, the DCCF unsubscribes with the NWDAF.

- The DCCF deconfigures the MFAF so it no longer maps notifications received from the ADRF or NWDAF to the notification endpoints configured in step 3.

END OF CHANGES