3GPP TSG-SA/WG2 Meeting #143e	S2-2100781

# Elbonia, Feb 24-Mar9, 2021

Source:	ZTE

Title:	KI#5 conclusion

Document for:	Agreement

Agenda Item: 	8.4.1

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#5

# 1	Discussion

# 2	Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

## 8.5	Conclusion for Key Issue #5

To enable a 5GS to support KI#5(dynamic adjustment to meet the limitation of data rate per network slice in UL and DL), the following new functionalities in the 5GS are needed:

1)	The functionality of the NSAC to count the UEs register in the network slice as per KI#1 is reused and the data rate limitation per network slice is applied by restricting the number of registered UEs which have a data rate limitation in the Network slice in the subscription information and restricting the number of UEs not subject to a subscribed rate limitation for the Network slice while also assigning to these a "run-time" data rate restriction which can be time varying based on policy. The rate per UE is restricted then as per KI#3. This avoids UL data transmission above the expected aggregate rate, but it may underutilize the Network Slice.

2) 	Another option which is allowing higher utilization is the enforcement at peering links on the N6 interface, but this is outside the scope of 3GPP. The downside is that it does not stop UL data above the rate and there may be packets dropped in UL at the peering points after having been needlessly transmitted at the Uu. If more than one link is used at N6, then the enforcement may require splitting the rate across these links, so underutilization is possible if there are links some of which not fully utilized. However, this option has no 3GPP impact.

-

/*******************End of Change****************/