3GPP TSG-SA/WG2 Meeting #143e	S2-2100781r03

# Elbonia, Feb 24-Mar9, 2021

Source:	ZTE

Title:	KI#5 conclusion

Document for:	Agreement

Agenda Item: 	8.4.1

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#5

1	Discussion

2	Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

8.5	Conclusion for Key Issue #5

To enable a 5GS to support KI#5(dynamic adjustment to meet the limitation of data rate per network slice in UL and DL), the following new functionalities in the 5GS are needed:

-	Storage: If a network slice is subject to NSAC on limitation of data rate per network slice in UL and DL, it is assumed that the O&M should have for this network slice the NW Slice maximum data rate parameter, total slice-MBR for UL and DL, which limit the aggregated data rate across all GBR and Non-GBR flows within the slice. This parameter is configured and stored in new network function NSACF.

-	Monitoring: The new NSACF monitors the aggregated data rate for the network slice by collecting the  Average data rate of the network slice from UPF, or from NWDAF, or by aggregating the slice-MBR for each registered UE in the network slice. The new NSACF perform admission control by comparing the aggregated data rate for the network slice with the maximum value.

-	Enforcement: The NSACF ensures that the maximum data rate per slice is not exceeded. If exceeded then the NSACF can take the following actions: a) inform the AMF to adjust the S-MBR as per KI#3, or b) inform the PCF to adjust the Session AMBR or MFBR for each PDU Session in the network slice, or c) inform the AMF to reject new PDU session establishments with the network slice as per KI#2 or reject the new UE registration in the network slice as per KI#1.

NOTE: The service operation for NWDAF will be defined in eNA_Ph2

NOTE: How the data from UPF is retrieved is not specified in this release.

/*******************End of Change****************/