3GPP TSG-SA/WG2 Meeting #143e	S2-2100781r05

# Elbonia, Feb 24-Mar9, 2021

Source:	ZTE

Title:	KI#5 conclusion

Document for:	Agreement

Agenda Item: 	8.4.1

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#5

# 1	Discussion

# 2	Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

## 8.5	Conclusion for Key Issue #5

To enable a 5GS to support KI#5(dynamic adjustment to meet the limitation of data rate per network slice in UL and DL), the following new functionalities in the 5GS are needed:

-	Storage: If a network slice is subject to NSAC on limitation of data rate per network slice in UL and DL, it is assumed that the O&M should have for this network slice the NW Slice maximum data rate parameter, total slice-MBR for UL and DL, which limit the aggregated data rate across all GBR and Non-GBR flows within the slice. This parameter is configured and stored in the UDR.

-	Monitoring: The new PCF monitors the aggregated data rate for the network slice by collecting, or by 3) aggregating the slice-MBR for each registered UE in the network slice.

-	Enforcement: The PCF ensures that the maximum data rate per slice is not exceeded. If exceeded then can take the following actions: a) PCF to adjust the S-MBR as per KI#3, or b) inform the PCF to adjust the Session AMBR or MFBR for each PDU Session in the network slice, or c

/*******************End of Change****************/