3GPP TSG-SA/WG2 Meeting #143e	S2-2100781r08

# Elbonia, Feb 24-Mar9, 2021

Source:	ZTE

Title:	KI#5 conclusion

Document for:	Agreement

Agenda Item: 	8.4.1

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#5

1	Discussion

2	Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

8.5	Conclusion for Key Issue #5

To enable a 5GS to support KI#5(dynamic adjustment to meet the limitation of data rate per network slice in UL and DL), the following new functionalities in the 5GS are needed:

-	Storage: it is assumed the PCF stores the maximum data rate for the network slice. If there are multiple PCFs subject to the S-NSSAI, the data rate should be coordinated among PCFs with UDR.

-	Monitoring: The PCF can make decisions of admission based on:

a)	aggregated data rate for the slice as provided by the NSACF. The NSACF computes the aggregated data rate for the slice by considering the Slice-MBR for each UE already admitted in the network slice; or

b)	estimated experienced average rate in the Network Slice over a configurable time window as provided by the NWDAF

-	Enforcement: The PCF ensures that the maximum data rate per slice is not exceeded. The PCF can modulate the S-MBR per UE (for those UEs that have no Subscribed S-MBR), the number of admitted UEs in the PCF, and the rate (GFBR/MFBR+Session-AMBR if possible) or number of PDU sessions in the PCF.

/*******************End of Change****************/