3GPP TSG-SA/WG2 Meeting #143e	S2-2100781r14

# Elbonia, Feb 24-Mar9, 2021

Source:	ZTE

Title:	KI#5 conclusion

Document for:	Agreement

Agenda Item: 	8.4.1

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#5

1	Discussion

2	Proposal

It is proposed to agree the changes:

/********************Start of Change***************/

8.5	Conclusion for Key Issue #5

To enable a 5GS to support KI#5(dynamic adjustment to meet the limitation of data rate per network slice in UL and DL), the following new functionalities in the 5GS are needed:

-	Storage: If a network slice is subject to limitation of data rate per network slice, it is assumed that NW Slice maximum data rate parameter, which is the aggregate data rate across all GBR and Non-GBR flows within the slice,is configured and stored in the UDR.

-	Monitoring: The PCF monitors the aggregated data rate for the network slice by computing the aggregate NW Slice data rate of the UEs that have PDU sessions admitted in the network slice and/or the utilized bandwidth of the Network Slice.

-	Enforcement: the PCF ensures that no new PDU sessions or QoS flows of network slices with registered UEs are admitted if this may cause overflowing the NW Slice maximum data rate parameter stored in the UDR.

NOTE 1: Multiple PCFs for this key issue is to be addressed in normative phase.

NOTE 2: NWDAF aspects is to be confirmed with eNA_Ph2 during normative phase.

/*******************End of Change****************/