3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100865

February 24 – March 09, 2021, Online

# Title:	LS on feedback on RAN3 service continuity solutions

# Release:	Release 17

# Work Item:	FS_NR_Slice

Source:	SA2

To:	RAN3

Cc:

Contact Person:

Name:	alessio.casati

Tel. Number:

E-mail Address:	alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

# 1	Overall description

SA2 has examined the candidate solutions described in RAN3 TR 38.832 for slice service continuity from a feasibility and preference standpoint and would like to give the following feedback:

From SA2 standpoint non CN/UE impacting solutions are feasible, but only address scenarios 1,3,5,6.

To address scenarios 2,4, UE impact and CN impact are expected as the S-NSSAI changes upon remapping. . As a minimum, this would require SA2 and RAN3 to specify a mechanism to make the source gNB aware if the UE supports and accept re-mapping or not (to even consider the handover to the target). Unless there are showstoppers, solutions that do not restrict the type of handover supported, and minimize the impacts on the CN, should be preferred. If RAN3 would agree to support scenario 2,4 for rel-17, SA2 would propose extending Rel-17 eNS_Ph2 scope  to SA plenary #92 and  SA plenary agreement would be required to add this in the scope of rel-17. The specific solution to be supported for scenario 2 should be agreed between RAN3 and SA2

# 2	Actions

To RAN3

ACTION: 	SA2 kindly requests RAN3 to take the above feedback into account.

# 3	Dates of next TSG SA WG2 meetings

SA WG2 Meeting #144E 12 April – 16 April 2021                             Elbonia