3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100865

February 24 – March 09, 2021, Online

Title:	LS on feedback on RAN3 service continuity solutions

Release:	Release 17

Work Item:	FS_NR_Slice

Source:	SA2

To:	RAN3

Cc:

Contact Person:

Name:	alessio.casati

Tel. Number:

E-mail Address:	alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA2 has examined the candidate solutions described in RAN3 TR 38.832 for slice service continuity from a feasibility and preference standpoint and would like to give the following feedback:

The scenarios described in the TR are valid.

From SA2 standpoint non CN/UE impacting solutions are feasible, but only address scenarios 1,3,5,6.

To address scenarios 2,4, UE impact and CN impact are expected as the S-NSSAI changes upon remapping. From SA2 consideration,  this would require to specify a mechanism to make the network is aware if the UE supports and accept re-mapping or not. The 5GC may determine the re-mapping policy to NG-RAN based on UE subscription data, the S-NSSAI(s) for target RA, relationship between S-NSSAI(s) and NFs. And 5GC should also update UE Allowed NSSAI to keep the alignment between S-NSSAI and PDU session. For roaming case, the changing of mapping relationship between V-PLMN S-NSSAI(s) and H-PLMN S-NSSAI(s) should also be considered.

Unless there are showstoppers, solutions that do not restrict the type of handover supported, nor restrictions to the current principles of dynamic selection of CN NFs selected for the UE, should be preferred. If RAN3 would agree to support scenario 2,4 for rel-17, SA2 would propose extending Rel-17 eNS_Ph2 scope to SA plenary #92 and  SA plenary agreement would be required to add this in the scope of rel-17. The specific solution to be supported for scenario 2 should be agreed between RAN3 and SA2

2	Actions

To RAN3

ACTION: 	SA2 kindly requests RAN3 to take the above feedback into account.

3	Dates of next TSG SA WG2 meetings

SA WG2 Meeting #144E 12 April – 16 April 2021                             Elbonia