3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100871r09

Elbonia, February 24 – March 09, 2021

Source:	China Mobile, Huawei, Hisilicon, Alibaba

Title:	Network exposure via local NEF

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC/Rel-17

Abstract of the contribution: This contribution proposes the Network exposure via local NEF according to conclusion in clause 9.4 in TR 23.748.

# Discussion

This paper is to incorporate network exposure via local NEF part conclusion in 9.4 of TR 23.748 in TS 23.548 clause 6.4.

# Proposal

It is proposed to add the following solution to TS 23.548.

************************ 1st Change, all new text************************

## 6.4	Network Exposure to Edge Application Server

### 6.4.1	General

Editor’s Note: This clause describes how to expose information that needs to be delivered timely to applications deployed in edge hosing environments. This clause refers to TS 23.501, TS 23.502 and TS 23.503 for detailed exposure procedures including the enhancements proposed in TR 23.748 clause 9.4. Privacy considerations might be addressed as well.

Editor’s Note: New clauses 6.4.x may be added later for alternative solutions described in NOTES in 9.4 based on input contributions.

Some real time network information, e.g. user path latency, are useful for application layer. In this release,  in order to expose network information timely to local AF, the L-PSA UPF exposes network information i.e. QoS monitoring results as defined in TS 23.501 [2], clause 5.33.3,  to the local AF.

NOTE 1: For Edge Computing scenario, the local AF may refer to the EHE or EAS deployed in EHE.

NOTE 2:	Local PSA UPF can expose the QoS monitoring results to local AF via N6. How to deliver the information on N6 is out of SA2 scope.

### 6.4.2	Network Exposure to Edge Application Server via Local NEF

Editor’s Note: This clause describes network exposure via local NEF as described in bullets in TR 23.748 clause 9.4.

Local NEF deployed at the edge is used to support network exposure timely to local AF. The local NEF may support one or more of the functionalities described in TS 23.501[2] clause 6.2.5.0. and may support a subset of the APIs specified for capability exposure based on local policy. In order to support the network exposure locally, the local NEF shall support event exposure service operation to the local AF. The local NEF selection by AF is described in TS 23.501[2] clause 6.2.5.0 and 6.3.14.

The local AF subscribes the low latency exposure of QoS Monitoring results to PCF via a local NEF or NEF. The local AF may also subscribe the events via PCF directly. Based on the indication of local event notification and operator's policy, the PCF shall include an indication of local event notification (including target local NEF address) within the PCC rule that it provides to the SMF.

The SMF sends the QoS monitoring request to the RAN and N4 rules to the L-PSA UPF to indicate the service data flow needs local notification of QoS Monitoring. When GTP-U Path monitoring is used for QoS monitoring, that is also activated if needed. This is as defined in 23.501 clause 5.33.3. Upon the detection of the QoS monitoring event, the L-PSA UPF sends the notification to the AF via Local NEF.

During UE mobility, the SMF may trigger the L-PSA UPF relocation/reselection and then send the N4 rules to the new L-PSA UPF to indicate the service data flow needs local notification of QoS Monitoring. The UE mobility may also trigger AF relocation or local NEF reselection,  then the local AF should update the subscription for local exposure with QoS monitoring results via local NEF, towards the  PCF. This updated /new subscription is then propagated via SMF (via PCC rule updates) and then to the L-PSA UPF via N4 rules.

# Figure 6.46.2-1: The association establishment between local UPF and local NEF

0.   The UE establishes a PDU session as defined in clause 4.3.2.2.1 of TS 23.502 [3] A Local PSA is used by this PDU Session.

1.	The AF initiates an AF session with required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3].

In the request, the AF may subscribe local notification of QoS monitoring to PCF via Local NEF or NEF. For the QoS monitoring, the AF shall include the corresponding QoS monitoring parameters as defined in clause 5.33.3 of TS 23.501[2].

The local AF may find a local NEF based on existing method as in TS 23.501 clause 6.2.5.0 and parameters in clause 6.3.14. The indication for AF request network real-time information is also provided.

The AF may also first initiate an AF Session with PCF and later subscribe to local notification of QoS monitoring to PCF by invoking Npcf_Authorization_Subscribe service operation.

Editor’s Note: how to handle the duplicated subscription (via both SMF->PCF->NEF and UPF->NEF) is FFS.

2. The PCF makes the policy decision and initiates the PDU session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3], step 1b, 3b, 4-8b.	If the local notification of QoS monitoring is subscribed, the PCF includes the indication of local event notification (including target local NEF address) within the PCC rule. The SMF sends local QoS monitoring parameters to the L-PSA UPF via N4 rules.

3-The L-PSA UPF obtains QoS monitoring information as defined in 23.501 clause 5.33.3,.

4	The L-UPF sends the notification related with QoS monitoring information over Nx

Editor’s Note: it is FFS whether the QoS monitoring result report from L-PSA UPF to local NEF is based on SBI or not.

5.	Local NEF reports the real-time network information to local AF through Nnef_EventExposure_Notify.

6.	Due to e.g. UE mobility, the PSA relocation and/or EAS relocation may happen as described in clause 6.3.

7.	The new AF may initiate a new AF session to the new Local NEF or NEF/PCF to (re-)subscribe the local notification of QoS monitoring as described in steps 2-4.

8. The old AF revokes the AF session with the old local NEF or NEF/PCF.

********************************* End Change ****************************