3GPP TSG-SA2 # 143E (e-meeting)	S2-2100882

# Febuarary.24 – March 1, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 244 | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Increasing efficiency of data collection (Architecture part) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2020-12-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of TR23700-91 of FS_eNA_Ph2, “ KI#11: Increasing efficiency of data collection”, it is conclude that: “- When a Data Collection Coordination Functionality (DCCF) is deployed, it is used to coordinate collection of data from one or more NF(s) based on data collection requests from one or more Consumer NF(s). - A Data Repository Functionality (DRF) allows to store collected data and analytics.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR: [Change-1] Clause 6.2.x Add two procedures to TS 23.288, one describes the procedure of data collection by using DCCF to control signalling and forward data, and the other describes the procedure of data collection by using DCCF to control signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R17 TS 23.288 does not involve the conclusion for KI#11: Increasing efficiency of data collection of FS_eNA_Ph2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * (all new text)

### 6.2.X	 Data collection using DCCF

6.2.x.w 	 Data collection profile registration

In some cases data consumers (e.g. NWDAF or ADRF) collect data from data source NF directly, e.g. when NWDAF is co-located with 5GC NF.

To enable data consumers can get the data which has been collected by NWDAF or ADRF directly (i.e. not via DCCF) , the NWDAF or ADRF may register/update the data collection profile to the DCCF during/after the procedure of data collection. DCCF can then determine some requested data is available in NWDAF or ARDF and can coordinate data collection based on the data collection profile.

The procedure depicted in figure 6.2.w.x-1 is used by data source(e.g. NWDAF or ADRF) to register data profile to DCCF.

Figure 6.2.w.x-1: Procedure for the NWDAF or ADRF register data profile to DCCF

1.	An ADRF or NWDAF instance is collecting or has collected data directly, e.g. from collocated NF.

2.	The ADRF or NWDAF requests to register/update data collection profile (Analytics ID, Event ID list, ADRF ID or NWDAF ID) to DCCF by invoking the Ndccf_ContextManagement_Request. The registration / update request can be triggered by the acceptation of subscription for data collection responded by the data source (e.g. collocated NF), it can be before the start of data collection or after the completion of data collection.

Editor’s Note:	Whether the data collection profile contains other parameters (such as time period of data collection, area of interest, data obtaining status, etc) is FFS.

3.	The DCCF responds to the ADRF or NWDAF with a Ndccf_ContextManagement_Response.

4.	To obtain historical data and if the data consumer is configured to be collect data via the DCCF using Ndccf_DataManagement_Subscribe service operation, the data consumer uses the procedures described in clause 6.2.x.2 or 6.2.x.3.

* * *End of Changes * * *