SA WG2 Meeting #143E (e-meeting)	S2-2100925r02

Elbonia, Feb 24 – Mar 9, 2021

Source:	Qualcomm Incorporated, OPPO, Nokia, Nokia Shanghai Bell, CATT

Title:	Closing outstanding issues for UE-to-Network Relay conclusions

Document for:	Discussion/Approval

Agenda Item:	8.8.1

Work Item / Release:	FS_5G_ProSe/Rel-17

Abstract of the contribution: This contribution proposes changes to close the outstanding issues to conclude, KI#3: UE-to-NW relay, considering RAN2 WG input.

>>>>Start Changes<<<<

## 8.3	Key Issue #3: Support of UE-to-Network Relay

For Key Issue #3 (Support of UE-to-Network Relay), SA WG2 recommends:

-

-	L2 UE-to-Network Relay solution proceed into normative work, as an optional feature for UEs and the network..

-	L3 UE-to-Network Relay solution proceed into normative work, as an optional feature for UEs and the network.

-	SA WG3 conclusions will be addressed by SA WG2 during the normative work.

NOTE:	TSG SA/RAN plenary to coordinate WI content update/approval at TSGs#91e to ensure alignment between RAN and SA.

-

-    Layer-3 UE-to-Network Relay and Layer-2 UE-to-Network Relay discovery and co-existence can be discussed in the normative phase e.g.  based on the solution of TR 23.733 [26] clause 6.2.3, as necessary.

The following are concluded for the L3 UE-to-Network Relay solution:

-	No showstopper has been identified by SA WG2 for L3 UE-to-Network solution. SA WG2 recommends L3 UE-to-Network Relay proceed into normative work. SA WG3 conclusions will be addressed by SA WG2 during the normative work.

-	Solution #6 is taken as baseline in case the UE-to-Network Relay is a trusted entity by the Remote UE. Solution#23 is agreed to be used to provide end-to-end security for the Remote UE in case the UE-to-Network Relay is not a trusted entity by the Remote UE.

-	For L3 Relay discovery procedure, it is proposed to adopt the standalone discovery procedure (i.e. Model A and Model B), and, the additional information advertised by Relay UE as described in Sol#28 as the basis for normative work.

-	For the L3 relay operation support, Remote UE uses URSP rules to route the traffic on suitable communication path (as described in Sol#26).

-	Security aspects will be addressed in the normative phase based on SA WG3 conclusions.

NOTE 1:	The procedures to support authentication of Remote UE and Relay UE by the network will be determined by SA WG3.

-	For the Remote UE to use the network resources (e.g. PDU Session and Network Slice) of the Relay UE's serving network, the network-controlled authorization procedures will be determined in the normative phase with coordination with SA WG3. The alignment with the associated security procedures to authenticate the Remote UE and Relay UE will be carried out in normative phase via coordination with SA WG3.

-	The secondary authentication for a Remote UE will be determined by SA WG3. The alignment with the associated security procedures for secondary authentication of the Remote UE will be carried out in normative phase via coordination with SA WG3.

-	For QoS handling, following aspects in Solution #24 and Option #2 of Solution #25 are selected as basis for normative work:

-	L3 Relay can be configured with the 5QIs and PQIs mapping. Based on the mapping or, in case of a non-configured mapping of a requested QoS parameter, based on its implementation, the L3 relay translates the Uu QoS parameters to PC5 QoS parameters and vice versa.

-	To support the dynamic QoS handling, relay UE determines the Uu QoS parameters and PC5 QoS parameters by taking into account the end-to-end QoS requirements provided by remote UE based on its configured QoS mapping information or, in case of a non-configured mapping of a requested QoS parameter, based on its implementation, and initiates PDU session modification procedure and L2 link modification procedure to setup corresponding QoS Flows over Uu and PC5.

-	The SMF of the L3 Relay provides the corresponding QoS rules and flow level QoS parameters to the L3 Relay as part of the PDU session establishment or modification procedures as defined in TS 23.502 [8], clause 4.3.2 and 4.3.3. Alternatively, reflective QoS control over Uu as defined in TS 23.501 [6], clause 5.7.5.3 can be leveraged for dynamic QoS handling of Remote UE to save on signalling between SMF and L3 Relay.

-	Based on signalled QoS rules (via SMF) or derived QoS rules (Uplink Uu via reflective QoS), the UE-to-Network Relay may use the L2 Link Modification procedures as defined in TS 23.287 [5], clause 6.3.3.4 to either move the corresponding ProSe service(s) to the mapped existing PC5 QoS flow or to set up a new PC5 QoS flow.

-     Service continuity for L3 UE-to-Network Relay can be supported using solution#23 (not in a lossless manner, but with IP address preservation) or application layer mechanisms (e.g. solution #6) if supported through change of IP address.

NOTE 2:	The radio aspects of relay (re-)selection criteria and procedures for L3 UE-to-Network Relay are covered by RAN WG2 in TR 38.836[32].

The followings are concluded for the L2 UE-to-Network Relay solution:

-	No showstopper has been identified by SA WG2 for L2 UE-to-Network Relay solution. SA WG2 recommends L2 UE-to-Network Relay solution proceed into normative work. SA WG3 conclusions will be addressed by SA WG2 during the normative work.

-	Adopt Control and User Plane Protocols as described in Annex A, with confirmation from RAN WGs.

-	The Remote UE has a NAS connection with 5GC and Remote UE Registration and Connection establishment/management, the related procedure in solution #7 can be taken as baseline.

-	For Relay discovery, the standalone discovery is used, and both Model A and Model B are supported.

-	For paging the concluded solution in clause 6.6.2 of TR 23.733 [26] can be reused based on the assumption captured in clause 4.5.5.2 of TR 38.836 [32] adopted by RAN WG2.

-	For mobility, RAN2 WG agreed on reusing the Rel-15/16 NR handover procedure as the baseline AS layer solution to guarantee lossless service continuity described in clause 4.5.3 of TR 38.836 [32].

NOTE 3:	Support of end-to-end QoS between the Remote UE and RAN will be specified by RAN WG2.

NOTE 4:	Support end-to-end security between the Remote UE and RAN will be specified by RAN WG2 and SA WG3.

NOTE 5:	The radio aspects of relay (re-)selection criteria and procedures for L2 UE-to-Network Relay are covered by RAN WG2 in TR 38.836[32].

>>>>End Changes<<<<