3GPP TSG-SA/WG2 Meeting #143e	S2-2100959r04

# Elbonia, Feb. 24 – Mar.9 , 2020			   (revision of S2-210xxxx)

Source:	Intel, LG Electronics, Samsung

Title:	ProSe Policy/Parameter provisioning for Direct Communication

Document for:	Agreement

Agenda Item: 	8.8.2

Work Item / Release:	5G_ProSe / Rel-17

Abstract of the contribution: This contribution captures the aspect of Policy/parameter provisioning for Direct Communication concluded in TR 23.752.

# 1	Proposal

It is proposed to agree the changes based on the conclusion in TR 23.752:

/************Start of Change (all new text) **************/

### 5.1.3 Authorization and Provisioning for ProSe Direct Communication

#### 5.1.3.1 Policy/Parameter Provisioning

The following sets of information for ProSe Direct Communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN".

NOTE 1:	In this specification, {When the UE is "served by NG-RAN"} and {When the UE is "not served by NG-RAN"} are relevant to ProSe Direct Communications over NR PC5 reference point.

2)	Groupcast mode ProSe Direct Communication policy/parameters:

-	For each application layer group supported include the parameters that enable the UE to perform Groupcast mode ProSe Direct Communication when provisioned from PCF or pre-configured in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID

-	ProSe Group IP multicast address

-	Indication whether the UE should use IPv4 or IPv6 for that group

-	For a specific Group configured to operate using IPv4, optionally an IPv4 address to be used by the UE as a source address. If none is provisioned, then the UE shall use Dynamic Configuration of IPv4 Link-Local Addresses IETF RFC 3927 [xx] to obtain a link local address for the Group.

-	Include group security related content for Groupcast mode ProSe Direct Communication.

NOTE 2:	More details on the necessary security aspect will be defined in SA3 specifications.

NOTE 3:	Groupcast mode ProSe Direct Communication parameters can be provided from ProSe Application Server to the UE.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 4:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

4)	Policy/parameters related to privacy:

-	The list of ProSe services (i.e. Application IDs) with Geographical Area(s) that require privacy support.

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (i.e. Application IDs) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (i.e. Application IDs) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (i.e. Application IDs) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (i.e. Application IDs) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 5:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one ProSe service. In the case where different ProSe services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one ProSe service, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

-	The mapping of ProSe services (i.e. Application IDs) to PC5 QoS parameters defined in clause 5.4.2 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	SLRB configurations, i.e. the mapping of PC5 QoS profile(s) to SLRB(s), when the UE is "not served by NG-RAN".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2 of TS 23.287 [xx], and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1 of TS 23.287 [xx].

6)	Path selection policy:

-	The mapping of ProSe services to path preference (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.12.

Editor’s note: It is FFS on how to identify ProSe services for Path selection.

NOTE 6:	The path selection policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

7)	Validity time indicating the expiration time of the Policy/Parameter for ProSe Direct Communication.

NOTE 7:	The Application ID is defined in TS 23.503 [yy].

/*******************End of Change****************/