SA WG2 Meeting #143E (e-meeting)	S2-2100965r03

# Feb 24 - Mar 09, 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2674 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Detection of satellite backhaul based on configuration information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | Mobile backhaul interconnects gNB with AMF over N2 and interconnects gNB with UPF over N3. If AMF and UPF are preconfigured that some connections are reserved for satellite backhaul or a list of gNB ID corresponds to satellite backhaul, when it receives traffic from the communication connection reserved for satellite backhaul or from one of the gNB in the list, it can then determine that the UE traffic is over satellite backhaul or traffic of the PDU session is over satellite backhaul. The satellite backhaul information will then be used by PCF to determine the policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UPF can determine a satellite backhaul with long delay is used over N3 and report it to the SMF over N4 during N4 Association Setup procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PCF cannot provide a policy based on satellite backhaul information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | -5.8.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core secifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

5.8.2.X	Reporting of satellite backhaul to SMF

In case the UPF is aware that a satellite backhaul with long delay is used towards 5G AN, the UPF may report this to SMF as part of the N4 Node level procedures as described in TS 23.502, clause 4.4.3.

* * * End of changes * * *