3GPP TSG-SA WG2 Meeting #143E e-meeting	S2-2100975r03

# Elbonia, February 24 – March 09, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2590 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS23.502 KI#1 Network Slice Admission Control Function (NSACF) services and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Apple, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Further to: - the eNS-Ph2 conclusions in the TR23.700-40 1.2.0, clause 8.1 for KI#1; and - a decision from SA2#143E meeting preparation telco on eNS-Ph2 to submit CRs based on the so-called push method. this contribution proposes the new feature addition to TS23.502. The changes are based on TR solution ‘6.2 Solution #2: Max number of UEs per Network Slice control at registration’ which is fully compatible with the conclusions in the TR. r01 update 28/02/2021 Based on the feedback from the discussions, made the following changes: - the number of UEs per network slice abvalablty check procedure call for network slices subject to NSAC is moved from the beginning of the registration procedure to the end, just before the Registration Accept message to avoid any delay between the availability check and the potential update of the number of UEs registered with the network slice. - Further clarifications that the number of UEs registered with a network slice subject to NSAC is updated at every UE registration status change from non-registered to registered and vice versa for that network slice. - Added the following Editor’s note as a potential further signalling optimisation for the availability check procedure call: Editor’s note: Whether to trigger a request for availability check only when the number of UEs registered with the network slice has reached a certain threshold (e.g. certain percentage of the maximum number). r02 update 01/03/2021 – Added NSAC start/stop service operation and procedure to resolve the Editor’s Note. r03 update 03/03/2021 – Combined the availability check and update procedures in one procedure and some tidying. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New eNS_Ph2 feature addition for KI#1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNS_Ph2 KI#1 conclusions are not implemented in TS 23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x (new), 5.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 ... CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

### 4.2.x	Network Slice Admission Control Function (NSACF) procedures

The NSACF is configured by the O&M with the maximum number of UEs that is allowed to be served by the network slice that is subject to NSAC. The NSACF controlls the number of the UEs registered per network slice so that the maximum number of UEs per network slice is not exceeded. The NSACF maintains a list of UE IDs registered with the network slice to make sure the UEs are not counted multiple times.

#### 4.2.x.1	Number of UEs per network slice availability check and update procedure

The number of UEs per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of UEs registered with a network slice which is subject to NSAC. The AMF is configured with the information indicating which network slice is subject to NSAC.

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceAvalabltyCheckAndUpdate_Request

4. Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Response

3. The number of UEs per network slice availability check and update.

1. A trigger to perform number of UEs per network slice availability check and update

Figure 4.2.x.1-1: Number of UEs per network slice availability check and update procedure

- The AMF triggers the number of UEs per network slice availability check and update procedure to update the number of UEs registered with a network slice subject to NSAC for the newly registered network slices or for the network slices for which the UE did not renew its registration or deregistered or lost or gain registration for any other reason as a last step of:

- UE Registration procedure, as per clause 4.2.2.2.2, before the Registration Accept in step 21 if the SAC mode is active or after the Registration Accept message if the SAC mode is not active;

- UE Deregistration procedure, as per clause 4.2.2.3;

- Network Slice-Specific Authentication and Authorisation, as per clause 4.2.9.2, before the UE Configuration Update message if the SAC mode is active or after the UE Configuration Update message if the SAC mode is not active;

- AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure, as per clause 4.2.9.3, before the UE Configuration Update message if the SAC mode is active or after the UE Configuration Update message if the SAC mode is not active; or

- AAA Server triggered Slice-Specific Authorization Revocation, as per clause 4.2.9.4, before the UE Configuration Update message if the SAC mode is active or after the UE Configuration Update message if the SAC mode is not active.

- The AMF sends Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Request message to the NSACF. The AMF includes in the message the UE ID, the S-NSSAI(s) for which the number of UEs registered per network slice update is required and the update flag which indicates whether the number of UEs registered with the S-NSSAI(s) is to be increased when the UE has gained registration to network slice(s) subject to NSAC (e.g. after UE registration and NSSAA) or the number of UEs registered with the S-NSSAI(s) is to be decreased when the UE has deregistered from network slice(s) (e.g. after UE deregistration or S-NSSAI(s) status change from allowed to rejected after AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure (clause 4.2.9.3) or AAA Server triggered Slice-Specific Authorization Revocation (clause 4.2.9.4)).

- The NSACF updates the current number of UEs registered for the S-NSSAI, i.e. increases or decrease the number of UEs per network slice based on the information provided by the AMF in the update flag parameter.

If the update flag parameter from the AMF indicates increase and the NSACF finds the UE ID is already in the list of UEs registered with the network slice, the number of the UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the current number of the UEs registered with the network slice. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result.

If the update flag parameter from the AMF indicates decrease, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the AMF and also the NSACF decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices.

- The NSACF returns the Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Response in which the NSACF includes the S-NSSAI(s) for which the maximum number of UEs per network slice has already been reached along with ‘the maximum number of UEs per network slice has already been reached’ result parameter.

If all the S-NSSAI(s) that the UE requested to register for and was entitled to register after checking with the UDM were subject to NSAC and for all of them the NSACF returned the maximum number of UEs per network slice has been reached, the AMF rejects the UE request for registration. In the Registration Reject message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter, a reject cause set to 'maximum number of UEs per network slice reached' for each S-NSSAI and optionally a back-off timer.

Otherwise, the AMF returns Registration Accept message in which the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the NSACF has indicated that the maximum number of UEs per network slice has been reached, a reject cause set to 'maximum number of UEs per network slice reached' for each rejected S-NSSAI and optionally a back-off timer.

#### 4.2.x.2	Slice Availability Check start/stop procedure

The Slice Availability Check start/stop procedure indicates the activation or the dectivation of the Slice Availability Check (SAC) mode in the NSACF for the network slices subject to NSAC.

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceNotify(S-NSSAI, SAC flag=activated/deactivated)

3. Based on the SAC flag the AMF decides when to trigger the update calls to the NSACF

1. The number of UEs registered with a network slice subject to NSAC reaches a certain threshold

Figure 4.2.x.3-1: Slice Availability Check (SAC) start/stop procedure

- The number of UEs registered with a network slice subject to NSAC crosses a certain threshold.

- The NSACF triggers Nnsacf_NumberOfUEsPerSliceNotify in which it includes the S-NSSAI for which the SAC is to be activated or deactivated and a SAC flag set to activated if the number of UEs registered with the network slice is above certain threshold or set to deactivated if the number of the UEs registered with the network slice is below certain threshold.

- The AMF uses the SAC flag to decide when to trigger the number of UEs per network slice availability check and update procedure so that delays to the registration procedure and impact to the already allowed network slices are avoided.

If the SAC flag indicates SAC mode activated, the AMF triggers the number of UEs per network slice availability check and update procedure before the Registration Accept step of the registration procedure or before the UE Configuration Update message as in SAC activated mode the NSACF is to check whether the maximum number of UEs per network slice is reached which may impact the allowed S-NSSAI(s) confirmed by the Registration Accept message and the UE Configuration Update message.

If the SAC flag indicates SAC mode deactivated, the AMF triggers the number of UEs per network slice availability check and update procedure after Registration Accept step of the registration procedure or after the UE Configuration Update message as in SAC deactivated mode the NSACF does not check whether the maximum number of UEs per network slice is reached, i.e. no impact to the allowed S-NSSAI(s) confirmed by the Registration Accept message and the UE Configuration Update message.

* * * * end of 1st change * * * *

* * * * start of 2nd change * * * *

### 5.2.xx	Network Slice Admission Control Function (NSACF) services

#### 5.2.xx.1	General

The following table illustrates the NSACF services.

Table 5.2.xx.1-1: List of NSQCF services

| Service Name | Service Operations |  |  | Operation Semantics |  | Example Consumer(s) |
| --- | --- | --- | --- | --- | --- | --- |
| Nnsacf_NumberOfUEsPerSlice | AvailabilityCheckAndUpdate |  |  | Request/Response |  | AMF |
|  | Notify |  |  |  |  | AMF |

#### 5.2.xx.2	Nnsacf_NumberOfUEsPerSlice services

##### 5.2.xx.2.1	General

Service Description: The Nnsacf_NumberOfUEsPerSlice services control the number of UEs registered with a network slice for the network slices subject to NSAC. The AMF can request the NSACF to check whether the number of UEs registered with a network slice has reached the maximum number of UEs per network slice and the AMF can also request the NSACF to update the number of UEs registered with a network slice.

##### 5.2.xx.2.2	Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate service operation

Service Operation name: Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate

Description: Updates the number of UEs registered with a network slice (e.g. increase or decrease) when the UE registration status for a network slice subject to NSAC has changed. Also, if the number of the UEs registered with the network slice is to be increased and the Slice Availability Check (SAC) mode in the NSACF is activated for that network slice (see Nnsacf_NumberOfUEsPerSliceNotify service operation), the NSACF first checks whether the number of UEs registered with the network slice has reached the maximum number of UEs per network slice threshold. If the maximum number of UEs registered with the network slice has already been reached, the UE registration for that network slice is rejected. If the SAC is not activated, the NSACF increases or decreases the number of UEs per network slice as per the input parameters below.

Inputs, Required: S-NSSAI(s), UE ID, update flag.

The S-NSSAI(s) parameter is a list of one or more network slices for which the number of UEs registered with a network slice is to be updated and potentially checked if the maximum number of UEs per network slice threshold has already been reached.

The UE ID is used by the NSACF to maintain a list of UE IDs registered with the network slice.

The update flag input parameter indicates whether the number of UEs registered with a network slice is to be:

- increased when the UE registers to a new network slice subject to NSAC. If the UE ID is already in the list of UEs registered with the network slice, the number of UEs registered with the network slice is not increased as the UE has already be counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the number of the UEs registered with the network slice. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result;

- decreased when the UE deregisters for a network slice that is subject to NASC. The NSACF decreases the number of the UEs registered with the network slice and removes the UE ID from the list of UEs registered with the network slice.

The NSACF may optionally return the current status of the network slice availability (e.g. a percentage out of the max number) in the network slice availability status parameter. The AMF may use this information for NSACF load balancing in case multiple NSACFs are serving the same network slice.

Outputs, Required: maximum number of UEs per network slice reached, network slice availability status

##### 5.2.xx.2.3	Nnsacf_NumberOfUEsPerSliceNotify service operation

Service Operation name: Nnsacf_NumberOfUEsPerSliceNotify

Description: The NSACF is configured with the information about which network slices are subject to NSAC. The NSACF  may trigger notification to the AMF to indicate the activation of the Slice Availability Check (SAC) mode in the NSACF for a certain network slice which is subject to NSAC when the number of the UEs registered with the network slice is above certain operator defined threshold (e.g. a percentage of the maximum number of UEs allowed to register with the network slice) and the deactivation of the SAC when the number of the UEs registered with the network slice is below certain operator defined threshold which may be same or different from the activation threshold.

Inputs, Required: S-NSSAI, availability check flag

The S-NSSAI input parameter is the network slice for which the NSACF activates or deactivates the Slice Availability Check.

The availability check flag input parameter indicates whether the Slice Availability Check is activated or deactivated. The NSACF checks for network slice availability (i.e. whether the max number of UEs registered with the network slice has been reached only when the NSACF is in the SAC activated mode and the request from the AMF is to increase the number of the UEs registered with the network slice.

* * * * end of 2nd change * * * *