3GPP TSG-SA WG2 Meeting #143E e-meeting	S2-2100975r12

# Elbonia, February 24 – March 09, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2590 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS23.502 KI#1 Network Slice Admission Control Function (NSACF) services and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Apple, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Further to: - the eNS-Ph2 conclusions in the TR23.700-40 1.2.0, clause 8.1 for KI#1; and - a decision from SA2#143E meeting preparation telco on eNS-Ph2 to submit CRs based on the so-called push method. this contribution proposes the new feature addition to TS23.502. The changes are based on TR solution ‘6.2 Solution #2: Max number of UEs per Network Slice control at registration’ which is fully compatible with the conclusions in the TR. r01 update 28/02/2021 Based on the feedback from the discussions, made the following changes: - the number of UEs per network slice abvalablty check procedure call for network slices subject to NSAC is moved from the beginning of the registration procedure to the end, just before the Registration Accept message to avoid any delay between the availability check and the potential update of the number of UEs registered with the network slice. - Further clarifications that the number of UEs registered with a network slice subject to NSAC is updated at every UE registration status change from non-registered to registered and vice versa for that network slice. - Added the following Editor’s note as a potential further signalling optimisation for the availability check procedure call: Editor’s note: Whether to trigger a request for availability check only when the number of UEs registered with the network slice has reached a certain threshold (e.g. certain percentage of the maximum number). r02 update 01/03/2021 – Added NSAC start/stop service operation and procedure to resolve the Editor’s Note. r03 update 03/03/2021 – Combined the availability check and update procedures in one procedure and some tidying. r04 update 03/03/2021 – clarification updates by Hoyeon (Samsung) r05 update 03/03/2021 – clarification and naming updates Genadi (Lenovo) r06 update 03/03/2021 – cleanup Iskren (NEC) r07 update 04/03/2021 – editorials by George (Ericsson) r08 upfate 04/03/2021 – Editors Note update by Srisakul (DoCoMo) r09 update 04/03/2021 – removed the subscripe operation and tidying- Iskren (NEC) |  |  |  |  |  |  |  |  |
| Summary of change: |  | New eNS_Ph2 feature addition for KI#1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNS_Ph2 KI#1 conclusions are not implemented in TS 23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x (new), 5.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 ... CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

4.2.x	Network Slice Admission Control Function (NSACF) procedures

4.2.x.1	General

The Network Slice Admission Control Function procedures are performed for an S-NSSAI which is subject to Network Slice Admission Control (NSAC) as described in TS 23.501 [2].

4.2.x.2	Number of UEs per network slice availability check and update procedure

The number of UEs per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of UEs registered with a S-NSSAI which is subject to NSAC. The AMF is configured with the information indicating which network slice is subject to NSAC.

Editor's Note:	It is FFS how the AMF configuration work in the roaming case.

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceAvalabltyCheckAndUpdate_Request

4. Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Response

3. The number of UEs per network slice availability check and update.

1. A trigger to perform number of UEs per network slice availability check and update

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceAvalabltyCheckAndUpdate_Request

4. Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Response

3. The number of UEs per network slice availability check and update.

1. A trigger to perform number of UEs per network slice availability check and update

Figure 4.2.x.2-1: Number of UEs per network slice availability check and update procedure

- The AMF triggers the Number of UEs per network slice availability check and update procedure to update the number of UEs registered with a network slice when a network slice subject to NSAC is included or removed from the Allowed NSSAI for a UE. The procedure is triggered in the following cases:

- At UE Registration procedure, as per clause 4.2.2.2.2,

-	before the Registration Accept in step 21 if the EAC mode is active; or

-	after the Registration Accept message if the EAC mode is not active;

- At UE Deregistration procedure, as per clause 4.2.2.3, after the Deregistration procedure is completed;

- At UE Configuration Update procedure (which may result from NSSAA procedure):

-	before the UE Configuration Update message if the EAC mode is active; or

-	after the UE Configuration Update message if the EAC mode is not active;

NOTE: in one registration procedure, depend on the deployment the NSACF to be accessed can be more than one.

Editor's Note:	if one S-NSSAI the number of the UE/PDU session is managed by serveral different NSACF, it is FFS how to assure the AMF reach the same NSACF and how to coordinate the number of UE among the NSACF(s)

- If the S-NSSAI is verified that it can be included in the Allowed NSSAI, and the S-NSSAI is subject to NSAC, the AMF sends Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Request message to the NSACF. The AMF includes in the message the UE ID, the S-NSSAI(s) for which the number of UEs registered per network slice update is required and the update flag which indicates whether the number of UEs registered with the S-NSSAI(s) is to be increased when the UE has gained registration to network slice(s) subject to NSAC or the number of UEs registered with the S-NSSAI(s) is to be decreased when the UE has deregistered from S-NSSAI(s) or could not reniew its registration to an S-NSSAI subject to NSAC.

- The NSACF updates the current number of UEs registered for the S-NSSAI, i.e. increases or decrease the number of UEs registered per network slice based on the information provided by the AMF in the update flag parameter.

If the update flag parameter from the AMF indicates increase and if the UE ID is not in the list of UE IDs registered with the network slice and:

-	the maximum number of UEs for that S-NSSAI has not been reached yet, the NSACF adds the UE ID in the list of registered UEs and increases the current number of the registered UEs..

-	the maximum number of UEs for that S-NSSAI has already been reached, then the NSACF returns a result parameter indicating that the UE is rejected.

If the update flag parameter from the AMF indicates decrease, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI indicated in the request from the AMF and also the NSACF decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices.

- The NSACF returns the Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Response in which the NSACF includes the result whether the UE is accepted or rejected.

If all the S-NSSAI(s) are rejected by the NSACF(s), the AMF sends a Registration Reject message including the rejected S-NSSAI(s) in the rejected NSSAI parameter, an appropriate reject cause and optionally a back-off timer.

Editor's note:	It is FFS whether the AMF shall send the Registration Reject message when for all the S-NSSAI(s) that the UE requested to register for and were subject to NSAC the NSACF returned the maximum number of UEs per network slice has been reached.

Otherwise, the AMF returns Registration Accept message in which the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the NSACF has indicated that the maximum number of UEs per network slice has been reached, a reject cause set to 'maximum number of UEs per network slice reached' for each rejected S-NSSAI and optionally a back-off timer.

Editor's Note:	It is FFS whether and how to restrict the signalling sent from the AMFs to the NSACF in case the maximum number of UEs has been reached for prolonged time.

4.2.x.3	Configuration for Early Admission Control (EAC) update procedure

The configuration for Early Admission Control (EAC) update procedure indicates to the AMF the activation or the dectivation of the EAC mode for the S-NSSAI subject to NSAC. EAC mode means that the AMF is required to perform the number of UEs per network slice availability check and update procedure before the S-NSSAI subject to NSAC is included in the Allowed NSSAI and sent to the UE.

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceEACNotify (EAC flag=activated/deactivated)

3. Based on the EAC flag the AMF decides when to trigger the update calls to the NSACF

1. The number of UEs registered with a network slice subject to NSAC reaches a certain threshold

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceEACNotify (EAC flag=activated/deactivated)

3. Based on the EAC flag the AMF decides when to trigger the update calls to the NSACF

1. The number of UEs registered with a network slice subject to NSAC reaches a certain threshold

Figure 4.2.x.3-1: Early Admission Control (EAC) update procedure

- The NSACF determines based on certain threshold of the number of currently registered UEs with the S-NSSAI subject to NSAC whether to activate or deactivate the EAC mode.

- The NSACF triggers Nnsacf_NumberOfUEsPerSliceEACNotify operation including the S-NSSAI for which the EAC mode is to be activated or deactivated and a EAC flag set to activated if the number of UEs registered with the network slice is above certain threshold or set to deactivated if the number of the UEs registered with the network slice is below certain threshold.

Editor's Note:	It is FFS how to trigger the notification without subscription. Also how to determine which AMF is to be notified need be clarified.

- The AMF uses the EAC flag to decide when to trigger the number of UEs per network slice availability check and update procedure so that delays to the registration procedure and impact to the already allowed network slices are avoided.

If the EAC flag indicates EAC mode activated, the AMF triggers the number of UEs per network slice availability check and update procedure before the Registration Accept step of the registration procedure or before the UE Configuration Update message as in EAC activated mode the NSACF is to check whether the maximum number of UEs per network slice is reached which may impact the allowed S-NSSAI(s) confirmed by the Registration Accept message and the UE Configuration Update message.

If the EAC flag indicates EAC mode deactivated, the AMF triggers the number of UEs per network slice availability check and update procedure after Registration Accept step of the registration procedure or after the UE Configuration Update message as in EAC deactivated mode the NSACF does not check whether the maximum number of UEs per network slice is reached, i.e. no impact is expected to the allowed S-NSSAI(s) confirmed by the Registration Accept message and the UE Configuration Update message.

* * * * end of 1st change * * * *

* * * * start of 2nd change * * * *

5.2.xx	Network Slice Admission Control Function (NSACF) services

5.2.xx.1	General

The following table illustrates the NSACF services.

Table 5.2.xx.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NumberOfUEsPerSlice | AvailabilityCheckAndUpdate | Request/Response | AMF |
|  | EACNotify | Subscription/Notify | AMF |

5.2.xx.2	Nnsacf_NumberOfUEsPerSlice services

5.2.xx.2.1	General

Service Description: The Nnsacf_NumberOfUEsPerSlice services control the number of UEs registered with a network slice for the network slices subject to NSAC. The AMF can request the NSACF to check whether the number of UEs registered with a network slice has reached the maximum number of UEs per network slice and the AMF can also request the NSACF to update the number of UEs registered with a network slice.

5.2.xx.2.2	Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate service operation

Service Operation name: Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate

Description: Updates the number of UEs registered with a network slice (e.g. increase or decrease) when the UE registration status for a network slice subject to NSAC has changed. Also, if the number of the UEs registered with the network slice is to be increased and the Early Availability Check (EAC) mode in the NSACF is activated for that network slice (see Nnsacf_NumberOfUEsPerSliceEACNotify service operation), the NSACF first checks whether the number of UEs registered with the network slice has reached the maximum number of UEs per network slice threshold. If the maximum number of UEs registered with the network slice has already been reached, the UE registration for that network slice is rejected. If the EAC is not activated, the NSACF increases or decreases the number of UEs per network slice as per the input parameters below.

Inputs, Required: S-NSSAI(s), UE ID (SUPI), update flag.

The S-NSSAI(s) parameter is a list of one or more network slices for which the number of UEs registered with a network slice is to be updated and checked if the maximum number of UEs per network slice threshold has already been reached.

The UE ID is used by the NSACF to maintain a list of UE IDs registered with the network slice.

The update flag input parameter indicates whether the number of UEs registered with a network slice is to be:

- increased when the UE registers to a new network slice subject to NSAC. If the UE ID is already in the list of UEs registered with the network slice, the number of UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the number of the UEs registered with the network slice. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result;

- decreased when the UE deregisters for a network slice that is subject to NSAC. The NSACF decreases the number of the UEs registered with the network slice and removes the UE ID from the list of UEs registered with the network slice.

The NSACF may optionally return the current status of the network slice availability (e.g. a percentage out of the max number of UEs registered with a network slice) in the availability status parameter. This information may be used for NSACF signalling and load balancing in case multiple NSACFs are serving the same network slice.

Editor's Note:	It is FFS how to support in case multi NSACF is supported, e.g. discover the same NSACF, coordination of the local maximum number among NSACF .

Outputs, Required: maximum number of UEs per network slice reached, availability status

5.2.xx.2.3	Nnsacf_NumberOfUEsPerSliceEACNotify service operation

Service Operation name: Nnsacf_NumberOfUEsPerSliceEACNotify

Description: The NSACF is configured with the information about which network slices are subject to NSAC. The NSACF  may trigger notification to the AMF to indicate the activation of the Early Availability Check (EAC) mode for a certain network slice which is subject to NSAC when the number of the UEs registered with the network slice is above certain operator defined threshold (e.g. a percentage of the maximum number of UEs allowed to register with the network slice) and the deactivation of the EAC mode when the number of the UEs registered with the network slice is below certain operator defined threshold which may be same or different from the activation threshold.

Inputs, Required: S-NSSAI, EAC flag

The S-NSSAI input parameter is the network slice for which the NSACF activates or deactivates the EAC mode.

The EAC flag input parameter indicates whether the Slice EAC mode is activated or deactivated.

* * * * end of 2nd change * * * *