3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2101007r03

# Elbonia, February 24 – March 09, 2021										    (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0255 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improvements on analytics subsets, accuracy levels, and ordering of results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR implements some conclusions of TR 23.700-91 for KI#11 and KI#17. - The principle of selecting the relevant subsets in the output is useful in order to reduce the amount of output data (see TR 23.700-91 clause 8.11.3: "Additional Analytics filters shall be standardized on a per-analytics basis, in order to narrow down the scope of analytics, thus reducing signalling both for analytics provision and data collection"). - Accuracy depends on the subsets of analytics produced (continuous versus discrete values, vectors) per analytics ID. Accuracy constraints, in the scope of the NWDAF, are requirements on the quality of expected outputs. - Accuracy levels with finer grain can guide the choice of relevant data collection and calculation models, and the amount of resources (signalling, calculation) to be mobilized. The principle of additional accuracy levels together with finer grain per analytics subsets is part of conclusion of TR 23.700-91 (clause 8.17). - As concluded in TR 23.700-91 clause 8.11.3, criteria for ordering of results (together with maximum number or results) are useful to specify the desired order of results in a list of items according to certain criteria, in order to obtain the most important items in a list with less signalling. - Small errors were found in the specification while writing the CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The "subset of analytics that are requested" parameter of Analytics Filter Information, already defined for Network Performance Analytics, is renamed " list of analytics subsets that are requested" and extended to other analytics, as appropriate. Possible analytics susbsets are defined in analitycs output tables with a Note. - A parameter "Accuracy level per analytics subset" is defined. - Additional accuracy values are added in order to permit to refine the denotation of accuracy levels in requests or subscriptions; - A parameter "prefered order of results" is added in order to permit to select the ordering of results on the most useful criteria. - Errors in the specification are corrected: - "(average and variance)" removed for Duration of Time slot entry in Table 6.7.2.3-1 - In tables 6.7.3.3-1 and 6.7.3.3-2, "Duration time" and "Duration interval time" replaced by "Duration" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The impossibility to limit the output by selection of relevant subsets, the limited number of accuracy levels, as well as the lack of definition of semantics may lead to excess or deficiency in the quality of provided analytics as well as high signalling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3, 6.4.1, 6.5.1, 6.5.3, 6.6.1, 6.6.3, 6.7.2.1, 6.7.2.3, 6.7.3.1, 6.7.3.3, 6.8.1, 6.8.3, 7.2.2, 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in procedures.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	(Only for Nnwdaf_AnalyticsSubscription) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF. A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	Preferred level of accuracy of the analytics ("Low", "Medium", "High" or "Highest");

-	[OPTIONAL] Accuracy level per analytics subset ("Low", "Medium", "High" or "Highest"). When an accuracy level is expressed for a given analytics subset, it takes precedence for this subset over the above preferred level of accuracy. Analytics subsets are defined in the "Output Analytics" clause of applicable analytics;

-	(Only for Nnwdaf_AnalyticsInfo_Request) Time when analytics information is needed (if applicable). If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response to the consumer.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Preferred order of results when a list of analytics is returned, possibly with a criterion for identifying the property of the results to which the preferred ordering is applied;

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shal return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	In addition, the following additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: confidence in prediction.

# * * * * Next of change

## 6.4	Observed Service Experience related network data analytics

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average of observed Service MoS and/or variance of observed Service MoS indicating service MOS distribution for services such as audio-visual streaming as well as services that are not audio-visual streaming such as V2X and Web Browsing services) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	Service Experience for a Network Slice: Service Experience for a UE or a group of UEs or any UE in a Network Slice;

-	Service Experience for an Application: Service Experience for a UE or a group of UEs or any UE in an Application or a set of Applications.

Therefore, Observed Service experience may be provided individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1;

-	optionally, maximum number of objects and maximum number of SUPIs;

-	optionally, preferred level of accuracy of the analytics;

-	optionally, accuracy level per analytics subset (Network Slice instance service experience, Application service Experience); and

- 	optionally, prefered order of results for the list of Application Service Experiences and/or Slice instance service experiences: "ascending" or "descending".

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Description | Mandatory |  |
| --- | --- | --- | --- |
|  |  | Application | Network Slice |
| Application ID (0...max) | The identification of the application(s) for which the analytics information is subscribed or requested. | Y | N |
| S-NSSAI | Identifies the Network Slice for which analytics information is subscribed or requested. | N | Y |
| NSI ID(s) | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. | N | N |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. | N | N |
| Media/application bandwidth | Identifies the Media/application bandwidth requirement of the application. | N | N |
| DNN | DNN to access the application. | N | N |
| DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. | N | N |

NOTE:	A service consumer may use the Area of Interest in order to reduce the amount of signalling that the analytics subscription or request generates.

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provide statistics and predictions on the observed service experience to a consumer NF or to OAM.

Based on the Analytics Filter information in Table 6.4.1-1 and the Target of Analytics reporting provided by the service consumer in the analytics subscription or request, NWDAF determines whether service experience analytics should be delivered for:

i)	Application(s);

ii)	Network Slice;

iii)	both Application(s) and Network Slice.

If NWDAF is unable to differentiate based on the analytics subscription or request, it provides service experience analytics for both Application(s) and Network Slice.

If service experience for both Application(s) and Network Slice is desired but the Target of Analytics reporting or Analytics Filter information values (e.g. Area of Interest) need to be different, separate subscriptions/requests may be provided by the service consumer.

# * * * * Next change

## 6.5	NF load analytics

### 6.5.1	General

The clause 6.5 describes how NWDAF can provide NF load analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an NF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID set to "NF load information";

-	The Target of Analytics Reporting: an optional SUPI;

-	Analytics Filter Information:

-	optional S-NSSAI;

-	an optional list of NF Instance IDs, NF Set IDs, or NF types;

-	an optional list of analytics subsets that are requested (see clause 6.5.3);

-	Optional preferred level of accuracy of the analytics;

-	Optional Accuracy level per analytics subset (see clause 6.5.3);

- 	Optional prefered order of results for the list of resource status: ascending or descending NF load;

-	Optional Reporting Threshold; the Reporting Threshold is unique for all NFs matching the above Analytics Filter and the reporting applies when the conditions are met for at least one of these NFs;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.5.3.

If a list of the NF Instance IDs (or respectively of NF Set IDs) is provided, the NWDAF shall provide the analytics for each designated NF instance (or respectively for each NF instance belonging to each designated NF Set). In such case the Target of Analytics Reporting should be ignored.

Otherwise, if a SUPI is provided, the NWDAF shall use the SUPI to determine which NF instances (AMF and SMF) are serving this specific UE, filter them according to the provided S-NSSAI and NF types using data collected from NRF or OAM, and provide analytics for these NF instances.

NOTE:	Only NF instances of type AMF and SMF can be determined using a SUPI.

# * * * * Next change * * * *

### 6.5.3	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics. NF load statistics information are defined in Table 6.5.3-1. NF load predictions information are defined in Table 6.5.3-2.

Table 6.5.3-1: NF load statistics

| Information | Description |
| --- | --- |
| List of resource status (1..max) | List of observed load information for each NF instance along with the corresponding NF id / NF Set ID (as applicable) |
| > NF type | Type of the NF instance |
| > NF instance ID | Identification of the NF instance |
| > NF status (NOTE 1) | The availability status of the NF on the Analytics target period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |
| > NF resource usage (NOTE 1) | The average usage of assigned resources (CPU, memory, disk) |
| > NF load (NOTE 1) | The average load of the NF instance over the Analytics target period |
| > NF peak load (optional) (NOTE 1) | The maximum load of the NF instance over the Analytics target period |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Accuracy level per analytics subset". |  |  |

Table 6.5.3-2: NF load predictions

| Information | Description |
| --- | --- |
| List of resource status (1..max) | List of predicted load information for each NF instance along with the corresponding NF id / NF Set ID (as applicable) |
| > NF type | Type of the NF instance |
| > NF instance ID | Identification of the NF instance |
| > NF status (NOTE 2) | The availability status of the NF on the Analytics target period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |
| > NF resource usage (NOTE 2) | The average usage of assigned resources (CPU, memory, disk) |
| > NF load (NOTE 2) | The average load of the NF instance over the Analytics target period |
| > NF peak load (optional) (NOTE 2) | The maximum load of the NF instance over the Analytics target period |
| > Confidence | Confidence of this prediction |
| NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested" and "Accuracy level per analytics subset". |  |

NOTE 3:	The variations on per-instance NF load and resource usage could be influenced by the number of running NF instances in addition to the load itself.

The predictions are provided with a Validity Period, as defined in clause 6.1.3.

The number of resource status is limited by the maximum number of objects provided as part of Analytics Reporting Information.

# * * * * Next change

## 6.6	Network Performance Analytics

### 6.6.1	General

With Network Performance Analytics, NWDAF provides either statistics or predictions on the gNB status information, gNB resource usage, communication performance and mobility performance in an Area of Interest; in addition, NWDAF it may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The service consumer may be an NF (e.g. PCF, NEF, AF), or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID set to "Network Performance";

-	Target of Analytics Reporting containing either a UE, or Internal Group Identifier that refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Analytics target period is requested or any UE;

-	Analytics Filter Information containing:

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target Of Analytics Reporting is set to "any UE", optional otherwise);

-	Optionally, a list of analytics subsets that are requested among those specified in clause 6.6.3;

-	Optionally, a preferred level of accuracy of the analytics;

-	Optionally, Accuracy level per analytics subset (see clause 6.6.3);

- 	Optionally, prefered order of results for the list of Network Performance information:

-	ordering criterion: "number of UEs", "communication performance" or "mobility performance";

-	order: ascending or descending;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics information (see clause 6.6.3) in order to be notified by the NWDAF;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested; and

-	Optionally, maximum number of objects;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

# * * * * Next change

### 6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Observed statistics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area of interest as defined in clause 6.6.1 |
| > Analytics target period subset | Time window within the requested Analytics target period as defined in clause 6.6.1. |
| > gNB status information (NOTE 1) | Average ratio of gNBs that have been up and running during the entire Analytics target period in the area subset |
| > gNB resource usage (NOTE 1) | Average usage of assigned resources (CPU, memory, disk) |
| > Number of UEs (NOTE 1) | Average number of UEs observed in the area subset |
| > Communication performance (NOTE 1) | Average ratio of successful setup of PDU Sessions |
| > Mobility performance (NOTE 1) | Average ratio of successful handover |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Accuracy level per analytics subset". |  |

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-2: Network performance predictions

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Predicted analytics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area of interest as defined in clause 6.6.1 |
| > Analytics target period subset | Time window within the requested Analytics target period as defined in clause 6.6.1. |
| > gNB status information (NOTE 2) | Average ratio of gNBs that will be up and running during the entire Analytics target period in the area subset |
| > gNB resource usage (NOTE 2) | Average usage of assigned resources (CPU, memory, disk) (average, peak) |
| > Number of UEs (NOTE 2) | Average number of UEs predicted in the area subset |
| > Communication performance (NOTE 2) | Average ratio of successful setup of PDU Sessions |
| > Mobility performance (NOTE 2) | Average ratio of successful handover |
| > Confidence | Confidence of this prediction |
| NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested" and "Accuracy level per analytics subset". |  |

NOTE 1:	The predictions are provided with a Validity Period, as defined in clause 6.1.3.

NOTE 2:	The analytics on number of UEs are related to the information retrieved from the AMFs.

The number of network performance information entries is limited by the maximum number of objects provided as part of Analytics Reporting Information.

The NWDAF provides Network Performance Analytics to a consumer at the time requested by the consumer in the Analytics target period:

-	Analytics ID set to "Network Performance".

-	Notification Target Address including the address of the consumer.

-	Notification Correlation Id, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytics specific parameters at the time indicated in the Analytics target period.

# * * * * Next change

### 6.7.2	UE mobility analytics

#### 6.7.2.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from NF, OAM, and to perform data analytics to provide UE mobility statistics or predictions.

The service consumer may be a NF (e.g. AMF).

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytics Filter Information optionally containing:

-	Area of Interest;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Optionally, maximum number of objects.

-	Preferred level of accuracy of the analytics

- 	Prefered order of results for the time slot entries: ascending or descending time slot start;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

# * * * * Next change

#### 6.7.2.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics as defined in table 6.7.2.3-1, UE mobility predictions as defined in Table 6.7.2.3-2:

Table 6.7.2.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Observed location statistics |
| >> UE location | TA or cells which the UE stays |
| >> Ratio | Percentage of UEs in the group (in the case of an UE group) |

Table 6.7.2.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or SUPI (see NOTE). |
| Time slot entry (1..max) | List of predicted time slots |
| >Time slot start | Time slot start time within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Predicted location prediction during the Analytics target period |
| >> UE location | TA or cells where the UE or UE group may move into |
| >> Confidence | Confidence of this prediction |
| >> Ratio | Percentage of UEs in the group (in the case of an UE group) |

NOTE:	When target of analytics reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE.

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location at a given time.

The number of time slots and UE locations is limited by the maximum number of objects provided as part of Analytics Reporting Information

The time slots shall be provided by order of time, possibly overlapping. The locations shall be provided by decreasing value of ratio for a given time slot. The sum of all ratios on a given time slot must be equal or less than 100%. Depending on the list size limitation, the least probable locations on a given Analytics target period may not be provided.

# * * * * Next change

### 6.7.3	UE Communication Analytics

#### 6.7.3.1	General

In order to support some optimized operations, e.g. customized mobility management, traffic routing handling, or QoS improvement, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic, and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF, SMF and UPF that corresponds to this application ID.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytics Filter Information optionally including:

-	S-NSSAI;

-	DNN;

-	Application ID;

-	Area of Interest.

-	an optional list of analytics subsets that are requested (see clause 6.7.3.3);

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics

-	Optional Accuracy level per analytics subset (see clause 6.7.3.3);

- 	Optional prefered order of results for the list of UE Communications:

-	ordering criterion: "start time" or "duration",

-	order: ascending or descending;

-	Optionally, maximum number of objects;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

# * * * * Next change

#### 6.7.3.3	Output Analytics

The NWDAF supporting UE Communication Analytics provides the analytics results to consumer NFs. The analytics results provided by the NWDAF include the UE communication statistics as defined in Table 6.7.3.3-1 or predictions as defined in Table 6.7.3.3-2.

Table 6.7.3.3-1: UE Communication Statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI (see NOTE). |
| UE communications (1..max) | List of communication time slots. |
| > Periodic communication indicator (NOTE 1) | Identifies whether the UE communicates periodically or not. |
| > Periodic time (NOTE 1) | Interval Time of periodic communication (average and variance) if periodic. Example: every hour |
| > Start time (NOTE 1) | Start time observed (average and variance) |
| > Duration (NOTE 1) | Duration of communication (average and variance). |
| > Traffic characterization | S-NSSAI, DNN, ports, other useful information. |
| > Traffic volume (NOTE 1) | Volume UL/DL (average and variance). |
| > Ratio | Percentage of UEs in the group (in the case of an UE group). |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Accuracy level per analytics subset". |  |

Table 6.7.3.3-2: UE Communication Predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI (see NOTE). |
| UE communications (1..max) | List of communication time slots. |
| > Periodic communication indicator (NOTE 2) | Identifies whether the UE communicates periodically or not. |
| > Periodic time (NOTE 2) | Interval Time of periodic communication (average and variance) if periodic. Example: every hour. |
| > Start time (NOTE 2) | Start time predicted (average and variance). |
| > Duration time (NOTE 2) | Duration interval time of communication. |
| > Traffic characterization | S-NSSAI, DNN, ports, other useful information. |
| > Traffic volume (NOTE 2) | Volume UL/DL (average and variance). |
| > Confidence | Confidence of the prediction. |
| > Ratio | Percentage of UEs in the group (in the case of an UE group). |
| NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested" and "Accuracy level per analytics subset". |  |

NOTE:	When target of analytics reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics communication result (i.e. list of (predicted) communication time slots) to NF service consumer(s) for the UE.

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group for a given communication at a given time and duration.

The number of UE communication entries (1..max) is limited by the maximum number of objects provided as part of Analytics Reporting Information. The communications shall be provided by order of time, possibly overlapping.

Depending on the list size limitation, the least probable communications on a given Analytics target period may not be provided.

# * * * * Next change

## 6.8	User Data Congestion Analytics

### 6.8.1	General

The NWDAF can provide user data congestion related analytics, by one-time reporting or continuous reporting, in the form of statistics or predictions or both, to another NF. User Data Congestion related analytics can relate to congestion experienced while transferring user data over the control plane or user plane or both. A request for user data congestion analytics relates to a specific area or to a specific user. If the consumer of these analytics provides a UE ID, the NWDAF determines the area where the UE is located. The NWDAF then collects measurements per cell and uses the measurements to determine user data congestion analytics.

The request for user data congestion related analytics indicates the location area information where congestion related analytics is desired or indicates a UE Identity that can be used by the NWDAF to determine the location area information where congestion related analytics is desired. When the consumer of user data congestion related analytics subscribes to user data congestion related analytics, it may indicate a threshold and the NWDAF will provide analytics to the consumer when the congestion level crosses the threshold. The consumer can indicate an S-NSSAI in the request when congestion analytics are needed on a per slice level.

The service consumer may be an NF (e.g. NEF, AF).

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID set to "User Data Congestion".

-	Target of Analytics Reporting containing either a SUPI, or "any UE".

-	Analytics Filter Information containing:

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target Of Analytics Reporting is set to "any UE", optional otherwise);

-	Optional S-NSSAI, in order to obtain congestion analytics only on a given slice.

-	Optional Reporting Threshold, which applies only for subscriptions and indicates conditions on the congestion level (Network Status Indication, see clause 6.8.3) to be reached in order to be notified by the NWDAF.

-	Preferred level of accuracy of the analytics;

-	Optional Accuracy level per analytics subset (see clause 6.8.3);

-	Prefered order of results for the list of User Data Congestion statistics or predictions:

-	ordering by Applicable Time Window, chronological or reverse chronological order; or

-	ordering by Network Status Indication, ascending or descending;

-	Optional maximum number of objects;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested, either in the past or in the future.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

# * * * * Next change

### 6.8.3	Output analytics

The NWDAF outputs the user data congestion analytics for transfer over the user plane, for transfer over the control plane, or for both. The output may consist of statistics, predictions, or both. The detailed information provided by the NWDAF is defined in Table 6.8.3-1 for statistics and in Table 6.8.3-2 for predictions.

Table 6.8.3-1: User Data Congestion statistics

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell IDs |
| List of user data congestion Analytics (1..max) |  |
| >Type | User Plane or Control Plane |
| >Applicable Time Window (NOTE 1) | The time period that the analytics applies to |
| >Network Status Indication (NOTE 1) | Congestion Level |
| NOTE 1: Analytics subset that can be used in "Accuracy level per analytics subset". |  |

Table 6.8.3-2: User Data Congestion predictions

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell IDs |
| List of user data congestion Analytics (1..max) |  |
| >Type | User Plane or Control Plane |
| >Applicable Time Window (NOTE 2) | The time period that the analytics applies to |
| >Network Status Indication (NOTE 2) | Congestion Level |
| > Confidence | Confidence of this prediction |
| NOTE 2: Analytics subset that can be used in "Accuracy level per analytics subset". |  |

The number of user data congestion analytics entries is limited by the maximum number of objects provided as part of Analytics Reporting Information.

# * * * * Next change

### 7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe.

Description: Subscribes to NWDAF analytics with specific parameters.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, Target of Analytics Reporting, Notification Target Address (+ Notification Correlation ID), Analytics Reporting Parameters, Analytics target period.

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID in a set of Analytics IDs.

Inputs, Optional: Analytics Filter Information, Subscription Correlation ID (in the case of modification of the analytics subscription), preferred level of accuracy of the analytics, accuracy level per analytics subset, Reporting Thresholds, Maximum number of objects requested (max), Prefered order of results, Maximum number of SUPIs requested (SUPImax).

NOTE 2:	Analytics Filter Information, Reporting Thresholds, Maximum number of objects requested (max) and Maximum number of SUPIs requested (SUPImax) can be provided per individual Analytics ID in a set of Analytics IDs.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription).

Outputs, Optional: None.

# * * * * Next change

### 7.3.2	Nnwdaf_AnalyticsInfo_Request service operation

Service operation name: Nnwdaf_AnalyticsInfo_Request.

Description: the consumer requests NWDAF operator specific analytics.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, Target of Analytics Reporting, Analytics target period.

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID in a set of Analytics IDs.

Inputs, Optional: Analytics Filter Information, preferred level of accuracy of the analytics, accuracy level per analytics subset, time when analytics information is needed, Maximum number of objects requested (max), Prefered order of results, Maximum number of SUPIs requested (SUPImax).

NOTE 2:	Analytics Filter Information, Maximum number of objects requested (max) and Maximum number of SUPIs requested (SUPImax) can be provided per individual Analytics ID in a set of Analytics IDs.

Outputs, Required: Set of the tuple (Analytics ID, Analytics specific parameters).

Outputs, Optional: Timestamp of analytics generation, validity period, probability assertion.

NOTE 3:	Some NWDAF output analytics already include confidence of predictions, which provides the same information as probability assertion.

NOTE 4:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

# * * * * End of changes * * * *